O-RAN.WG3.RICAPI-R003-v02.00

Technical Specification

O-RAN Work Group 3 (Near-Real-time RAN Intelligent Controller and E2 Interface) Near-RT RIC APIs

Copyright © 2024 by the O-RAN ALLIANCE e.V. The copying or incorporation into any other work of part or all of the material available in this specification in any form without the prior written permission of O-RAN ALLIANCE e.V. is prohibited, save that you may print or download extracts of the material of this specification for your personal use, or copy the material of this specification for the purpose of sending to individual third parties for their information provided that you acknowledge O-RAN ALLIANCE as the source of the material and that you inform the third party that these conditions apply to them and that they must comply with them. O-RAN ALLIANCE e.V., Buschkauler Weg 27, 53347 Alfter, Germany Register of Associations, Bonn VR 11238, VAT ID DE321720189

Contents

Foreword	4

Modal verbs terminology	4

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	General	6

4.1	Introduction	6

4.2	General Principles	6

4.3	Version Control	6

5	A1-related API	7

5.1	Overview	7

5.2	A1-related API Procedures	8

5.2.1	A1 Policy Setup procedure	8

5.2.2	A1 Policy Update procedure	10

5.2.3	A1 Policy Delete procedure	11

5.3	Elements for A1-related API Communication	13

5.3.1	General	13

5.3.2	Message Functional Definition and Content	13

5.3.3	Information Element Definitions	15

5.3.4	Message and Information Element Abstract Syntax	17

5.3.5	Message Transfer Syntax	20

6	E2-related API	20

6.1	Overview	20

6.1.1	Protocol Stack	20

6.1.2	List of E2 related APIs, procedures and messages	21

6.2	E2-related API Procedures	22

6.2.1	E2_Subscription_API procedures	22

6.2.2	E2_Indication_API procedures	28

6.2.3	E2_Control_API procedures	29

6.3	Elements for E2-related API Communication	31

6.3.1	General	31

6.3.2	Message Functional Definition and Content	31

6.3.3	Information Element Definitions	40

6.3.4	Message and Information Element Abstract Syntax	43

6.3.5	Message Transfer Syntax	54

7	SDL API	54

7.1	Overview	54

7.1.1	Protocol Stack	54

7.1.2	List of SDL APIs, procedures and messages	54

7.2	SDL API Procedures	56

7.2.1	SDL Client Registration procedure	56

7.2.2	SDL Client Deregistration procedure	56

7.2.3	SDL Subscribe Information procedure	56

7.2.4	SDL Information Push procedure	57

7.2.5	SDL Information Update Notify procedure	58

7.2.6	SDL Information Fetch procedure	59

7.2.7	Store Data procedure	60

7.2.8	Modify Data procedure	60

7.3	Elements for SDL API Communication	65

7.3.1	General	65

7.3.2	Message Functional Definition and Content	66

7.3.3	Information Element Definitions	72

7.3.4	Message and Information Element Abstract Syntax	82

7.3.5	Message Transfer Syntax	93

8	Management API	93

8.1	Overview	93

8.1.1	Protocol Stack	93

8.1.2	List of Management APIs, procedures and messages	93

8.2	Management API Procedures	94

8.2.1	xApp Registration Procedure	94

8.3	Elements for Management API Communication	95

8.3.1	General	95

8.3.2	Message Functional Definition and Content	95

8.3.3	Information Element Definitions	97

8.3.4	Message and Information Element Abstract Syntax	98

8.3.5	Message Transfer Syntax	100

9	Enablement API	100

9.1	Overview	100

9.1.1	Protocol Stack	100

9.1.2	List of Enablement APIs, procedures and messages	100

9.2	Enablement API Procedures	101

9.2.1	Enabl_Discovery_API Procedures	101

9.2.2	Enabl_Events_API Procedures	103

9.3	Elements for Enablement API Communication	111

9.3.1	General	111

9.3.2	Message Functional Definition and Content	111

9.3.3	Information Element Definitions	118

9.3.4	Message and Information Element Abstract Syntax	121

9.3.5	Message Transfer Syntax	128

10	Common definitions across APIs	128

10.1	Information element definitions	128

10.1.1	PTI	128

10.1.2	xApp ID	128

10.1.3	API Name	128

10.1.4	API Version	129

10.1.5	A1 Policy Type ID	129

10.1.6	Global E2 Node ID	129

10.1.7	RAN Function ID	129

10.1.8	SCTP API PDU	129

10.1.9	Endpoint Information	129

10.2	Information element abstract syntax	129

10.2.1	Abstact syntax in Protocol Buffers	129

Annex (informative):  Change History	133

Foreword

This Technical Specification (TS) has been produced by O-RAN Alliance.

The content of the present document is subject to continuing work within O-RAN and may change following formal O-RAN approval. Should the O-RAN Alliance modify the contents of the present document, it will be re-released by O-RAN with an identifying change of version date and an increase in version number as follows:

version xx.yy.zz

where:

xx:	the first digit-group is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc. (the initial approved document will have xx=01).  Always 2 digits with leading zero if needed.

yy:	the second digit-group is incremented when editorial only changes have been incorporated in the document. Always 2 digits with leading zero if needed.

zz:	the third digit-group included only in working versions of the document indicating incremental changes during the editing process. External versions never include the third digit-group.  Always 2 digits with leading zero if needed.

Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the O-RAN Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in O-RAN deliverables except when used in direct citation.

1	Scope

The present document specifies the signalling and data transport protocols for Near-RT RIC APIs. The protocols are developed in accordance with the functionalities and interactions stated in [2].

2	References

2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, O-RAN cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]		O-RAN.WG3.RICARCH-v03.00.00: “O-RAN, Near-Real-time RAN Intelligent Controller, Near-RT RIC Architecture”.

[3] 	O-RAN.WG3.E2AP-v02.01.00: “O-RAN, Near-Real-time RAN Intelligent Controller, E2 Application Protocol(E2AP)”.

[4] 	Protocol buffers. Available at https://developers.google.com/protocol-buffers/.

[5]	gRPC. Available at https://www.grpc.io/.

[6] 	O-RAN.WG11.SecReqSpecs: “O-RAN, Security Requirements Specifications”.

[7]	O-RAN.WG2.A1TD-v04.00: “O-RAN, A1 interface: Type Definitions”.

[8]		O-RAN.WG3.E2SM-v02.01.02: “O-RAN, Near-Real-time RAN Intelligent Controller, E2 Service Model (E2SM)”.

[9] 	IETF RFC 4122: “A Universally Unique IDentifier (UUID) URN Namespace”.

[10]	O-RAN.WG3.Y1AP-v01.00: " O-RAN Y1 interface: Application Protocol".

[11]	O-RAN.WG3.Y1TD-v01.00: " O-RAN Y1 interface: Type Definitions".

2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, O-RAN cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1] 	Semantic Versioning Specification. Available at https://semver.org.

3	Definition of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the following terms apply:

Near-RT RIC: as defined in [2].

Near-RT RIC APIs: interfaces that connect xApp and Near-RT RIC platform.

Near-RT RIC Platform: a software platform to host xApps. The Near-RT RIC is comprised of the Near-RT RIC Platform and xApps.

xApp: as defined in [2].

3.2	Symbols

No Symbol is defined in this document.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply:

API	Application Programming Interface

Near-RT RIC	Near-real-time RAN Intelligent Controller

PTI	Procedure Transaction Identity

4	General

4.1	Introduction

Clause 4 specifies the principles and conventions commonly applied to Near-RT RIC API design with the Network API approach as described in clause 7.1.2 of [2].

4.2	General Principles

For each Near-RT RIC API, the following principles should be followed:

-	A default solution that includes network protocol and message encoding should be defined to ensure interoperability. The default solution requires mandatory support by all Near-RT RIC platforms and xApps. The optional protocols available for a given APIs are discoverable by xApps.

-	The IEs in the Near-RT RIC API which refer to different specifications shall be encoded as defined in the referred specification.

The common data types applicable to several API categories should be collected in clause 10.

In addition, the following design principles shall apply:

- Parallel transactions, i.e., multiple ongoing Near-RT RIC API procedures of the same types related to the same xApp, and platform functionality are supported.

4.3	Version Control

An API version shall consist of at least 3 fields, following a MAJOR.MINOR.PATCH pattern according to the Semantic Versioning Specification [i.1] unless specified otherwise.

EXAMPLE:	"1.0.0".

The basic rules as reproduced from [i.1] shall apply for incrementing the versions:

-	MAJOR version is incremented if backward incompatible change is made.

-	MINOR version is incremented if functional change is made in a backward compatible way.

-	PATCH version is incremented if correction is made in a backward compatible way.

5	A1-related API

5.1	Overview

### 5.1.1	Protocol Stack

In this version of specification, the default protocol stack for the A1 related APIs is shown on Figure 5.1.1-1.

Note: the network layer for security protection is FFS.

Figure 5.1.1-1: Protocol stack for A1 related APIs

The A1 related APIs use gRPC [5] with Protocol Buffers [4] as the application layer serialization protocol.

### 5.1.2	List of A1 related APIs, procedures and messages

The A1 related API procedures introduced in clause 9.2 of [2] are categorized under the A1 related APIs. These APIs include:

A1 Policy API:  enabling A1 policy related operations for the xApps.

A1 Enrichment Information (A1-EI) API: enabling A1-EI related operations for the xApps.

These APIs are summarized in Table 5.1.2-1.

Table 5.1.2-1: List of A1 related APIs

| API Name | Description |
| --- | --- |
| A1_Policy_API | Service for A1 policy related operations |
| A1_EI_API | Service for A1 enrichment information related operations |

The table 5.1.2-2 lists the API procedures for each A1 related API.

Table 5.1.2-2: List of A1 related API procedures

| API Name | API procedure | Communication type |
| --- | --- | --- |
| A1_Policy_API | A1 Policy Setup | Request/Response |
|  | A1 Policy Update | Request/Response |
|  | A1 Policy Delete | Request/Response |
|  | A1 Policy Query | Request/Response |
|  | A1 Policy Status Update | Request/Response |
| A1_EI_API | A1 EI Subscription Setup | Request/Response |
|  | A1 EI Subscription Update | Request/Response |
|  | A1 EI Subscription Delete | Request/Response |
|  | A1 EI Delivery | Notify |
|  | A1 EI Query | Request/Response |

The table 5.1.2-3 lists the messages for each A1 related API procedure.

Table 5.1.2-3: List of A1 related API messages

| API Procedure | Initiated by | Initiating Message | Outcome Message |  |
| --- | --- | --- | --- | --- |
|  |  |  | Successful Outcome | Unsuccessful Outcome |
| A1 Policy Setup | Near-RT RIC platform | A1 POLICY SETUP REQUEST | A1 POLICY SETUP RESULT (SUCCESS) | A1 POLICY SETUP RESULT (FAILURE) |
| A1 Policy Update | Near-RT RIC platform | A1 POLICY UPDATE REQUEST | A1 POLICY UPDATE RESULT (SUCCESS) | A1 POLICY UPDATE RESULT (FAILURE) |
| A1 Policy Delete | Near-RT RIC platform | A1 POLICY DELETE REQUEST | A1 POLICY DELETE RESULT (SUCCESS) | A1 POLICY DELETE RESULT (FAILURE) |
| A1 Policy Query | Near-RT RIC platform | A1 POLICY QUERY REQUEST | A1 POLICY QUERY RESULT (SUCCESS) | A1 POLICY QUERY RESULT (FAILURE) |
| A1 Policy Status Update | xApp | A1 POLICY STATUS UPDATE | A1 POLICY STATUS UPDATE ACK | A1 POLICY STATUS UPDATE ERROR |
| A1 EI Subcription Setup | xApp | A1 EI SUBSCRIPTION SETUP REQUEST | A1 EI SUBSCRIPTION SETUP RESULT (SUCCESS) | A1 EI SUBSCRIPTION SETUP RESULT (FAILURE) |
| A1 EI Subcription Update | xApp | A1 EI SUBSCRIPTION UPDATE REQUEST | A1 EI SUBSCRIPTION UPDATE RESULT (SUCCESS) | A1 EI SUBSCRIPTION UPDATE RESULT (FAILURE) |
| A1 EI Subcription Delete | xApp | A1 EI SUBSCRIPTION DELETE REQUEST | A1 EI SUBSCRIPTION DELETE RESULT (SUCCESS) | A1 EI SUBSCRIPTION DELETE RESULT (FAILURE) |
| A1 EI Delivery | Near-RT RIC platform | A1 EI Delivery | - | - |
| A1 EI Query | xApp | A1 EI QUERY REQUEST | A1 EI QUERY RESULT (SUCCESS) | A1 EI QUERY RESULT (FAILURE) |

5.2	A1-related API Procedures

5.2.1	A1 Policy Setup procedure

5.2.1.1	General

The purpose of the A1 Policy Setup procedure is to enable the execution of the transferred A1 policy.

5.2.1.2	Successful operation

@startuml

Skin rose

Participant rp as “Near-RT RIC Platform”

Participant ap as “xApp”

rp->ap: A1 POLICY SETUP REQUEST

ap->rp: A1 POLICY SETUP RESULT\n[A1 Policy Setup Result=success]

@enduml

Figure 5.2.1.2-1 : A1 Policy Setup procedure, successful operation

The Near-RT RIC Platform initiates the procedure by sending A1 POLICY SETUP REQUEST message to xApp.

The A1 POLICY SETUP REQUEST message shall contain the information required by the xApp to setup one A1 policy including A1 Policy Type ID IE, A1 Policy ID IE, and A1 Policy Object IE.

Upon reception of the A1 POLICY SETUP REQUEST message, the xApp shall transform the received A1 policy into appropriate E2 / A1-EI operation(s) requirements, and shall trigger Near-RT RIC Platform to accommodate E2 / A1-EI operation(s) requirements.

If Near-RT RIC Platform is able to accommodate E2 / A1-EI operation(s) requirements, xApp replies A1 POLICY SETUP RESULT message with A1 Policy Setup Result IE set to success to Near-RT RIC Platform. And xApp should store the received A1 Policy Type ID IE, A1 Policy ID IE, A1 Policy Object IE as basic elements of A1 policy profile, and label the status of corresponding A1 policy as “enforced”.

5.2.1.3	Unsuccessful operation

@startuml

Skin rose

Participant rp as “Near-RT RIC Platform”

Participant ap as “xApp”

rp->ap: A1 POLICY SETUP REQUEST

ap->rp: A1 POLICY SETUP RESULT\n[A1 Policy Setup Result=failure]

@enduml

Figure 5.2.1.3-1 : A1 Policy Setup procedure, unsuccessful operation

If xApp cannot transform the received A1 policy into appropriate E2 / A1-EI operation(s) requirements due to unsupported A1 policy type, xApp replies A1 POLICY SETUP RESULT message with A1 Policy Setup Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Not supported A1 policy type.

If xApp cannot transform the received A1 policy into appropriate E2 / A1-EI operation(s) requirements due to violation of of the requirements as stated in 5.2.1.2.2, 5.2.2.2.2, 5.2.3.2.2, 5.2.4.2.2, 5.2.5.2.2, 5.2.6.2.2, 5.2.7.2.2 and 5.2.8.2.2 of [7], xApp replies A1 POLICY SETUP RESULT message with A1 Policy Setup Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Not allowed statements, restrictions and extensions.

If Near-RT RIC Platform cannot accommodate E2 / A1-EI operation(s) requirements, xApp replies A1 POLICY SETUP RESULT message with A1 Policy Setup Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Near-RT RIC Platform cannot accommodate E2 / A1-EI operations requirements.

5.2.1.4	Abnormal conditions

If A1 policy indicated by A1 Policy Type ID IE, A1 Policy ID IE contained in A1 POLICY SETUP REQUEST message is being enforced by xApp, xApp replies A1 POLICY SETUP RESULT message with A1 Policy Setup Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to A1 policy already enabled at xApp.

5.2.2	A1 Policy Update procedure

5.2.2.1	General

The purpose of the A1 Policy Update procedure is to update the execution of the transferred A1 policy.

5.2.2.2	Successful operation

@startuml

Skin rose

Participant rp as “Near-RT RIC Platform”

Participant ap as “xApp”

rp->ap: A1 POLICY UPDATE REQUEST

ap->rp: A1 POLICY UPDATE RESULT\n[A1 Policy Update Result=success]

@enduml

Figure 5.2.2.2-1 : A1 Policy Update procedure, successful operation

The Near-RT RIC Platform initiates the procedure by sending A1 POLICY UPDATE REQUEST message to xApp.

The A1 POLICY UPDATE REQUEST message shall contain the information required by the xApp to update one A1 policy including A1 Policy Type ID IE, A1 Policy ID IE, and A1 Policy Object IE.

Upon reception of the A1 POLICY SETUP REQUEST message, the xApp shall transform the received A1 policy into appropriate E2 / A1-EI operation(s) requirements. If the transformed E2 / A1-EI operation(s) requirements are the same as before, xApp replies A1 POLICY UPDATE RESULT message with A1 Policy Update Result IE set to success to Near-RT RIC Platform; else, xApp shall trigger Near-RT RIC Platform to accommodate E2 / A1-EI operation(s) requirements.

If Near-RT RIC Platform to accommodate E2 / A1-EI operation(s) requirements succeeds, xApp replies A1 POLICY UPDATE RESULT message with A1 Policy Update Result IE set to success to Near-RT RIC Platform. And xApp should update the stored information of A1 Policy Object IE of A1 policy profile, and label the status of corresponding A1 policy as “enforced”.

5.2.2.3	Unsuccessful operation

@startuml

Skin rose

Participant rp as “Near-RT RIC Platform”

Participant ap as “xApp”

rp->ap: A1 POLICY UPDATE REQUEST

ap->rp: A1 POLICY UPDATE RESULT\n[A1 Policy Update Result=failure]

@enduml

Figure 5.2.2.3-1 : A1 Policy Update procedure, unsuccessful operation

If xApp cannot transform the received A1 policy into appropriate E2 / A1-EI operation(s) requirements due to not support the corresponding A1 policy type, xApp replies A1 POLICY UPDATE RESULT message with A1 Policy Update Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Not supported A1 policy type.

If xApp cannot transform the received A1 policy into appropriate E2 / A1-EI operation(s) requirements due to violation of the requirements as stated in 5.2.1.2.2, 5.2.2.2.2, 5.2.3.2.2, 5.2.4.2.2, 5.2.5.2.2, 5.2.6.2.2, 5.2.7.2.2 and 5.2.8.2.2 of [7], xApp replies A1 POLICY UPDATE RESULT message with A1 Policy Update Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Not allowed statements, restrictions and extensions.

If Near-RT RIC Platform cannot accommodate E2 / A1-EI operation(s) requirements, xApp replies A1 POLICY UPDATE RESULT message with A1 Policy Update Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Near-RT RIC Platform cannot accommodate E2 / A1-EI operations requirements.

5.2.2.4	Abnormal conditions

If A1 policy indicated by A1 Policy Type ID IE, A1 Policy ID IE contained in A1 POLICY UPDATE REQUEST message is not enabled by xApp, xApp replies A1 POLICY UPDATE RESULT message with A1 Policy Update Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Not enabled A1 policy at xApp.

5.2.3	A1 Policy Delete procedure

5.2.3.1	General

The purpose of the A1 Policy Delete procedure is to terminate the execution of the transferred A1 policy.

5.2.3.2	Successful operation

@startuml

Skin rose

Participant rp as “Near-RT RIC Platform”

Participant ap as “xApp”

rp->ap: A1 POLICY DELETE REQUEST

ap->rp: A1 POLICY DELETE RESULT\n[A1 Policy Delete Result=success]

@enduml

Figure 5.2.3.2-1 : A1 Policy Delete procedure, successful operation

The Near-RT RIC Platform initiates the procedure by sending A1 POLICY DELETE REQUEST message to xApp.

The A1 POLICY DELETE REQUEST message shall contain the information required by the xApp to delete one A1 policy including A1 Policy Type ID IE, A1 Policy ID IE.

Upon reception of the A1 POLICY DELETE REQUEST message, the xApp shall locate the E2 / A1-EI operation(s) associated with the A1 policy indicated by A1 Policy Type ID IE, A1 Policy ID IE contained in A1 POLICY DELETE REQUEST message, and shall trigger Near-RT RIC Platform to terminate the accommodation of located E2 / A1-EI operation(s).

No matter Near-RT RIC Platform to terminate the accommodation of located E2 / A1-EI operation(s) succeeds or not, xApp replies A1 POLICY DELETE RESULT message with A1 Policy Delete Result IE set to success to Near-RT RIC Platform. And xApp should delete the stored A1 policy profile associated.

5.2.3.3	Unsuccessful operation

@startuml

Skin rose

Participant rp as “Near-RT RIC Platform”

Participant ap as “xApp”

rp->ap: A1 POLICY DELETE REQUEST

ap->rp: A1 POLICY DELETE RESULT\n[A1 Policy Delete Result=failure]

@enduml

Figure 5.2.3.3-1 : A1 Policy Delete procedure, unsuccessful operation

If xApp cannot locate the E2 / A1-EI operation(s) associated with the A1 policy indicated by A1 Policy Type ID IE, A1 Policy ID IE contained in A1 POLICY DELETE REQUEST message, xApp replies A1 POLICY DELETE RESULT message with A1 Policy Delete Result IE set to failure to Near-RT RIC Platform, possible with Cause IE set to Not enabled A1 policy at xApp.

5.2.3.4	Abnormal conditions

Not applicable.

5.3	Elements for A1-related API Communication

5.3.1	General

5.3.2	Message Functional Definition and Content

5.3.2.1	A1 POLICY SETUP REQUEST

This message is sent by the RIC Platform and is used to request the destinated xApp to enable the execution of the nested A1 policy.

Direction: RIC Platform → xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.3.3.1 |  | YES | reject |
| A1 Policy Type ID | M |  | 5.3.3.2 |  | YES | reject |
| A1 Policy ID | M |  | 5.3.3.3 |  | YES | reject |
| A1 Policy Object | M |  | OCTET SRTING | Defined in [7]. | YES | reject |

5.3.2.2	A1 POLICY SETUP RESULT

This message is sent by the xApp as response to the request to enable the execution of the nested A1 policy.

Direction: xApp → RIC Platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.3.3.1 |  | YES | reject |
| A1 Policy Type ID | M |  | 5.3.3.2 |  | YES | reject |
| A1 Policy ID | M |  | 5.3.3.3 |  | YES | reject |
| A1 Policy Setup Result | M |  | 5.3.3.4 |  | YES | reject |
| Cause | O |  | 5.3.3.5 |  | YES | ignore |
| A1 Policy Object | C |  | OCTET SRTING | Defined in [7]. This IE shall present if the A1 Policy Setup Result IE is set to success. When present, this IE is copied from the corresponding Request message. | YES | ignore |

5.3.2.3	A1 POLICY UPDATE REQUEST

This message is sent by the RIC Platform and is used to request the destinated xApp to update the execution of the nested A1 policy.

Direction: RIC Platform → xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.3.3.1 |  | YES | reject |
| A1 Policy Type ID | M |  | 5.3.3.2 |  | YES | reject |
| A1 Policy ID | M |  | 5.3.3.3 |  | YES | reject |
| A1 Policy Object | M |  | OCTET SRTING | Defined in [7]. | YES | reject |

5.3.2.4	A1 POLICY UPDATE RESULT

This message is sent by xApp as response to the request to update the execution of the nested A1 policy.

Direction: xApp → RIC Platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.3.3.1 |  | YES | reject |
| A1 Policy Type ID | M |  | 5.3.3.2 |  | YES | reject |
| A1 Policy ID | M |  | 5.3.3.3 |  | YES | reject |
| A1 Policy Update Result | M |  | 5.3.3.4 |  | YES | reject |
| Cause | O |  | 5.3.3.5 |  | YES | ignore |
| A1 Policy Object | C |  | OCTET SRTING | Defined in [7]. This IE shall present if the A1 Policy Update Result IE is set to success. When present, this IE is copied from the corresponding Request message. | YES | ignore |

5.3.2.5	A1 POLICY DELETE REQUEST

This message is sent by the RIC Platform and is used to request the destinated xApp to terminate the execution of the nested A1 policy.

Direction: RIC Platform → xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.3.3.1 |  | YES | reject |
| A1 Policy Type ID | M |  | 5.3.3.2 |  | YES | reject |
| A1 Policy ID | M |  | 5.3.3.3 |  | YES | reject |

5.3.2.6	A1 POLICY DELETE RESULT

This message is sent by xApp as response to the request to terminate the execution of the nested A1 policy.

Direction: xApp → RIC Platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.3.3.1 |  | YES | reject |
| A1 Policy Type ID | M |  | 5.3.3.2 |  | YES | reject |
| A1 Policy ID | M |  | 5.3.3.3 |  | YES | reject |
| A1 Policy Delete Result | M |  | 5.3.3.4 |  | YES | reject |
| Cause | O |  | 5.3.3.5 |  | YES | ignore |

5.3.2.7	A1 POLICY QUERY REQUEST

5.3.2.8	A1 POLICY QUERY RESULT

5.3.2.9	A1 POLICY STATUS UPDATE

5.3.2.10	A1 POLICY STATUS UPDATE ACK

5.3.2.11	A1 POLICY STATUS UPDATE ERROR

5.3.2.12	A1-EI SUBSCRIPTION SETUP REQUEST

5.3.2.13	A1-EI SUBSCRIPTION SETUP RESULT

5.3.2.14	A1-EI SUBSCRIPTION UPDATE REQUEST

5.3.2.15	A1-EI SUBSCRIPTION UPDATE RESULT

5.3.2.16	A1-EI SUBSCRIPTION DELETE REQUEST

5.3.2.17	A1-EI SUBSCRIPTION DELETE RESULT

5.3.2.18	A1-EI DELIVERY

5.3.2.19	A1-EI QUERY REQUEST

5.3.2.20	A1-EI QUERY RESULT

5.3.3	Information Element Definitions

5.3.3.1	Message Type

The Message Type IE identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Type | M |  | CHOICE (A1 policy setup, A1 policy update, A1 policy delete, …) |  |
| >Type of Message | M |  | CHOICE (Request, Result, …) | Refer to Table 5.1.2-3 for applicable choices for each procedure type |

5.3.3.2	A1 Policy Type ID

Defined in 10.1.5.

5.3.3.3	A1 Policy ID

The A1 Policy ID IE identifies one A1 policy under one A1 policy type.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| A1 Policy ID | M |  | STRING |  |

5.3.3.4	Result

The Result IE indicates the procedure succeeds or not.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Result | M |  | ENUMERATED (success, failure, …) |  |

5.3.3.5	Cause

The Cause IE indicates the reason for a particular event in A1-related API.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >A1-related API Layer |  |  |  |  |
| >>A1-related API Layer Cause | M |  | ENUMERATED (Unspecified A1-related API layer reason, Not supported A1 policy type, Not allowed statements, restrictions and extensions, Near-RT RIC Platform cannot accommodate E2 / A1-EI operations requirements, A1 policy already enabled at xApp, Not enabled A1 policy at xApp, …) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified transport layer reason, Transport Resource Unavailable, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Unspecified protocol reason, Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified miscellaneous reason, Control Processing Overload Hardware failure, O&M intervention, …) |  |

The meaning of the different cause values is described in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| A1-related API Layer cause | Meaning |
| --- | --- |
| Unspecified A1-related API layer reason | Sent for A1-related API layer cause when none of the specified cause values applies. |
| Not supported A1 policy type | The A1 policy type is not supported. |
| Not allowed statements, restrictions and extensions | The A1 policy object is illegal due to violation of statements, restrictions and extensions required for corresponding A1 policy type. |
| Near-RT RIC Platform cannot accommodate E2 / A1-EI operations requirements | The RIC platform failed to setup xApp’s requested E2 / A1-EI operation(s). |
| A1 policy already enabled at xApp | The A1 policy indicated by A1 Policy Type ID and A1 Policy ID is enforced already. |
| Not enabled A1 policy at xApp | The A1 policy indicated by A1 Policy Type ID and A1 Policy ID is not enforced. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified transport layer reason | Sent for transport layer cause when none of the specified cause values applies. |
| Transport Resource Unavailable | The required transport resources are not available. |

| Protocol cause | Meaning |
| --- | --- |
| Unspecified protocol reason | Sent for protocol cause when none of the specified cause values applies. |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated “reject”. |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated “ignore and notify”. |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Unspecified miscellaneous reason | Sent for miscellaneous cause when none of the specified cause values applies. |
| Control Processing Overload | Control processing overload |
| Hardware Failure | Action related to hardware failure |
| O&M Intervention | The action is due to O&M intervention. |

5.3.4	Message and Information Element Abstract Syntax

5.3.4.1	Solution 1: gRPC with Protocol Buffers

5.3.4.1.1			Usage of gRPC with Protocol Buffers

This solution uses Protocol Buffers as message encoding, and also as the Interface Definition Language (IDL) for gRPC [5].

Table 5.3.4.1.1-1 lists the mapping of A1-related APIs and corresponding gRPC services.

Table 5.3.4.1.1-1: Mapping of A1-related APIs and gRPC services

| API Name | gRPC service(s) |
| --- | --- |
| A1_Policy_API | A1_Policy_API |

A procedure of an A1-related API is mapped to a gRPC method.

Table 5.3.4.1.1-2 lists the A1-related APIs abstract syntax included in this document.

Table 5.3.4.1.1-2: List of A1-related APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| A1_Policy_API | 5.3.4.1.2.1 | 1.0.0 | a1_policy_api_ver_1_0_0.proto |
| A1-related API common information elements | 5.3.4.1.3 | 1.0.0 | a1_related_apis_common_ies_ver_1_0_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

5.3.4.1.2			API Definitions

5.3.4.1.2.1	A1_Policy_API

syntax = "proto3";

package ricapi.a1_policy_api.v1;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "a1_related_apis_common_ies_ver_1_0_0.proto";

// gRPC service and rpc definition

service A1_Policy_API {

rpc A1PolicySetupProc(A1PolicySetupInitMsg) returns (A1PolicySetupOutMsg) {};

rpc A1PolicyUpdateProc(A1PolicyUpdateInitMsg) returns (A1PolicyUpdateOutMsg) {};

rpc A1PolicyDeleteProc(A1PolicyDeleteInitMsg) returns (A1PolicyDeleteOutMsg) {};

}

message A1PolicySetupInitMsg {

optional A1PolicySetupRequest a1_policy_setup_request = 1;

}

message A1PolicySetupOutMsg {

oneof type_of_message {

A1PolicySetupResult a1_policy_setup_result = 1;

}

}

message A1PolicyUpdateInitMsg {

optional A1PolicyUpdateRequest a1_policy_update_request = 1;

}

message A1PolicyUpdateOutMsg {

oneof type_of_message {

A1PolicyUpdateResult a1_policy_update_result = 1;

}

}

message A1PolicyDeleteInitMsg {

optional A1PolicyDeleteRequest a1_policy_delete_request = 1;

}

message A1PolicyDeleteOutMsg {

oneof type_of_message {

A1PolicyDeleteResult a1_policy_delete_result = 1;

}

}

// ---------------- messages of a1_policy_API ----------------

// -------- messages of A1 Policy Setup procedure --------

message A1PolicySetupRequest {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

optional a1_related_apis_common_ies.v1.A1PolicyId a1_policy_id = 2;

optional a1_related_apis_common_ies.v1.A1PolicyObject a1_policy_object = 3;

}

message A1PolicySetupResult {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

optional a1_related_apis_common_ies.v1.A1PolicyId a1_policy_id = 2;

optional a1_related_apis_common_ies.v1.Result a1_policy_setup_result = 3;

optional a1_related_apis_common_ies.v1.Cause cause = 4;

optional a1_related_apis_common_ies.v1.A1PolicyObject a1_policy_object = 5;

}

// -------- messages of A1 Policy Update procedure --------

message A1PolicyUpdateRequest {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

optional a1_related_apis_common_ies.v1.A1PolicyId a1_policy_id = 2;

optional a1_related_apis_common_ies.v1.A1PolicyObject a1_policy_object = 3;

}

message A1PolicyUpdateResult {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

optional a1_related_apis_common_ies.v1.A1PolicyId a1_policy_id = 2;

optional a1_related_apis_common_ies.v1.Result a1_policy_update_result = 3;

optional a1_related_apis_common_ies.v1.Cause cause = 4;

optional a1_related_apis_common_ies.v1.A1PolicyObject a1_policy_object = 5;

}

// -------- messages of A1 Policy Delete procedure --------

message A1PolicyDeleteRequest {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

optional a1_related_apis_common_ies.v1.A1PolicyId a1_policy_id = 2;

}

message A1PolicyDeleteResult {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

optional a1_related_apis_common_ies.v1.A1PolicyId a1_policy_id = 2;

optional a1_related_apis_common_ies.v1.Result a1_policy_delete_result = 3;

optional a1_related_apis_common_ies.v1.Cause cause = 4;

}

5.3.4.1.3			Information Element Definitions

syntax = "proto3";

package ricapi.a1_related_apis_common_ies.v1;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

// ----------------------------- Information Elements -----------------------------

// --------------A--------------

message A1PolicyObject {

optional string value = 1;

}

message A1PolicyId {

optional string value = 1;

}

// --------------C--------------

message Cause {

oneof cause_group {

A1RelatedApiLayerCause a1_related_api_layer_cause = 1;

TransportLayerCause transport_layer_cause = 2;

ProtocolLayerCause protocol_layer_cause = 3;

MiscCause misc_cause = 4;

}

enum A1RelatedApiLayerCause {

A1_RELATED_API_LAYER_UNSPECIFIED = 0;

NOT_SUPPORTED_A1_POLICY_TYPE = 1;

NOT_ALLOWED_STATEMENTS_RESTRICTIONS_AND_EXTENSIONS = 2;

NEAR_RT_RIC_PLATFORM_CANNOT_ACCOMMODATE_E2_A1_EI_OPERATIONS_REQUIREMENTS = 3;

A1_POLICY_ALREADY_ENABLED_AT_XAPP = 4;

NOT_ENABLED_A1_POLICY_AT_XAPP = 5;

}

enum TransportLayerCause {

TRANSPORT_UNSPECIFIED = 0;

TRANSPORT_RESOURCE_UNAVAILABLE = 1;

}

enum ProtocolLayerCause {

PROTOCOL_UNSPECIFIED = 0;

TRANSFER_SYNTAX_ERROR = 1;

ABSTRACT_SYNTAX_ERROR_REJECT = 2;

ABSTRACT_SYNTAX_ERROR_IGNORE_AND_NOTIFY = 3;

MESSAGE_NOT_COMPATIBLE_WITH_RECEIVER_STATE = 4;

SEMANTIC_ERROR = 5;

ABSTRACT_SYNTAX_ERROR_FALSELY_CONSTRUCTED_MESSAGE = 6;

}

enum MiscCause {

MISC_UNSPECIFIED = 0;

CONTROL_PROCESSING_OVERLOAD = 1;

HARDWARE_FAILURE = 2;

OAM_INTERVENTION = 3;

}

}

// --------------R--------------

enum Result {

RESULT_SUCCESS = 0;

RESULT_FAILURE = 1;

}

5.3.5	Message Transfer Syntax

5.3.5.1	Solution 1: gRPC with Protocol Buffers

The message transfer syntax for Solution 1 is serialized Protocol Buffers [4].

6	E2-related API

6.1	Overview

6.1.1	Protocol Stack

In this version of specification, the default and optional protocol stacks for the E2 related APIs are shown on Figure 6.1.1-1.

Note: the network layer for security protection is FFS.

| Default protocol stack : SCTP | Optional protocol stack : gRPC |
| --- | --- |

Figure 6.1.1-1: Protocol stacks for E2 related APIs

The E2 related APIs may use SCTP or gRPC with Protocol Buffers [4] as the application layer serialization protocol. The supported protocol by the platform APIs is presented to the the xApps during the xApp Discovery Procedure in clause 9.2.1.

6.1.2	List of E2 related APIs, procedures and messages

The E2 related API procedures introduced in clause 9.3 of [2] are categorized under the E2 related APIs. These APIs include:

E2 Subscription API: enabling xApp to setup, modify and delete E2 subscriptions with the Near-RT RIC platform.

E2 Indication API: enabling xApp to receive E2 indications associated with an E2 subscription.

E2 Control API: enabling xApp to consume E2 control service from the Near-RT RIC platform.

E2 Guidance API: enabling xApp to acquire E2 related guidance from the Near-RT RIC platform and mitigate potential conflicts with other xApps.

These APIs are summarized in Table 6.1.2-1.

Table 6.1.2-1: List of E2 related APIs

| API Name | Description |
| --- | --- |
| E2_Subscription_API | Near-RT RIC platform service for subscription of E2 REPORT, INSERT and POLICY services |
| E2_Indication_API | Near-RT RIC platform service for delivery of subscribed information associated with E2 REPORT and INSERT services |
| E2_Control_API | Near-RT RIC platform service for E2 CONTROL |
| E2_Guidance_API | Near-RT RIC platform service for E2 related guidance |

The table 6.1.2-2 lists the API procedures for each E2 related API.

Table 6.1.2-2: List of E2 related API procedures

| API Name | API procedure | Communication type |
| --- | --- | --- |
| E2_Subscription_API | E2 Subscription | Request/Response |
|  | E2 Subscription Delete | Request/Response |
|  | E2 Subscription Delete Query | Request/Response |
|  | E2 Subscription Delete Notification | Notify |
| E2_Indication_API | E2 Indication | Notify |
| E2_Control_API | E2 Control | Request/Response |
| E2_Guidance_API | E2 Guidance | Request/Response |
|  | E2 Guidance Modification | Notify |

The table 6.1.2-3 lists the messages for each E2 related API procedure.

Table 6.1.2-3: List of E2 related API messages

| API Procedure | Initiated by | Initiating Message | Outcome Message |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Successful Outcome | Unsuccessful Outcome 1 | Unsuccessful Outcome 2 |
| E2 Subscription | xApp | E2 SUBSCRIPTION REQUEST | E2 SUBSCRIPTION SUCCESS | E2 SUBSCRIPTION FAILURE | E2 SUBSCRIPTION REJECT |
| E2 Subscription Delete | xApp | E2 SUBSCRIPTION DELETE REQUEST | E2 SUBSCRIPTION DELETE SUCCESS | E2 SUBSCRIPTION DELETE FAILURE | E2 SUBSCRIPTION DELETE REJECT |
| E2 Subscription Delete Query | Near-RT RIC platform | E2 SUBSCRIPTION DELETE REQUIRED | E2 SUBSCRIPTION DELETE ACCEPT | E2 SUBSCRIPTION DELETE DECLINE | - |
| E2 Subscription Delete Notification | Near-RT RIC platform | E2 SUBSCRIPTION DELETE NOTIFICATION | - | - | - |
| E2 Indication | Near-RT RIC platform | E2 INDICATION PUSH | - | E2 INDICATION FAILURE | - |
| E2 Control | xApp | E2 CONTROL REQUEST | E2 CONTROL SUCCESS | E2 CONTROL FAILURE | E2 CONTROL REJECT |
| E2 Guidance | xApp | E2 GUIDANCE REQUEST | E2 GUIDANCE SUCCESS | - | - |
| E2 Guidance Modification | Near-RT RIC platform | E2 GUIDANCE MODIFICATION | - | - | - |

6.2	E2-related API Procedures

6.2.1	E2_Subscription_API procedures

6.2.1.1	E2 Subscription Procedure

6.2.1.1.1	General

This procedure is used by an xApp to request an E2 subscription to Near-RT RIC platform.

6.2.1.1.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 SUBSCRIPTION (Request)

near -> app: E2 SUBSCRIPTION (Success)

@enduml

Figure 6.2.1.1.2-1: E2 Subscription procedure, successful operation

The xApp initiates the procedure by sending an E2 SUBSCRIPTION (Request) message to the Near-RT-RIC platform. The message contains a unique Procedure Transaction ID IE assigned by the initiating xApp, the Global E2 Node ID IE, the RAN Function ID IE, and the E2 Subscription Details IE including one or more requested E2 actions and optionally the E2 Subscription Timer IE. The message may include the E2 Indication Destination IE which specifies the xApp's endpoint to receive E2 indications (see clause 6.2.2.1), and the E2 Subscription Delete Notification Destination IE which specifies the xApp's endpoint to receive query/notification from the Near-RT RIC platform for E2 subscription deletion (see clause 6.2.1.3/6.2.1.4).

Note: When execution order of multiple actions is not concerned, it is recommended that an E2 Subscription procedure contains a single action, to allow efficient reuse of E2 subscriptions.

At reception of the E2 SUBSCRIPTION (Request) message, the Near-RT-RIC platform shall:

-	Search existing E2 subscription(s) for matched Global E2 Node ID IE, RAN Function ID IE and matched contents in E2 Subscription Details IE. If a matched E2 subscription exists, add the initiating xApp as a new subscriber of the E2 subscription.

-	Initiate appropriate E2AP procedure(s) to establish RIC subscription with the target E2 Node, if existing E2 subscriptions are insufficient to fulfill the request.

The Near-RT-RIC platform may use the value in the E2 Subscription Timer IE, if present, as a one-time value of the E2AP timer TRICEVENTcreate in clause 9.5 of [3], which is valid only for this transaction.

If the subscription request is successfully fulfilled, the Near-RT RIC platform shall send an E2 SUBSCRIPTION (Success) message back to the xApp. Similar to [3], a successfully fulfilled request shall be sent if the E2 event trigger and at least one requested E2 action are admitted.

In the E2 SUBSCRIPTION (Success) message, Near-RT RIC platform shall include the associated Procedure Transaction ID IE, Global E2 Node ID IE, RAN Function ID IE and an E2 Request ID IE assigned by the Near-RT RIC platform. The message shall also indicate the admitted E2 action(s) in the Actions Admitted List IE, each with the associated E2 Action ID IE assigned by the Near-RT RIC platform.  The same E2 Request ID IE and E2 Action ID IE shall be present in any subsequent E2 INDICATION message(s). The message shall indicate, if exist, the E2 action(s) not admitted in the Actions Not Admitted List IE with appropriate cause(s).

6.2.1.1.3	Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 SUBSCRIPTION (Request)

near -> app: E2 SUBSCRIPTION (Reject)

@enduml

Figure 6.2.1.1.3-1: E2 Subscription procedure, unsuccessful operation (Reject)

If the subscription request is rejected by the Near-RT RIC platform, an E2 SUBSCRIPTION (Reject) message shall be sent back to the xApp. The message shall include associated Procedure Transaction ID IE which is replicated from the corresponding request, the Global E2 Node ID IE, the RAN Function ID IE and appropriate Cause IE.

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 SUBSCRIPTION (Request)

near -> app: E2 SUBSCRIPTION (Failure)

@enduml

Figure 6.2.1.1.3-2: E2 Subscription procedure, unsuccessful operation (Failure)

If the subscription request fails due to failure in E2AP procedure, the Near-RT RIC platform shall send an E2 SUBSCRIPTION (Failure) message back to the xApp. The message shall include the associated Procedure Transaction ID IE which is replicated from the corresponding request, Global E2 Node ID IE, RAN Function ID IE and appropriate cause.

6.2.1.1.4	Abnormal Conditions

Not applicable.

6.2.1.2	E2 Subscription Delete Procedure, xApp initiated

6.2.1.2.1	General

This procedure is used by an xApp to unsubscribe from an E2 subscription on Near-RT RIC platform.

6.2.1.2.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 SUBSCRIPTION DELETE (Request)

near -> app: E2 SUBSCRIPTION DELETE (Success)

@enduml

Figure 6.2.1.2.2-1: E2 Subscription Delete procedure (xApp initiated), successful operation

The xApp initiates the procedure by sending the E2 SUBSCRIPTION DELETE (Request) message to the Near-RT-RIC platform. The message contains the Global E2 Node ID IE and the E2 Request ID IE to indicate the target E2 subscription and optionally the E2 Subscription Delete Timer IE.

At reception of the E2 SUBSCRIPTION DELETE (Request) message, the Near-RT-RIC platform shall, if the delete request is accepted by the Near-RT-RIC platform:

-	Check for other subscriber(s) of the target E2 subscription. If such subscriber(s) exists, the E2 subscription shall be maintained by the Near-RT RIC platform but no longer serve the initiating xApp; if not, the E2 subscription shall be deleted.

-	Initiate appropriate E2AP procedure(s) with the target E2 Node, if necessary, to map the change in the target E2 subscription to its corresponding RIC subscription.

The Near-RT-RIC platform may use the value in the E2 Subscription Delete Timer IE, if present, as a one-time value of the E2AP timer TRICEVENTdelete in clause 9.5 of [3], which is valid only for this transaction.

Upon success of the above processing for the target E2 subscription, the Near-RT RIC platform shall stop delivering its associated E2 INDICATION message(s) to the initiating xApp and respond the xApp with an E2 SUBSCRIPTION DELETE(Success) message. The E2 SUBSCRIPTION DELETE (Success) message shall include the same Global E2 Node ID IE and E2 Request ID IE from the request message.

6.2.1.2.3	Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 SUBSCRIPTION DELETE (Request)

near -> app: E2 SUBSCRIPTION DELETE (Reject)

@enduml

Figure 6.2.1.2.3-1: E2 Subscription Delete procedure (xApp initiated), unsuccessful operation (Reject)

If the Near-RT RIC platform rejects the request message, an E2 SUBSCRIPTION (Reject) message shall be sent back to the initiating xApp. The message shall include associated Procedure Transaction ID IE which is replicated from the corresponding request, the Global E2 Node ID IE, the RAN Function ID IE and appropriate Cause IE.

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 SUBSCRIPTION DELETE (Request)

near -> app: E2 SUBSCRIPTION DELETE (Failure)

@enduml

Figure 6.2.1.2.3-2: E2 Subscription Delete procedure (xApp initiated), unsuccessful operation (Failure)

If the associated E2AP procedure(s) fails for the E2 subscription, the Near-RT RIC platform shall send an E2 SUBSCRIPTION DELETE (Failure) message back to the initiating xApp.   The message shall include associated Procedure Transaction ID IE which is replicated from the corresponding request, the Global E2 Node ID IE, the RAN Function ID IE and appropriate Cause IE.

6.2.1.2.4	Abnormal Conditions

Not applicable.

6.2.1.3	E2 Subscription Delete Query procedure

6.2.1.3.1	General

This procedure is used by Near-RT RIC platform to consult the xApp about deleting an existing E2 subscription.

6.2.1.3.2	Successful Operations

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

near -> app: E2 SUBSCRIPTION DELETE QUERY (Required)

app -> near: E2 SUBSCRIPTION DELETE QUERY (Accept)

@enduml

Figure 6.2.1.3.2-1: E2 Subscription Delete Query procedure, successful operation

The Near-RT RIC platform may consult each concerned xApp before deciding to delete an E2 subscription. The Near-RT RIC platform initiates the procedure by sending an E2 SUBSCRIPTION DELETE Query (Required) message to the xApp. The message contains the Global E2 Node ID IE and the E2 Request ID IE to identify the target E2 subscription.

At reception of the E2 SUBSCRIPTION DELETE Query (Required) message, the xApp shall respond by an E2 SUBSCRIPTION DELETE Query (Accept) message, if the xApp agrees to delete the E2 subscription based on its own evaluation. The reply message shall contain the same Global E2 Node ID IE, E2 Request ID IE, and RAN Function ID IE as in the corresponding E2 SUBSCRIPTION DELETE (Required) message. The Near-RT RIC platform should consider the suggestions from all concerned xApps, but the decision for the E2 subscription is up to the Near-RT RIC platform.

6.2.1.3.3	Unsuccessful Operation

If the xApp disagrees to delete the E2 subscription proposed in the E2 SUBSCRIPTION DELETE Query (Required) message, it shall reply an E2 SUBSCRIPTION DELETE Query (Decline) message to the Near-RT RIC platform.

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

near -> app: E2 SUBSCRIPTION DELETE QUERY (Required)

app -> near: E2 SUBSCRIPTION DELETE QUERY (Decline)

@enduml

Figure 6.2.1.3.3-2: E2 Subscription Delete Query procedure, unsuccessful operation

6.2.1.3.4	Abnormal Conditions

Not applicable.

6.2.1.4	E2 Subscription Delete Notification procedure

6.2.1.4.1	General

This procedure is used by Near-RT RIC platform to notify the xApp that an existing E2 subscription has been deleted as required by the E2 Node.

6.2.1.4.2	Successful Operations

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

near -> app: E2 SUBSCRIPTION DELETE NOTIFICATION (Deleted)

@enduml

Figure 6.2.1.4.2-1: E2 Subscription Delete Notification procedure, successful operation

After successfully deleting the E2 subscription required by the E2 Node (including removal of the RIC subscription over E2 interface and removal of corresponding subscription record in Near-RT RIC platform), the Near-RT RIC platform shall send an E2 SUBSCRIPTION DELETE NOTIFICATION (Deleted) message to the xApp to notify the deletion.

The message shall contain the Global E2 Node ID IE and the E2 Request ID IE to identify the target E2 subscription, and the Cause IE with appropriate value.

At reception of the message, the xApp shall remove its local subscription record of the E2 subscription.

6.2.1.4.3	Unsuccessful Operations

Not applicable.

6.2.1.4.4	Abnormal Conditions

Not applicable.

6.2.2	E2_Indication_API procedures

6.2.2.1	E2 Indication Procedure

6.2.2.1.1	General

This procedure is used by the Near-RT RIC platform to deliver to xApp the subscribed information associated with an E2 subscription. The procedure is triggered by the successful operation of an E2AP RIC indication procedure.

6.2.2.1.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

near -> app: E2 INDICATION (Push)

@enduml

Figure 6.2.2.1.2-1: E2 Indication procedure, successful operation

The Near-RT-RIC platform initiates the procedure by sending the E2 INDICATION (Push) message containing the Global E2 Node ID IE, the E2 Request ID IE, the E2 Action ID IE to the xApp. The message shall also contain the E2 Indication Type IE, the E2 Indication Header IE and the E2 Indication Message IE to carry the information from corresponding fields, i.e. the RIC Indication Type IE, the RIC Indication Header IE, and the RIC Indication Message IE, respectively, in the E2AP RIC INDICATION message. The E2 Indication SN IE and the E2 Call Process ID IE shall also be included, if corresponding fields, i.e. the RIC Indication SN IE, and the RIC Call Process ID IE, respectively, are present in the E2AP RIC INDICATION message.

At reception of the E2 INDICATION (Push) message an xApp shall:

- 	Use the Global E2 Node ID IE, the E2 Request ID IE and the E2 Action ID IE to associate the received message to the right E2 subscription. The E2 Request ID IE and the E2 Action ID IE have been assigned in the E2 SUBSCRIPTION (Success) message for each E2 subscription.

- 	Process the information carried in the E2 Indication SN IE, the E2 Indication Type IE, the E2 Indication Header IE, the E2 Indication Message IE and the E2 Call Process ID IE.

6.2.2.1.3	Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

near -> app: E2 INDICATION (Push)

app --> near: E2 INDICATION (Failure)

@enduml

Figure 6.2.2.1.3-1: E2 Indication procedure, unsuccessful operation (Failure)

If the received E2 INDICATION (Push) message cannot be handled successfully as specified in 6.2.2.1.2, the xApp may, according to operator’s OM configuration, send E2 INDICATION (Failure) message back to the Near-RT RIC platform. The message shall include the associated Global E2 Node ID IE, E2 Request ID IE, RAN Function ID IE, E2 Action ID IE, and appropriate Cause IE. The E2 Indication SN IE shall also be included if the same name IE is present in the corresponding E2 INDICATION (Push) message.

6.2.2.1.4	Abnormal Conditions

Not applicable.

6.2.3	E2_Control_API procedures

6.2.3.1	E2 Control procedure

6.2.3.1.1	General

This procedure is used by an xApp to request for E2 control service to Near-RT RIC platform.

6.2.3.1.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 CONTROL (Request)

near  app: E2 CONTROL (Success)

@enduml

Figure 6.2.3.1.2-1: E2 Contol procedure, successful operation

The xApp initiates the procedure by sending the E2 CONTROL (Request) message to the Near-RT-RIC platform, containing the Global E2 Node ID IE, the RAN Function ID IE, optionally the E2 Call Process ID IE, E2 Control Header IE, E2 Control Message IE, optionally the E2 Control Ack Request IE and optionally the E2 Control Timer IE.

At reception of the E2 CONTROL (Request) message, the Near-RT-RIC platform shall:

-	Evaluate whether to reject the request due to potential control conflict, when such evaluation is required by conflict mitigation related configuration.

-	Initiate appropriate E2AP RIC Control procedure with the target E2 Node, if the request is not rejected by the Near-RT-RIC platform.

The Near-RT-RIC platform may use the value in the E2 Control Timer IE, if present, as a one-time value of the E2AP timer TRICcontrol in Clause 9.5 of [3], which is valid only for this transaction.

If the control request is successfully performed, and if the optional E2 Control Ack Request IE in the request is absent or set to “Ack”, an E2 CONTROL (Success) message shall be responded to the initiating xApp. The E2 CONTROL (Success) message shall include the Global E2 Node ID IE, the RAN Function ID IE, optionally the E2 Call Process ID IE, and optionally the E2 Control Outcome IE. The xApp may use the information contained in the optional E2 Control Outcome IE to determine subsequent actions.

If the control request is successfully performed, and if the optional E2 Control Ack Request IE in the request is set to “NoAck”, an E2 CONTROL (Success) message is not needed and xApp may consider it success.

6.2.3.1.3	Unsuccessful Operations

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 CONTROL (Request)

near -> app: E2 CONTROL (Reject)

@enduml

Figure 6.2.3.1.3-1: E2 Control procedure, unsuccessful operation (Reject)

If the request is rejected by the Near-RT RIC platform, an E2 CONTROL (Reject) message shall be responded to the initiating xApp with appropriate Cause IE.  The xApp may use the information contained in the Cause IE to determine subsequent actions.

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: E2 CONTROL (Request)

near -> app: E2 CONTROL (Failure)

@enduml

Figure 6.2.3.1.3-2: E2 Control procedure, unsuccessful operation (Failure)

If the request is accepted by the Near-RT RIC platform but failed in the E2AP RIC Control procedure, the Near-RT RIC platform shall send an E2 CONTROL (Failure) message back to the xApp with appropriate Cause IE. The xApp may use the information contained in the Cause IE and the optional E2 Control Outcome IE to determine subsequent actions.

6.2.3.1.4	Abnormal Conditions

Not applicable.

6.3	Elements for E2-related API Communication

6.3.1	General

6.3.2	Message Functional Definition and Content

6.3.2.1	E2 SUBSCRIPTION (Request)

This message is sent by an xApp to the Near-RT RIC platform for an E2 Subscription.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value assigned by the initiating xApp. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Subscription Details |  |  |  |  | YES | Reject |
| >E2 Event Trigger Definition | M |  | RIC Event Trigger Definition, Clause 9.2.9 of [3] |  | - |  |
| >Sequence of Actions |  | 1.. <maxnoofE2Actions> |  |  | EACH | Ignore |
| >>Requested E2 Action ID | M |  | RIC Action ID, Clause 9.2.10 of [3] | Assigned by initiating xApp to uniquely identify an action within the sequence of actions. It should not be assumed that the E2 Action ID in the subsequent E2 SUBSCRIPTION (Success) message or RIC Action ID in the corresponding E2AP message has the same value. | - |  |
| >>E2 Action Type | M |  | RIC Action Type, Clause 9.2.11 of [3] |  | - |  |
| >>E2 Action Definition | O |  | RIC Action Definition, Clause 9.2.12 of [3] |  | - |  |
| >>E2 Subsequent Action | O |  | RIC Subscription Action, Clause 9.2.13 of [3] |  | - |  |
| E2 Subscription Timer | O |  | 6.3.3.7 |  |  |  |
| E2 Indication Destination | O |  | 10.1.9 | When present, this IE specifies the destination address for the Near-RT RIC platform to send E2 INDICATION (Push) messages. | YES | Reject |
| E2 Subscription Delete Notification Destination | O |  | 10.1.9 | When present, this IE specifies the destination address for the Near-RT RIC platform to send E2 SUBSCRIPTION DELETE QUERY (Required) and E2 SUBSCRIPTION DELETE NOTIFICATION (Deleted) messages. | YES | Reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofE2Actions | Maximum no. of E2 Actions requested in the E2 subscription. Value is 16. |

6.3.2.2	E2 SUBSCRIPTION (Success)

This message is sent by the Near-RT RIC platform after the xApp’s subscription request is successfully accepted with respect to the target E2 Node.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 | Value replicated from corresponding request. | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 | Value assigned by the Near-RT RIC platform to indicate the associated E2 subscription in Near-RT RIC. | YES | Reject |
| RAN Function ID | M |  | 10.1.7 | Value replicated from corresponding request. | YES | Reject |
| Actions Admitted List |  | 1.. <maxnoofE2Actions> |  |  | YES | Reject |
| >Requested E2 Action ID | M |  | RIC Action ID, Clause 9.2.10 of [3] | Value replicated from corresponding request message | - | - |
| >E2 Action ID | M |  | RIC Action ID, Clause 9.2.10 of [3] | Value assigned by Near-RT RIC platform to indicate in the associated E2 subscription the action ID that corresponds to the Requested E2 Action ID IE in the E2 SUBSCRIPTION REQUEST message. | - | - |
| Actions Not Admitted List |  | 0.. <maxnoofE2Actions> |  |  | YES | Reject |
| >Requested E2 Action ID | M |  | RIC Action ID, Clause 9.2.10 of [3] | Value replicated from corresponding request message | - | - |
| >Cause | M |  | 6.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofE2Actions | Maximum no. of E2 Actions requested in the E2 subscription. Value is 16. |

6.3.2.3	E2 SUBSCRIPTION (Reject)

This message is sent by the Near-RT RIC platform after the xApp’s subscription request is rejected by Near-RT RIC platform with respect to the target E2 Node.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 | Value replicated from corresponding request. | YES | Reject |
| RAN Function ID | M |  | 10.1.7 | Value replicated from corresponding request. | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Reject |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.4	E2 SUBSCRIPTION (Failure)

This message is sent by the Near-RT RIC platform after the xApp’s subscription request is accepted by Near-RT RIC platform but failed by E2 Node, with respect to the target E2 Node.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 | Value replicated from corresponding request. | YES | Reject |
| RAN Function ID | M |  | 10.1.7 | Value replicated from corresponding request. | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Reject |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.5	E2 SUBSCRIPTION DELETE (Request)

This message is sent by the xApp to the Near-RT RIC platform to request deletion of one target E2 subscription.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Subscription Delete Timer | O |  | 6.3.3.8 |  |  |  |

6.3.2.6	E2 SUBSCRIPTION DELETE (Success)

This message is sent by the Near-RT RIC platform to the xApp to indicate success for the delete request of the target E2 subscription.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 | Value replicated from corresponding request. | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 | Value replicated from corresponding request. | YES | Reject |
| RAN Function ID | M |  | 10.1.7 | Value replicated from corresponding request. | YES | Reject |

6.3.2.7	E2 SUBSCRIPTION DELETE (Reject)

This message is sent by the Near-RT RIC platform to the xApp when the Near-RT RIC platform rejects the delete request for the target E2 subscription.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 | Value replicated from corresponding request. | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 | Value replicated from corresponding request. | YES | Reject |
| RAN Function ID | M |  | 10.1.7 | Value replicated from corresponding request. | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.8	E2 SUBSCRIPTION DELETE (Failure)

This message is sent by the Near-RT RIC platform to the xApp when the delete request for a target E2 subscription fails due to failure in the associated E2AP procedure(s).

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 | Value replicated from corresponding request. | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 | Value replicated from corresponding request. | YES | Reject |
| RAN Function ID | M |  | 10.1.7 | Value replicated from corresponding request. | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.9	E2 SUBSCRIPTION DELETE QUERY (Required)

This message is sent by the Near-RT RIC platform to the xApp to consult the deletion of a target E2 subscription.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |

6.3.2.10	E2 SUBSCRIPTION DELETE QUERY (Accept)

This message is sent by the xApp to the Near-RT RIC platform to agree to deletion of a target E2 subscription.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |

6.3.2.11	E2 SUBSCRIPTION DELETE QUERY (Decline)

This message is sent by the xApp to the Near-RT RIC platform to decline deletion of a target E2 subscription.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |

6.3.2.12	E2 SUBSCRIPTION DELETE NOTIFICATION (Deleted)

This message is sent by the Near-RT RIC platform to the xApp to notify the deletion of a target E2 subscription.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |

6.3.2.13	E2 INDICATION (Push)

This message is sent by the Near-RT RIC platform to the xApp for delivery of subscribed information associated with an E2 subscription.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Action ID | M |  | RIC Action ID, Clause 9.2.10 of [3]. |  |  |  |
| E2 Indication SN | O |  | RIC Indication SN, Clause 9.2.14 of [3]. |  | YES | Reject |
| E2 Indication Type | M |  | RIC Indication Type, Clause 9.2.15 of [3]. |  | YES | Reject |
| E2 Indication Header | M |  | RIC Indication Header, Clause 9.2.17 of [3]. |  | YES | Reject |
| E2 Indication Message | M |  | RIC Indication Message, Clause 9.2.16 of [3]. |  | YES | Reject |
| E2 Call Process ID | O |  | RIC Call Process ID, Clause 9.2.18 of [3]. |  | YES | Reject |

6.3.2.14	E2 INDICATION (Failure)

This message is sent by the xApp to the Near-RT RIC platform if an error is detected in E2 INDICATION (Push) message.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| E2 Request ID | M |  | 6.3.3.2 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Action ID | M |  | RIC Action ID, Clause 9.2.10 of [3]. |  |  |  |
| E2 Indication SN | O |  | RIC Indication SN, Clause 9.2.14 of [3]. |  | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Reject |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.15	E2 CONTROL (Request)

This message is sent by the xApp to the Near-RT RIC platform for an E2 control request.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value assigned by the initiating xApp. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Call Process ID | O |  | RIC Call Process ID, Clause 9.2.18 of [3]. |  | YES | Reject |
| E2 Control Header | M |  | RIC Control Header, Clause 9.2.20 of [3]. |  | YES | Reject |
| E2 Control Message | M |  | RIC Control Message, Clause 9.2.19 of [3]. |  | YES | Reject |
| E2 Control Ack Request | O |  | RIC Control Ack Request, Clause 9.2.21 of [3]. |  | YES | Reject |
| E2 Control Timer | O |  | 6.3.3.6 |  | YES | Ignore |

6.3.2.16	E2 CONTROL (Success)

This message is sent by the Near-RT RIC platform to the xApp for a successfully performed E2 control request.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Call Process ID | O |  | RIC Call Process ID, Clause 9.2.18 of [3]. |  | YES | Reject |
| E2 Control Outcome | O |  | RIC Control Outcome, Clause 9.2.25 of [3]. |  | YES | Reject |

6.3.2.17	E2 CONTROL (Reject)

This message is sent by the Near-RT RIC platform to the xApp if the corresponding E2 control request is rejected by the Near-RT RIC platform.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.18	E2 CONTROL (Failure)

This message is sent by the Near-RT RIC platform to the xApp if the corresponding E2 control request is accepted by the Near-RT RIC platform but fails in the E2 Node.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 6.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Global E2 Node ID | M |  | 10.1.6 |  | YES | Reject |
| RAN Function ID | M |  | 10.1.7 |  | YES | Reject |
| E2 Call Process ID | O |  | RIC Call Process ID, Clause 9.2.18 of [3]. |  | YES | Reject |
| E2 Control Outcome | O |  | RIC Control Outcome, Clause 9.2.25 of [3]. |  | YES | Reject |
| Cause | M |  | 6.3.3.4 |  | YES | Ignore |
| Criticality Diagnostics | O |  | 6.3.3.5 |  | YES | Ignore |

6.3.2.19	E2 GUIDANCE (Request)

6.3.2.20	E2 GUIDANCE (Response)

6.3.2.21	E2 GUIDANCE (Modification)

6.3.3	Information Element Definitions

6.3.3.1	Message Type

This IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Type | M |  | CHOICE (E2 Subscription, E2 Subscription Delete, E2 Subscription Delete Query, E2 Subscription Delete Notification, E2 Indication, E2 Control, …) |  |
| >Type of Message | M |  | CHOICE (Request, Success, Reject, Failure, …) | Refer to Table 6.1.2-3 for applicable choices for each procedure type |

6.3.3.2	E2 Request ID

This IE is assigned by Near-RT RIC platform to uniquely identify an E2 subscription among all the E2 subscriptions associated with the same E2 Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Request ID | M |  | INTEGER (0 … 4294967295) |  |

6.3.3.3	Void

6.3.3.4	Cause

This IE is used to indicate the reason for a particular event for the E2-related APIs.

Note:	Not all causes from E2AP are applicable.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >E2-related API Cause |  |  |  |  |
| >>E2-related API Cause | M |  | 6.3.3.4a. |  |
| >E2AP Cause |  |  |  |  |
| >>E2AP Cause | M |  | Cause, Clause 9.2.1 of [3]. |  |

#### 6.3.3.4a	E2-related API Cause

The purpose of the E2-related API Cause IE is to indicate the reason for a particular event for the E2-related APIs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE E2-related API Cause Group | M |  |  |  |
| >Application Layer |  |  |  |  |
| >>Application Layer Cause | M |  | ENUMERATED (Unspecified, Global E2Node ID invalid, E2 Node Reset or Disconnected, RAN Function ID invalid, Action not supported, Excessive actions, Duplicate action, E2 Request ID unknown, Control conflict, E2 Indication unknown, E2AP Procedure timeout, Essential for operation, …) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, ...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Unspecified, Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, Control Processing Overload, Hardware Failure, O&M Intervention, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Application Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the cause values below applies but still the cause is Application Layer related. |
| Global E2Node ID invalid | Global E2Node ID invalid. |
| E2 Node Reset or Disconnected | E2Node Status is Reset or Disconnected. |
| RAN Function ID invalid | Requested RAN Function ID invalid. |
| Action not supported | Requested Action not supported by RAN function. |
| Excessive actions | Excessive number of actions requested for RAN Function. |
| Duplicate action | Same action requested more than once in same subscription request. |
| E2 Request ID unknown | E2 Request ID sent to Near-RT RIC is unknown. |
| Control Conflict | Requested Control command causes conflict. |
| E2 Indication unknown | The E2 Indication message is unknown, i.e. The received indication message is not subscribed to by the xApp. |
| E2AP Procedure timeout | An E2AP procedure triggered by the E2-related API procedure is time-out. |
| Essential for operation | xApp is unwilling to delete an E2 subscription essential for its operation. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the cause values below applies but still the cause is Transport Layer related. |
| Transport Resource Unavailable | The required transport resources are not available. |

| Protocol cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the below cause values applies but still the cause is Protocol related. |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Unspecified Failure | Sent when none of the below cause values applies and the cause is not related to any of the categories Application Layer, Transport Layer, Protocol. |
| Control Processing Overload | Control processing overload. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |

6.3.3.5	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the xApp or the Near-RT RIC Platform when parts of a received message have not been comprehended or were missing. When applicable, it contains information about which IEs were not comprehended or were missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Element Criticality Diagnostics |  | 0 .. <maxnoofErrors> |  |  |
| >IE Criticality | M |  | ENUMERATED (reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value 'ignore' shall not be used. |
| >IE ID | M |  | ENUMERATED (Message Type, Procedure Transaction ID, Global E2 Node ID, RAN Function ID…) | The IE ID of the not understood or missing IE. |
| >Type of Error | M |  | ENUMERATED (not understood, missing, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofErrors | Maximum no. of IE errors allowed to be reported with a single message. Value is 256. |

6.3.3.6	E2 Control Timer

This IE specifies the suggested time to wait before timeout for the RIC Control procedure in E2AP[3].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Control Timer | M |  | INTEGER (10..10000, ...) | Unit in milliseconds. |

6.3.3.7	E2 Subscription Timer

This IE specifies the suggested time to wait before timeout for the RIC subscription procedure in E2AP[3].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Subscription Timer | M |  | INTEGER (10..10000, ...) | Unit in milliseconds. |

6.3.3.8	E2 Subscription Delete Timer

This IE specifies the suggested time to wait before timeout for the RIC subscription deletion procedure in E2AP[3].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Subscription Delete Timer | M |  | INTEGER (10..10000, ...) | Unit in milliseconds. |

6.3.4	Message and Information Element Abstract Syntax

6.3.4.1	Solution 1: SCTP with Protocol Buffers

6.3.4.1.1	Usage of SCTP with Protocol Buffers

This solution uses Protocol Buffers as message encoding, and SCTP as the transport protocol.

The PDU on the SCTP connection uses the structure as defined in clause 10.1.8 where its API Payload IE serves as a container for individual API messages in clause 6.3.4.1.2 which can be E2SubscriptionApiPayload or E2IndicationApiPayload or E2ControlApiPayload.

Table 6.3.4.1.1-1 lists the E2 related APIs abstract syntax included in this document.

Table 6.3.4.1.1-1: List of E2 related APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| E2_Subscription_API | 6.3.4.1.2.1 | 2.0.0 | e2_subscription_api_ver_2_0_0.proto |
| E2_Indication_API | 6.3.4.1.2.2 | 2.0.0 | e2_indication_api_ver_2_0_0.proto |
| E2_Control_API | 6.3.4.1.2.3 | 2.0.0 | e2_control_api_ver_2_0_0.proto |
| E2 related API common information elements | 6.3.4.1.3 | 1.1.0 | e2_related_apis_common_ies_ver_1_1_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

6.3.4.1.2	API Definitions

6.3.4.1.2.1	E2_Subscription_API

syntax = "proto3";

package ricapi.e2_subscription_api.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "e2_related_apis_common_ies_ver_1_1_0.proto";

// top-level PDU

message E2SubscriptionApiPayload {

oneof procedure_type {

E2SubscriptionProc e2_subscription_proc = 1;

E2SubscriptionDeleteProc e2_subscription_delete_proc = 2;

E2SubscriptionDeleteQueryProc e2_subscription_delete_query_proc = 3;

E2SubscriptionDeleteNotificationProc e2_subscription_delete_notification_proc = 4;

}

}

// ---------------- procedures of E2_Subscription_API ----------------

message E2SubscriptionProc {

oneof type_of_message {

E2SubscriptionRequest e2_subscription_request = 1 ;

E2SubscriptionSuccess e2_subscription_success = 2 ;

E2SubscriptionReject e2_subscription_reject = 3 ;

E2SubscriptionFailure e2_subscription_failure = 4 ;

}

}

message E2SubscriptionDeleteProc {

oneof type_of_message {

E2SubscriptionDeleteRequest e2_subscription_delete_request = 1 ;

E2SubscriptionDeleteSuccess e2_subscription_delete_success = 2 ;

E2SubscriptionDeleteReject e2_subscription_delete_reject = 3 ;

E2SubscriptionDeleteFailure e2_subscription_delete_failure = 4 ;

}

}

message E2SubscriptionDeleteQueryProc {

oneof type_of_message {

E2SubscriptionDeleteQueryRequired e2_subscription_delete_query_required = 1 ;

E2SubscriptionDeleteQueryAccept e2_subscription_delete_query_accept = 2 ;

E2SubscriptionDeleteQueryDecline e2_subscription_delete_query_reject = 3 ;

}

}

message E2SubscriptionDeleteNotificationProc {

oneof type_of_message {

E2SubscriptionDeleteNotificationDeleted e2_subscription_delete_notification_deleted = 1 ;

}

}

// ---------------- messages of E2_Subscription_API ----------------

// -------- messages of E2 Subscription procedure --------

message E2SubscriptionRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional E2SubscriptionDetails e2_subscription_details = 4;

optional uint32 e2_subscription_timer = 5;

optional near_rt_ric_apis_common_ies.v2.EndpointInformation e2_indication_destination = 6;

optional near_rt_ric_apis_common_ies.v2.EndpointInformation e2_subscription_delete_notification_destination = 7;

message E2SubscriptionDetails {

optional e2_related_apis_common_ies.v1.E2EventTriggerDefinition e2_event_trigger_definition = 1;

repeated E2ActionToBeSetupItem sequence_of_actions = 2 ;

message E2ActionToBeSetupItem {

optional e2_related_apis_common_ies.v1.E2ActionId requested_e2_action_id = 1 ;

optional e2_related_apis_common_ies.v1.E2ActionType e2_action_type = 2 ;

optional e2_related_apis_common_ies.v1.E2ActionDefinition e2_action_definition = 3 ;

optional e2_related_apis_common_ies.v1.E2SubsequentAction e2_subsequent_action = 4 ;

}

}

}

message E2SubscriptionSuccess {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

repeated E2ActionAdmittedItem e2_actions_admitted_list = 5;

repeated E2ActionNotAdmittedItem e2_actions_not_admitted_list = 6;

message E2ActionAdmittedItem {

optional e2_related_apis_common_ies.v1.E2ActionId requested_e2_action_id = 1 ;

optional e2_related_apis_common_ies.v1.E2ActionId e2_action_id = 2 ;

}

message E2ActionNotAdmittedItem {

optional e2_related_apis_common_ies.v1.E2ActionId requested_e2_action_id = 1 ;

optional e2_related_apis_common_ies.v1.Cause cause = 2 ;

}

}

message E2SubscriptionFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.Cause cause = 4;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 5;

}

message E2SubscriptionReject {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.Cause cause = 4;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 5;

}

// -------- messages of E2 Subscription Delete procedure --------

message E2SubscriptionDeleteRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

optional uint32 e2_subscription_delete_timer = 5;

}

message E2SubscriptionDeleteSuccess {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

}

message E2SubscriptionDeleteFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

optional e2_related_apis_common_ies.v1.Cause cause = 5;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 6;

}

message E2SubscriptionDeleteReject {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

optional e2_related_apis_common_ies.v1.Cause cause = 5;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 6;

}

// -------- messages of E2 Subscription Delete Query procedure --------

message E2SubscriptionDeleteQueryRequired {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

optional e2_related_apis_common_ies.v1.Cause cause = 5;

}

message E2SubscriptionDeleteQueryAccept {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

}

message E2SubscriptionDeleteQueryDecline {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

optional e2_related_apis_common_ies.v1.Cause cause = 5;

}

// -------- messages of E2 Subscription Delete Notification procedure --------

message E2SubscriptionDeleteNotificationDeleted {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 3;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 4;

optional e2_related_apis_common_ies.v1.Cause cause = 5;

}

6.3.4.1.2.2	E2_Indication_API

syntax = "proto3";

package ricapi.e2_indication_api.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "e2_related_apis_common_ies_ver_1_1_0.proto";

// top-level PDU

message E2IndicationApiPayload {

oneof procedure_type {

E2IndicationProc e2_indication_proc = 1;

}

}

// ---------------- procedures of E2_Subscription_API ----------------

message E2IndicationProc {

oneof type_of_message {

E2IndicationPush e2_indication_push = 1 ;

E2IndicationFailure e2_indication_failure = 2 ;

}

}

// ---------------- messages of E2_Indication_API ----------------

// -------- messages of E2 Indication procedure --------

message E2IndicationPush {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 1;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.E2ActionId e2_action_id = 4;

optional e2_related_apis_common_ies.v1.E2IndicationSN e2_indication_sn = 5;

optional e2_related_apis_common_ies.v1.E2IndicationType e2_indication_type = 6;

optional e2_related_apis_common_ies.v1.E2IndicationHeader e2_indication_header = 7;

optional e2_related_apis_common_ies.v1.E2IndicationMessage e2_indication_message = 8;

optional e2_related_apis_common_ies.v1.E2CallProcessId e2_call_process_id = 9;

}

message E2IndicationFailure {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 1;

optional e2_related_apis_common_ies.v1.E2RequestId e2_request_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.E2ActionId e2_action_id = 4;

optional e2_related_apis_common_ies.v1.E2IndicationSN e2_indication_sn = 5;

optional e2_related_apis_common_ies.v1.Cause cause = 6;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 7;

}

6.3.4.1.2.3	E2_Control_API

syntax = "proto3";

package ricapi.e2_control_api.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "e2_related_apis_common_ies_ver_1_1_0.proto";

// top-level PDU

message E2ControlApiPayload {

oneof procedure_type {

E2ControlProc e2_control_proc = 1;

}

}

// ---------------- procedures of E2_Control_API ----------------

message E2ControlProc {

oneof type_of_message {

E2ControlRequest e2_control_request = 1;

E2ControlSuccess e2_control_success = 2;

E2ControlReject e2_control_reject = 3;

E2ControlFailure e2_control_failure = 4;

}

}

// ---------------- messages of E2_Control_API ----------------

// -------- messages of E2 Control procedure --------

message E2ControlRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.E2CallProcessId e2_call_process_id = 4;

optional e2_related_apis_common_ies.v1.E2ControlHeader e2_control_header = 5;

optional e2_related_apis_common_ies.v1.E2ControlMessage e2_control_message = 6;

optional e2_related_apis_common_ies.v1.E2ControlAckRequest e2_control_ack_request = 7;

optional uint32 e2_control_timer = 8;

}

message E2ControlSuccess {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.E2CallProcessId e2_call_process_id = 4;

optional e2_related_apis_common_ies.v1.E2ControlOutcome e2_control_outcome = 5;

}

message E2ControlReject {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.Cause cause = 4;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 5;

}

message E2ControlFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId global_e2_node_id = 2;

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 3;

optional e2_related_apis_common_ies.v1.E2CallProcessId e2_call_process_id = 4;

optional e2_related_apis_common_ies.v1.E2ControlOutcome e2_control_outcome = 5;

optional e2_related_apis_common_ies.v1.Cause cause = 6;

optional e2_related_apis_common_ies.v1.CriticalityDiagnostics criticality_diagnostics = 7;

}

6.3.4.1.3	Information Element Definitions

syntax = "proto3";

package ricapi.e2_related_apis_common_ies.v1;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

// ----------------------------- Information Elements -----------------------------

// --------------E--------------

// From E2AP Clause 9.2.11

enum E2ActionType {

E2ACTION_TYPE_REPORT = 0;

E2ACTION_TYPE_INSERT = 1;

E2ACTION_TYPE_POLICY = 2;

};

// From E2AP Clause 9.2.21

enum E2ControlAckRequest {

E2_CONTROL_ACK_REQUEST_NO_ACK = 0;

E2_CONTROL_ACK_REQUEST_ACK = 1;

}

// From E2AP Clause 9.2.15

enum E2IndicationType {

E2_INDICATION_TYPE_REPORT = 0;

E2_INDICATION_TYPE_INSERT = 1;

}

// --------------C--------------

message Cause {

oneof cause_group {

E2RelatedApiCause e2_related_api_cause = 1;

E2apCause e2ap_cause = 2;

}

message E2apCause {

oneof cause_group {

RicRequest ric_request = 1;

RicService ric_service = 2;

E2Node e2_node = 3;

Transport transport = 4;

Protocol protocol = 5;

Misc misc = 6;

}

enum E2Node {

E2NODE_COMPONENT_UNKNOWN = 0;

}

enum Misc {

CONTROL_PROCESSING_OVERLOAD = 0;

HARDWARE_FAILURE = 1;

OM_INTERVENTION = 2;

MISC_UNSPECIFIED = 3;

}

enum Protocol {

TRANSFER_SYNTAX_ERROR = 0;

ABSTRACT_SYNTAX_ERROR_REJECT = 1;

ABSTRACT_SYNTAX_ERROR_IGNORE_AND_NOTIFY = 2;

MESSAGE_NOT_COMPATIBLE_WITH_RECEIVER_STATE = 3;

SEMANTIC_ERROR = 4;

ABSTRACT_SYNTAX_ERROR_FALSELY_CONSTRUCTED_MESSAGE = 5;

PROTOCOL_UNSPECIFIED = 6;

}

enum RicRequest {

RAN_FUNCTION_ID_INVALID = 0;

ACTION_NOT_SUPPORTED = 1;

EXCESSIVE_ACTIONS = 2;

DUPLICATE_ACTION = 3;

DUPLICATE_EVENT_TRIGGER = 4;

FUNCTION_RESOURCE_LIMIT = 5;

REQUEST_ID_UNKNOWN = 6;

INCONSISTENT_ACTION_SUBSEQUENT_ACTION_SEQUENCE = 7;

CONTROL_MESSAGE_INVALID = 8;

RIC_CALL_PROCESS_ID_INVALID = 9;

CONTROL_TIMER_EXPIRED = 10;

CONTROL_FAILED_TO_EXECUTE = 11;

SYSTEM_NOT_READY = 12;

RIC_REQUEST_UNSPECIFIED = 13;

}

enum RicService {

RAN_FUNCTION_NOT_SUPPORTED = 0;

EXCESSIVE_FUNCTIONS = 1;

RIC_RESOURCE_LIMIT = 2;

}

enum Transport {

TRANSPORT_UNSPECIFIED = 0;

TRANSPORT_RESOURCE_UNAVAILABLE = 1;

}

}

message E2RelatedApiCause {

oneof cause_group {

ApplicationLayer  application_layer = 1;

Transport transport = 2;

Protocol protocol = 3;

Misc misc = 4;

}

enum ApplicationLayer {

APPLICATION_LAYER_UNSPECIFIED = 0;

GLOBAL_E2_NODE_ID_INVALID = 1;

E2_NODE_RESET_OR_DISCONNECTED = 2;

RAN_FUNCTION_ID_INVALID = 3;

ACTION_NOT_SUPPORTED = 4;

EXCESSIVE_ACTIONS = 5;

DUPLICATE_ACTION = 6;

E2_REQUEST_ID_UNKNOWN = 7;

CONTROL_CONFLICT = 8;

E2_INDICATION_UNKNOWN = 9;

E2AP_PROCEDURE_TIMEOUT = 10;

ESSENTIAL_FOR_OPERATION = 11;

}

enum Misc {

MISC_UNSPECIFIED = 0;

CONTROL_PROCESSING_OVERLOAD = 1;

HARDWARE_FAILURE = 2;

OM_INTERVENTION = 3;

}

enum Protocol {

PROTOCOL_UNSPECIFIED = 0;

TRANSFER_SYNTAX_ERROR = 1;

ABSTRACT_SYNTAX_ERROR_REJECT = 2;

ABSTRACT_SYNTAX_ERROR_IGNORE_AND_NOTIFY = 3;

MESSAGE_NOT_COMPATIBLE_WITH_RECEIVER_STATE = 4;

SEMANTIC_ERROR = 5;

ABSTRACT_SYNTAX_ERROR_FALSELY_CONSTRUCTED_MESSAGE = 6;

}

enum Transport {

TRANSPORT_UNSPECIFIED = 0;

TRANSPORT_RESOURCE_UNAVAILABLE = 1;

}

}

}

message CriticalityDiagnostics {

repeated IeCriticalityDiagnosticsItem ie_criticality_diagnostics_list = 1;

message IeCriticalityDiagnosticsItem {

optional IeCriticality ie_criticality = 1;

optional IeId ie_id = 2;

optional TypeOfError type_of_error = 3;

}

enum IeCriticality {

IE_CRITICALITY_REJECT = 0;

IE_CRITICALITY_IGNORE = 1;

IE_CRITICALITY_NOTIFY = 2;

}

enum IeId {

IE_ID_CAUSE = 0;

IE_ID_CRITICALITY_DIAGNOSTICS = 1;

IE_ID_E2_ACTION_ID = 2;

IE_ID_E2_ACTIONS_ADMITTED_LIST = 3;

IE_ID_E2_ACTIONS_NOT_ADMITTED_LIST = 4;

IE_ID_E2_CALL_PROCESS_ID = 5;

IE_ID_E2_CONTROL_ACK_REQUEST = 6;

IE_ID_E2_CONTROL_HEADER = 7;

IE_ID_E2_CONTROL_MESSAGE = 8;

IE_ID_E2_CONTROL_OUTCOME = 9;

IE_ID_E2_CONTROL_TIMER = 10;

IE_ID_E2_INDICATION_HEADER = 11;

IE_ID_E2_INDICATION_MESSAGE = 12;

IE_ID_E2_INDICATION_SN = 13;

IE_ID_E2_INDICATION_TYPE = 14;

IE_ID_E2_REQUEST_ID = 15;

IE_ID_E2_SUBSCRIPTION_DETAILS = 16;

IE_ID_GLOBAL_E2_NODE_ID = 17;

IE_ID_PROCEDURE_TRANSACTION_ID = 18;

IE_ID_RAN_FUNCTION_ID = 19;

IE_ID_SEQUENCE_OF_ACTIONS = 20;

IE_ID_E2_SUBSCRIPTION_TIMER = 21;

IE_ID_E2_SUBSCRIPTION_DELETE_TIMER = 22;

}

enum TypeOfError {

TYPE_OF_ERROR_NOT_UNDERSTOOD = 0;

TYPE_OF_ERROR_MISSING = 1;

}

}

// --------------E--------------

message E2ActionDefinition {

optional bytes value = 1;

};

message E2ActionId {

optional uint32 value = 1;

};

message E2CallProcessId {

optional bytes value = 1;

};

message E2ControlHeader {

optional bytes value = 1;

};

message E2ControlMessage {

optional bytes value = 1;

};

message E2ControlOutcome {

optional bytes value = 1;

};

message E2EventTriggerDefinition {

optional bytes value = 1;

};

message E2IndicationHeader {

optional bytes value = 1;

};

message E2IndicationMessage {

optional bytes value = 1;

};

message E2IndicationSN {

optional uint32 value = 1;

};

message E2RequestId {

optional uint32 value = 1;

};

// From E2AP Clause 9.2.13

message E2SubsequentAction {

optional SubsequentActionType subsequent_action_type = 1;

optional TimeToWait e2_time_to_wait = 2;

enum SubsequentActionType {

SUBSEQUENT_ACTION_TYPE_CONTINUE = 0;

SUBSEQUENT_ACTION_TYPE_WAIT = 1;

};

enum TimeToWait {

TIME_TO_WAIT_W1MS = 0;

TIME_TO_WAIT_W2MS = 1;

TIME_TO_WAIT_W5MS = 2;

TIME_TO_WAIT_W10MS = 3;

TIME_TO_WAIT_W20MS = 4;

TIME_TO_WAIT_W30MS = 5;

TIME_TO_WAIT_W40MS = 6;

TIME_TO_WAIT_W50MS = 7;

TIME_TO_WAIT_W100MS = 8;

TIME_TO_WAIT_W200MS = 9;

TIME_TO_WAIT_W500MS = 10;

TIME_TO_WAIT_W1S = 11;

TIME_TO_WAIT_W2S = 12;

TIME_TO_WAIT_W5S = 13;

TIME_TO_WAIT_W10S = 14;

TIME_TO_WAIT_W20S = 15;

TIME_TO_WAIT_W60S = 16;

}

};

#### 6.3.4.2	Solution 2: gRPC with Protocol Buffers

##### 6.3.4.2.1	Usage of gRPC with Protocol Buffers

This solution uses Protocol Buffers as message encoding, and also as the Interface Definition Language (IDL) for gRPC [5].

Table 6.3.4.2.1-1 lists the mapping of enablement APIs and corresponding gRPC services.

Table 6.3.4.2.1-1: Mapping of E2 related APIs and gRPC services

| API Name | gRPC service(s) |
| --- | --- |
| E2_Subscription_API | E2_Subscription_API, E2_Subscription_API_Callback |
| E2_Indication_API | E2_Indication_API |
| E2_Control_API | E2_Control_API |

A procedure of an E2 related API is mapped to a gRPC method.

Table 6.3.4.2.1-1 lists the E2 related APIs abstract syntax included in this document.

Table 6.3.4.2.1-1: List of E2 related APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| E2_Subscription_API | 6.3.4.2.2.1 | 1.0.0 | e2_subscription_api_grpc_ver_1_0_0.proto |
| E2_Indication_API | 6.3.4.2.2.2 | 1.0.0 | e2_indication_api_ grpc_ver_1_0_0.proto |
| E2_Control_API | 6.3.4.2.2.3 | 1.0.0 | e2_control_api_ver_ grpc_1_0_0.proto |
| E2 related API common information elements | 6.3.4.1.3 | 1.1.0 | e2_related_apis_common_ies_ver_1_1_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

##### 6.3.4.2.2	API Definitions

###### 6.3.4.2.2.1	E2_Subscription_API

syntax = "proto3";

package ricapi.e2_subscription_api_grpc.v1;

import "e2_subscription_api_ver_2_0_0.proto";

// gRPC service and rpc definition

service E2_Subscription_API {

rpc E2SubscriptionProc(E2SubscriptionInitMsg) returns (E2SubscriptionOutMsg) {};

rpc E2SubscriptionDeleteProc(E2SubscriptionDeleteInitMsg) returns (E2SubscriptionDeleteOutMsg) {};

}

message E2SubscriptionInitMsg {

optional ricapi.e2_subscription_api.v2.E2SubscriptionRequest e2_subscription_request = 1;

}

message E2SubscriptionOutMsg {

oneof type_of_message {

ricapi.e2_subscription_api.v2.E2SubscriptionSuccess e2_subscription_success = 1;

ricapi.e2_subscription_api.v2.E2SubscriptionReject e2_subscription_reject = 2;

ricapi.e2_subscription_api.v2.E2SubscriptionFailure e2_subscription_failure = 3;

}

}

message E2SubscriptionDeleteInitMsg {

optional ricapi.e2_subscription_api.v2.E2SubscriptionDeleteRequest e2_subscription_delete_request = 1;

}

message E2SubscriptionDeleteOutMsg {

oneof type_of_message {

ricapi.e2_subscription_api.v2.E2SubscriptionDeleteSuccess e2_subscription_delete_success = 1;

ricapi.e2_subscription_api.v2.E2SubscriptionDeleteReject e2_subscription_delete_reject = 2;

ricapi.e2_subscription_api.v2.E2SubscriptionDeleteFailure e2_subscription_delete_failure = 3;

}

}

service E2_Subscription_API_Callback {

rpc E2SubscriptionDeleteQueryProc(E2SubscriptionDeleteQueryInitMsg) returns (E2SubscriptionDeleteQueryOutMsg) {};

rpc E2SubscriptionDeleteNotificationProc(E2SubscriptionDeleteNotificationInitMsg) returns (E2SubscriptionDeleteNotificationOutMsg) {};

}

message E2SubscriptionDeleteQueryInitMsg {

optional ricapi.e2_subscription_api.v2.E2SubscriptionDeleteQueryRequired e2_subscription_delete_query_required = 1;

}

message E2SubscriptionDeleteQueryOutMsg {

oneof type_of_message {

ricapi.e2_subscription_api.v2.E2SubscriptionDeleteQueryAccept e2_subscription_delete_query_accept = 1;

ricapi.e2_subscription_api.v2.E2SubscriptionDeleteQueryDecline e2_subscription_delete_query_reject = 2;

}

}

message E2SubscriptionDeleteNotificationInitMsg {

optional ricapi.e2_subscription_api.v2.E2SubscriptionDeleteNotificationDeleted e2_subscription_delete_notification_deleted = 1;

}

message E2SubscriptionDeleteNotificationOutMsg {

}

###### 6.3.4.2.2.2	E2_Indication_API

syntax = "proto3";

package ricapi.e2_indication_api_grpc.v1;

import "e2_indication_api_ver_2_0_0.proto";

// gRPC service and rpc definition

service E2_Indication_API {

rpc E2IndicationProc(E2IndicationInitMsg) returns (E2IndicationOutMsg) {};

}

message E2IndicationInitMsg {

optional ricapi.e2_indication_api.v2.E2IndicationPush e2_indication_push = 1;

}

message E2IndicationOutMsg {

oneof type_of_message {

EmptyMessage empty_message = 1;

ricapi.e2_indication_api.v2.E2IndicationFailure e2_indication_failure = 2;

}

message EmptyMessage {

}

}

###### 6.3.4.2.2.3	E2_Control_API

syntax = "proto3";

package ricapi.e2_control_api.v2;

import "e2_control_api_ver_2_0_0.proto";

// gRPC service and rpc definition

service E2_Control_API {

rpc E2ControlProc(E2ControlInitMsg) returns (E2ControlOutMsg) {};

}

message E2ControlInitMsg {

optional ricapi.e2_control_api.v2.E2ControlRequest e2_control_request = 1;

}

message E2ControlOutMsg {

oneof type_of_message {

ricapi.e2_control_api.v2.E2ControlSuccess e2_control_success = 1;

ricapi.e2_control_api.v2.E2ControlReject e2_control_reject = 2;

ricapi.e2_control_api.v2.E2ControlFailure e2_control_failure = 3;

}

}

6.3.5	Message Transfer Syntax

6.3.5.1	Solution 1: SCTP with Protocol Buffers

The message transfer syntax for Solution 1 is serialized Protocol Buffers [4].

#### 6.3.5.2	Solution 2: gRPC with Protocol Buffers

The message transfer syntax for Solution 2 is serialized Protocol Buffers [4].

7	SDL API

7.1	Overview

7.1.1	Protocol Stack

In this version of specification, the default and optional protocol stacks for the SDL APIs are shown on Figure 7.1.1-1.

Note: The security protocols are specified in [6].

| Default protocol stack : SCTP | Optional protocol stack : gRPC |
| --- | --- |

Figure 7.1.1-1: Protocol stack for SDL APIs

The SDL APIs may use the SCTP or gRPC with Protocol Buffers [4] as the application layer serialization protocol. The supported protocol by the platform API is presented to the xApps during the xApp Discovery Procedure in clause 9.2.1.

7.1.2	List of SDL APIs, procedures and messages

The SDL API procedures introduced in clause 9.5 of [2] are categorized under the SDL APIs. These APIs include:

SDL Information API: enabling subscription and retrieval of information by xApps.

xApp Data Sharing API: enabling xApp to share specific data with Near-RT RIC platform and/or other data consumer xApps.

These APIs are summarized in Table 7.1.2-1.

Table 7.1.2-1: List of SDL APIs

| API Name | Description |
| --- | --- |
| SDL_Information_API | Near-RT RIC platform service for SDL information operation |
| Xapp_Data_Sharing_API | xApp service to share SDL data produced by the xApp |

The table 7.1.2-2 lists the API procedures for each SDL API.

Table 7.1.2-2: List of SDL API procedures

| API Name | API procedure | Communication type |
| --- | --- | --- |
| SDL_Information_API | SDL Information Subscribe | Request/Response |
|  | SDL Information Push | Notify |
|  | SDL Information Update Notify | Notify |
|  | SDL Information Fetch | Request/Response |
| Xapp_Data_Sharing_API | Xapp Shared Data Fetch | Request/Response |
|  | Xapp Shared Data Subscrbe | Request/Response |
|  | Xapp Shared Data Subscription Delete | Request/Response |
|  | Xapp Shared Data Push | Notify |

The table 7.1.2-3 lists the messages for each SDL API procedure.

Table 7.1.2-3: List of SDL API messages

| API Procedure | Initiated by | Initiating Message | Outcome Message |  |
| --- | --- | --- | --- | --- |
|  |  |  | Successful Outcome | Unsuccessful Outcome |
| SDL Information Subscribe | xApp | Subscribe Information REQUEST | Subscribe Information RESPONSE | Subscribe Information FAILURE |
| SDL Information Push | Near-RT RIC platform | Information PUSH | - | - |
| SDL Information Update Notify | Near-RT RIC platform | INFORMATION UPDATE NOTIFY | - | - |
| SDL Information Fetch | xApp | INFORMATION FETCH REQUEST | INFORMATION FETCH RESPONSE | INFORMATION FETCH FAILURE |
| Xapp Shared Data Fetch | Near-RT RIC platform or data consumer xApp | xAPP SHARED DATA FETCH REQUEST | xAPP SHARED DATA FETCH RESPONSE | xAPP SHARED DATA FETCH FAILURE |
| Xapp Shared Data Subscribe | Near-RT RIC platform or data consumer xApp | XAPP SHARED DATA SUBSCRIPTION REQUEST | XAPP SHARED DATA SUBSCRIPTION RESPONSE | XAPP SHARED DATA SUBSCRIPTION FAILURE |
| Xapp Shared Data Subscription Delete | Near-RT RIC platform or data consumer xApp | XAPP SHARED DATA SUBSCRIPTION DELETE REQUEST | XAPP SHARED DATA SUBSCRIPTION DELETE RESPONSE | XAPP SHARED DATA SUBSCRIPTION DELETE FAILURE |
| Xapp Shared Data Push | xApp | xAPP SHARED DATA PUSH | - | - |

The table 7.1.2-4 lists the supported data categories for each SDL API in this release.

Table 7.1.2-4: Supported Data Categories

| API Name | Supported Data Categories |
| --- | --- |
| SDL_Information_API | E2 Node Infomation |
| Xapp_Data_Sharing_API | RAN Analytics Information |

7.2	SDL API Procedures

7.2.1	SDL Client Registration procedure

7.2.2	SDL Client Deregistration procedure

7.2.3	SDL Subscribe Information procedure

7.2.3.1	General

This procedure is used by the xApp to subscribe to the SDL for the information that it is interested to obtain.

7.2.3.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Near-RT RIC Platform" as near

participant "xApp" as app

app -> near: SUBSCRIBE INFORMATION REQUEST

near -> app: SUBSCRIBE INFORMATION RESPONSE

@enduml

Figure 7.2.3.2-1: Subscribe Information procedure, successful operation

The xApp initiates the procedure by sending the SUBSCRIBE INFORMATION REQUEST message.

The message shall include:

-	the SDL Data Type ID IE, which indicates the requested SDL data type;

-	the Delivery Method IE, which indicates the method of data delivery;

-	the Notification Destination IE, which specifies the endpoint to receive associated notification(s);

-	the Notification Criteria IE, which specifies the timing related to the notification(s). It shall include:

-	the Notification Method IE, which indicates the notification(s) are periodic or event triggered;

-	the Notification Period IE if the Notification Method IE is set to "PERIODIC", which indicates the time inverval between periodic notifications;

-	the Trigger Event Definition IE if the Notification Method IE is set to "EVENT_TRIGGERED", which contains the description of the triggering event;

The message may include:

-	the SDL Data Description IE, which contains the detailed descriptions for the requested data.

When the Near-RT RIC platform receives the request, it searches the local database or data sources for the requested information.

If the Near-RT RIC platform successfully processes the request, it shall prepare necessary resource for the subscription, assign a subscription ID, and responds to the xApp with the SUBSCRIBE INFORMATION RESPONSE message.

7.2.3.3	Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Near-RT RIC Platform" as near

participant "xApp" as app

app -> near: SUBSCRIBE INFORMATION REQUEST

near -> app: SUBSCRIBE INFORMATION FAILURE

@enduml

Figure 7.2.3.3-1: Subscribe Information procedure, unsuccessful operation

If the Near-RT RIC platform can not accept the request it shall respond with the SUBSCRIBE INFORMATION FAILURE message and the proper cause. If the message contains Time To Wait IE the xApp shall wait at least the indicated time before reinitiating the request to the Near-RT RIC platform.

7.2.3.4	Abnormal Conditions

If the Near-RT RIC platform receives a SUBSCRIBE INFORMATION REQUEST message containing Information Type IE that is not supported by the Near-RT RIC platform, or the optional filter definition is invalid, the Near-RT RIC platform shall send the SUBSCRIBE INFORMATION FAILURE message to the xApp with an appropriate cause value.

7.2.4	SDL Information Push procedure

7.2.4.1	General

This procedure is used by the Near-RT RIC platform to send the latest information from the Near-RT RIC platform to the xApp based on what the xApp has subscribed when the Delivery Method IE set to "PUSH_DELTA" or "PUSH_FULL" in the subscribe information request.

7.2.4.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Near-RT RIC Platform" as near

participant "xApp" as app

near -> app: INFORMATION PUSH

@enduml

Figure 7.2.4.2-1: Information Push procedure, successful operation

The Near-RT RIC initiates the procedure by sending the INFORMATION PUSH message.

The message shall include:

-	the Subscription ID IE, which indicates the associated subscription;

-	the SDL Data Type ID IE, which indicates the SDL data Type carried in the message;

-	the SDL Data Block IE, which carries the SDL data and shall include:

-	the SDL Data Unit IE, which contains a single item of the SDL data;

-	the Modification Type IE, which indicates type of change (added, modified, removed, etc.) associated with the SDL data in the SDL Data Unit IE.

7.2.4.3	Unsuccessful Operation

Not applicable.

7.2.4.4	Abnormal Conditions

Not applicable.

7.2.5	SDL Information Update Notify procedure

7.2.5.1	General

This procedure is used by the Near-RT RIC platform to notify the xApp that certain information subscribed by the xApp has been updated and available to be fetched. The procedure is used when the Delivery Method IE set to "UPDATE_NOTIFY" in the subscribe information request.

7.2.5.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Near-RT RIC Platform" as near

participant "xApp" as app

near -> app: INFORMATION UPDATE NOTIFY

@enduml

Figure 7.2.5.2-1: Information Update Notify procedure, successful operation

The Near-RT RIC platform initiates the procedure by sending the INFORMATION UPDATE NOTIFY message.

The message shall include:

-	the Subscription ID IE, which indicates the associated subscription;

-	the SDL Data Type ID IE, which indicates the SDL data Type carried in the message;

-	the SDL Data Block IE, which carries the SDL data and shall include:

-	the SDL Update Notify Unit IE, which contains a single item of the SDL data;

-	the Modification Type IE, which indicates type of change (added, modified, removed, etc.) associated with the SDL data in the SDL Update Notify Unit IE.

Once the xApp has received the message, xApp can determine to retrieve the updated information through the Information Fetch procedure described in clause 7.2.6.

7.2.5.3	Unsuccessful Operation

Not applicable.

7.2.5.4	Abnormal Conditions

Not applicable.

7.2.6	SDL Information Fetch procedure

7.2.6.1	General

This procedure is used by the xApp to fetch certain information from the SDL.

7.2.6.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Near-RT RIC Platform" as near

participant "xApp" as app

app -> near: INFORMATION FETCH REQUEST

near -> app: INFORMATION FETCH RESPONSE

@enduml

Figure 7.2.6.2-1: SDL Information Fetch procedure, successful operation

The xApp initiates the procedure by sending the INFORMATION FETCH REQUEST message.

The message shall include:

-	the SDL Data Type ID IE, which indicates the requested SDL data type.

The message may include:

-	the SDL Data Description IE, which contains the detailed descriptions for the requested data.

When the Near-RT RIC platform receives the request, it searches the local database or data sources for the fetched information.

If the SDL contains matching data for the fetch request, the Near-RT RIC platform sends the requested data to the xApp in the INFORMATION FETCH RESPONSE message.

7.2.6.3	Unsuccessful Operation

If the data corresponding to a fetch request is not found, the Near-RT RIC platform sends an INFORMATION FETCH FAILURE message with an appropriate failure cause.

7.2.6.4	Abnormal Conditions

If the Near-RT RIC platform receives a INFORMATION FETCH REQUEST message containing SDL Data Type ID IE that is not supported by the Near-RT RIC platform, or the optional SDL Data Description IE definition is invalid, the Near-RT RIC platform shall send the INFORMATION FETCH FAILURE message to the xApp with an appropriate cause value.

7.2.7	Store Data procedure

7.2.8	Modify Data procedure

### 7.2.9	Xapp Shared Data Fetch procedure

#### 7.2.9.1		General

This procedure is used by the Near-RT RIC platform or the data consumer xApp to fetch specific types of data shared by the data producer xApp.

#### 7.2.9.2		Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

consumer -> producerxApp: XAPP SHARED DATA FETCH REQUEST

producerxApp -> consumer: XAPP SHARED DATA FETCH RESPONSE

@enduml

Figure 7.2.9.2-1: xApp Shared Data Fetch procedure, successful operation

The data consumer initiates the procedure by sending the XAPP SHARED DATA FETCH REQUEST message.

The message shall include:

-	the SDL Data Type ID IE, which indicates the requested SDL data type.

The message may include:

-	the SDL Data Description IE, which contains the detailed descriptions for the requested data.

Upon receiving the message, the data producer xApp shall check the authorization for the data consumer to access the requested data. If the requested data is available at the producer xApp, the producer xApp shall send the requested data to the data consumer in the XAPP SHARED DATA FETCH RESPONSE message.

#### 7.2.9.3		Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

consumer -> producerxApp: XAPP SHARED DATA FETCH REQUEST

producerxApp -> consumer: XAPP SHARED DATA FETCH FAILURE

@enduml

Figure 7.2.9.3-1: xApp Shared Data Fetch procedure, unsuccessful operation

If the data consumer is not authorized, or the requested data specified in the XAPP SHARED DATA FETCH REQUEST is unavailable, the producer xApp shall send an XAPP SHARED DATA FETCH FAILURE message with appropriate cause.

#### 7.2.9.4		Abnormal Conditions

Not applicable.

### 7.2.10	Xapp Shared Data Subscribe procedure

#### 7.2.10.1		General

This procedure is used by the Near-RT RIC platform or the data consumer xApp to subscribe to specific types of data shared by the data producer xApp.

#### 7.2.10.2		Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

consumer -> producerxApp: XAPP SHARED DATA SUBSCRIPTION REQUEST

producerxApp -> consumer: XAPP SHARED DATA SUBSCRIPTION RESPONSE

@enduml

Figure 7.2.10.2-1: xApp Shared Data Subscribe procedure, successful operation

The data consumer initiates the procedure by sending the XAPP SHARED DATA SUBSCRIPTION REQUEST message.

The message shall include:

-	the SDL Data Type ID IE, which indicates the requested SDL data type;

-	the Delivery Method IE, which indicates the method of data delivery;

-	the Notification Destination IE, which specifies the endpoint to receive associated notification(s);

-	the Notification Criteria IE, which specifies the timing related to the notification(s). It shall include:

-	the Notification Method IE, which indicates the notification(s) are periodic or event triggered;

-	the Notification Period IE if the Notification Method IE is set to "PERIODIC", which indicates the time inverval between periodic notifications;

-	the Trigger Event Definition IE if the Notification Method IE is set to "EVENT_TRIGGERED", which contains the description of the triggering event;

The message may include:

-	the SDL Data Description IE, which contains the detailed descriptions for the requested data.

Upon receiving the message, the data producer xApp shall check the authorization for the data consumer to subscribe to the requested data. If the requested subscription can be fulfilled, the producer xApp shall prepare necessary resource for the subscription,  assign a subscription ID, and send the XAPP SHARED DATA SUBSCRIPTION RESPONSE message to the data consumer.

The message shall include:

-	the Subscription ID IE, which contains the subscription ID.

#### 7.2.10.3		Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

consumer -> producerxApp: XAPP SHARED DATA SUBSCRIPTION REQUEST

producerxApp -> consumer: XAPP SHARED DATA SUBSCRIPTION FAILURE

@enduml

Figure 7.2.10.3-1: xApp Shared Data Subscribe procedure, unsuccessful operation

If the data consumer is not authorized, or the requested subscription cannot be fulfilled, the producer xApp shall send the XAPP SHARED DATA FETCH FAILURE message with appropriate cause.

#### 7.2.10.4		Abnormal Conditions

Not Applicable.

### 7.2.11	Xapp Shared Data Subscription Delete procedure

#### 7.2.11.1		General

This procedure is used by the Near-RT RIC platform or the data consumer xApp to delete an existing xApp shared data subscription toward the data producer xApp.

#### 7.2.11.2		Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

consumer -> producerxApp: XAPP SHARED DATA SUBSCRIPTION DELETE REQUEST

producerxApp -> consumer: XAPP SHARED DATA SUBSCRIPTION DELETE RESPONSE

@enduml

Figure 7.2.11.2-1: xApp Shared Data Subscription Delete procedure, successful operation

The data consumer initiates the procedure by sending the XAPP SHARED DATA SUBSCRIPTION DELETE message.

The message shall include:

-	the Subscription ID IE, which indicates the subscription to delete;

Upon receiving the message, the data producer xApp shall check the authorization for the data consumer. If the subscription is present, the data producer xApp shall release the resource for the subscription, and send the XAPP SHARED DATA SUBSCRIPTION DELETE RESPONSE message to the data consumer.

#### 7.2.11.3		Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

consumer -> producerxApp: XAPP SHARED DATA SUBSCRIPTION DELETE REQUEST

producerxApp -> consumer: XAPP SHARED DATA SUBSCRIPTION DELETE FAILURE

@enduml

Figure 7.2.11.3-1: xApp Shared Data Subscription Delete procedure, unsuccessful operation

If the data consumer is not authorized, or the subscription indicated by the Subscription ID IE is not present, the data producer xApp shall send the XAPP SHARED DATA SUBSCRIPTION DELETE FAILURE message with appropriate cause.

#### 7.2.11.4		Abnormal Conditions

Not applicable.

### 7.2.12	Xapp Shared Data Push procedure

#### 7.2.12.1	General

This procedure is used by the data producer xApp to push the subscribed SDL data to the data consumer according to an existing subscription. The procedure is used when the Delivery Method IE set to "PUSH_DELTA" or "PUSH_FULL" in the xApp shared data subscription request.

#### 7.2.12.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "Data consumer \n(Near-RT RIC platform or xApp)" as consumer

participant "Data producer xApp" as producerxApp

producerxApp -> consumer: XAPP SHARED DATA PUSH

@enduml

Figure 7.2.12.2-1: xApp Shared Data Push procedure, successful operation

The data producer xApp initiates the procedure by sending the XAPP SHARED DATA PUSH message.

The message shall include:

-	the Subscription ID IE, which indicates the associated subscription;

-	the SDL Data Type ID IE, which indicates the SDL data Type carried in the message;

-	the SDL Data Block IE, which carries the SDL data and shall include:

-	the SDL Data Unit IE, which contains a single item of the SDL data;

-	the Modification Type IE, which indicates type of change (added, modified, removed, etc.) associated with the SDL data in the SDL Data Unit IE.

#### 7.2.12.3	Unsuccessful Operation

Not applicable.

#### 7.2.12.4	Abnormal Conditions

Not applicable.

7.3	Elements for SDL API Communication

7.3.1	General

Sub clauses 7.3.2 and 7.3.3 describe the structure of the messages and information elements required for the SDL API in tabular format.

7.3.2	Message Functional Definition and Content

7.3.2.1	SDL CLIENT REGISTRATION REQUEST

7.3.2.2	SDL CLIENT REGISTRATION RESPONSE

7.3.2.3	SDL CLIENT REGISTRATION FAILURE

7.3.2.4	SDL CLIENT DE-REGISTRATION REQUEST

7.3.2.5	SDL CLIENT DE-REGISTRATION RESPONSE

7.3.2.6	SUBSCRIBE INFORMATION REQUEST

This message is sent by an xApp to the Near-RT RIC platform to subscribe to the interested information.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| Delivery Method | M |  | 7.3.3.4 |  | YES | Reject |
| SDL Data Description | O |  | 7.3.3.22 | This IE may be absent, if a default SDL Data Description is available for the SDL Type ID and used in this request. | YES | Reject |
| Notification Destination | M |  | 10.1.9 |  | YES | Reject |
| Notification Criteria | M |  | 7.3.3.24 |  | YES | Reject |

7.3.2.7	SUBSCRIBE INFORMATION RESPONSE

This message is sent by the Near-RT RIC platform after successful acceptance of an xApp’s request to subscribe to the information in the SDL.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription ID | M |  | 7.3.3.26 |  | YES | Reject |

7.3.2.8	SUBSCRIBE INFORMATION FAILURE

This message is sent by the Near-RT RIC platform after failure to accept an xApp’s request to subscribe to information in the SDL.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription Reject Cause | M |  | 7.3.3.9 |  |  |  |
| Time to Wait | O |  | 7.3.3.13 | If absent, no time to wait is required. |  |  |

7.3.2.9	INFORMATION PUSH

This message is sent by the Near-RT RIC platform to an xApp containing the information subscribed by the xApp.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription ID | M |  | 7.3.3.26 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  |  |  |
| SDL Data Block |  | 0..1 |  |  | YES | Reject |
| > SDL Data Block Item |  | 1..< maxnoofSDLDataUnits > |  |  | - | - |
| >> Modification Type | C |  | 7.3.3.10 | Shall be present when the Delivery Method IE in the subscription request is set to "PUSH_DELTA". | - | - |
| >> SDL Data Unit | M |  | 7.3.3.23 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSDLDataUnits | Maximum number of SDL Data Units that can be carried in an SDL Data Block. Value is 1048576. |

7.3.2.10	INFORMATION UPDATE NOTIFY

This message is sent by the Near-RT RIC platform to inform an xApp that some of the information subscribed by the xApp has been updated.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription ID | M |  | 7.3.3.26 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  |  |  |
| SDL Update Notify Block |  | 0..1 |  |  | YES | Reject |
| > SDL Update Notify Block Item |  | 1..< maxnoofSDLUpdateNotifyUnits > |  |  | - | - |
| >> Modification Type | M |  | 7.3.3.10 |  | - | - |
| >> SDL Update Notify Unit | M |  | 7.3.3.27 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSDLUpdateNotifyUnits | Maximum number of SDL Update Notify Units that can be carried in an SDL Update Notify Block. Value is 1048576. |

7.3.2.11	INFORMATION FETCH REQUEST

This message is sent by an xApp to the Near-RT RIC platform to request certain information subscribed by the xApp.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| SDL Data Description | O |  | 7.3.3.22 | This IE may be absent when a default definition of it exists and is used in the request. | YES | Reject |

7.3.2.12	INFORMATION FETCH RESPONSE

This message is sent by the Near-RT RIC platform to an xApp containing the information requested by the xApp.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| SDL Data Block |  | 0..1 |  |  | YES | Reject |
| > SDL Data Block Item |  | 1..< maxnoofSDLDataUnits > |  |  | - | - |
| >> SDL Data Unit | M |  | 7.3.3.23 |  | - | - |

7.3.2.13	INFORMATION FETCH FAILURE

This message is sent by the Near-RT RIC platform to an xApp if the Information fetch request failed.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Fetch Failure Cause | M |  | 7.3.3.18 | Cause for failure | YES | Reject |

7.3.2.14	STORE DATA REQUEST

7.3.2.15	STORE DATA RESPONSE

7.3.2.16	STORE DATA FAILURE

7.3.2.17	MODIFY DATA REQUEST

7.3.2.18	MODIFY DATA RESPONSE

7.3.2.19	MODIFY DATA FAILURE

7.3.2.20	XAPP SHARED DATA FETCH REQUEST

This message is sent by the Near-RT RIC platform or a data consumer xApp to fetch data shared by the data producer xApp.

Direction: Near-RT RIC platform / Data consumer xApp -> Data producer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| SDL Data Description | O |  | 7.3.3.22 | This IE may be absent when a default definition of it exists and is used in the request. | YES | Reject |

7.3.2.21	XAPP SHARED DATA FETCH RESPONSE

This message is sent by the data producer xApp containing the fetched data requested by the Near-RT RIC platform or the data consumer xApp.

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| SDL Data Block |  | 0..1 |  |  | YES | Reject |
| > SDL Data Block Item |  | 1..< maxnoofSDLDataUnits> |  |  | - | - |
| >> SDL Data Unit | M |  | 7.3.3.23 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSDLDataUnits | Maximum number of SDL Data Block Units that can be carried in an SDL Data Block. Value is 1048576. |

#### 7.3.2.22	XAPP SHARED DATA FETCH FAILURE

This message is sent by the data producer xApp to the Near-RT RIC platform or the data consuemr xApp if the xApp shared data fetch request fails.

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Procedure Transaction ID | YES | Reject |
| Fetch Failure Cause | M |  | 7.3.3.18 | Cause for failure | YES | Reject |

7.3.2.23	XAPP SHARED DATA SUBSCRIPTION REQUEST

This message is sent by the Near-RT RIC platform or a data consumer xApp to subscribe to data shared by the data producer xApp.

Direction: Near-RT RIC platform / Data consumer xApp -> Data producer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| SDL Data Description | O |  | 7.3.3.22 | This IE may be absent, if a default SDL Data Description is available for the SDL Type ID and used in this request. | YES | Reject |
| Delivery Method | M |  | 7.3.3.4 |  |  |  |
| Notification Destination | M |  | 10.1.9 |  | YES | Reject |
| Notification Criteria | M |  | 7.3.3.24 |  | YES | Reject |

7.3.2.24	XAPP SHARED DATA SUBSCRIPTION RESPONSE

This message is sent by the data producer xApp to indicate success of the subscription request.

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription ID | M |  | 7.3.3.26 |  | YES | Reject |

7.3.2.25	XAPP SHARED DATA SUBSCRIPTION FAILURE

This message is sent by the data producer xApp to indicate failure of the subscription request.

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription Reject Cause | M |  | 7.3.3.9 |  | YES | Reject |

#### 7.3.2.26	XAPP SHARED DATA PUSH

This message is sent by the data producer xApp to push its shared data to the subscriber (data consumer xApp or the Near-RT RIC platform).

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Subscription ID | M |  | 7.3.3.26 | The same Subscription ID as in the subscription response. | YES | Reject |
| SDL Data Type ID | M |  | 7.3.3.21 |  | YES | Reject |
| SDL Data Block |  | 0..1 |  |  | YES | Reject |
| > SDL Data Block Item |  | 1..< maxnoofSDLDataUnits > |  |  | - | - |
| >> Modification Type | C |  | 7.3.3.10 | Shall be present when the Delivery Method IE in the subscription request is set to "PUSH_DELTA". | - | - |
| >> SDL Data Unit | M |  | 7.3.3.23 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSDLDataUnits | Maximum number of SDL Data Units that can be carried in an SDL Data Block. Value is 1048576. |

7.3.2.27	XAPP SHARED DATA SUBSCRIPTION DELETE REQUEST

This message is sent by the Near-RT RIC platform or a data consumer xApp to delete an existing subscription to the xApp shared data.

Direction: Near-RT RIC platform / Data consumer xApp -> Data producer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription ID | M |  | 7.3.3.26 | The same Subscription ID as in the subscription response. | YES | Reject |

7.3.2.28	XAPP SHARED DATA SUBSCRIPTION DELETE RESPONSE

This message is sent by the data producer xApp to indicate successful deletion of an existing subscription to the xApp shared data.

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |

7.3.2.29	XAPP SHARED DATA SUBSCRIPTION DELETE FAILURE

This message is sent by the data producer xApp to indicate failure of the subscription request.

Direction: Data producer xApp  -> Near-RT RIC platform / Data consumer xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 7.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| Subscription Reject Cause | M |  | 7.3.3.9 |  | YES | Reject |

7.3.3	Information Element Definitions

7.3.3.1	Message Type

The Message Type IE identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Type | M |  | CHOICE (SDL Subscription Information, SDL Information Push, SDL Information Update Notify, SDL Information Fetch, Xapp Shared Data Fetch, Xapp Shared Data Subscribe, Xapp Shared Data Subscription Delete, Xapp Shared Data Push, …) |  |
| >Type of Message | M |  | CHOICE (Request, Response, Failure, …) | Refer to Table 7.1.2-3 for applicable choices for each procedure type |

7.3.3.2	Void

7.3.3.3	Void

7.3.3.4	Delivery Method

The Delivery Method IE indicates the delivery method in which the xApp wants the information to be delivered.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Delivery Method | M |  | ENUMERATED (UPDATE_NOTIFY, PUSH_DELTA, PUSH_FULL, …) |  |

Table 7.3.3.4-1: Meaning of Delivery Method

| Value | Description |
| --- | --- |
| UPDATE_NOTIFY | Notifies only the summary of change in the data specified by the SDL Data Description IE. It uses the SDL Information Update Notify procedure for the notification. |
| PUSH_DELTA | Notifies the data that has changed since the last notification associated with the same subscription, from the data specified by the SDL Data Description IE. It uses the SDL Information Push procedure for the notification. |
| PUSH_FULL | Notifies all the data specified by the SDL Data Description IE. It uses the SDL Information Push procedure for the notification. |

7.3.3.5	Void

7.3.3.6	Void

7.3.3.7	Void

7.3.3.8	Void

7.3.3.9	Subscription Reject Cause

The Subscription Reject Cause IE indicates the reason the SDL rejected the subscription.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subscription Reject Cause | M |  | ENUMERATED (SDL reached capacity, Information type not supported, Invalid filter, Unspecified, …) |  |

7.3.3.10	Modification Type

The Modification Type IE informs the type of modification that the SDL underwent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Modification Type | M |  | ENUMERATED (Added, Modified, Removed, ….) |  |

7.3.3.11	Void

7.3.3.12	Void

7.3.3.13	Time to wait

This IE defines the minimum allowed waiting times.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time to wait | M |  | INTEGER (1..300) | Unit in seconds. |

7.3.3.14	Void

7.3.3.15	Void

7.3.3.16	Void

7.3.3.17	Void

7.3.3.18	Fetch Failure Cause

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Fetch Failure Cause | M |  | ENUMERATED (Invalid filter, Information type not supported, No information present, Unspecified, …) |  |

7.3.3.19	Void

7.3.3.20	Void

#### 7.3.3.21	SDL Data Type ID

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SDL Data Type ID | M |  | STRING | Uniquely identifies a SDL data type. It shall be constructed with 3 fields concatenated by “:” (colon), as "{namespace}:{name}:{version}". Clause 7.3.3.28 collects the known SDL Data Type IDs per data category. |

#### 7.3.3.22	SDL Data Description

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Data Category |  |  |  |  |
| > E2 Node Information |  |  |  |  |
| >> SDL Data Description For E2 Node Information | M |  | 7.3.3.28.1.2 |  |
| > RAN Analytics Information |  |  |  |  |
| >> SDL Data Description For RAI | M |  | 7.3.3.28.2.2 |  |

#### 7.3.3.23	SDL Data Unit

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Data Category |  |  |  |  |
| > E2 Node Information |  |  |  |  |
| >> SDL Data Unit For E2 Node Information | M |  | 7.3.3.28.1.3 |  |
| > RAN Analytics Information |  |  |  |  |
| >> SDL Data Unit For RAI | M |  | 7.3.3.28.2.3 |  |

#### 7.3.3.24	Notification Criteria

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Method | M |  | ENUMERATED (PERIODIC, EVENT_TRIGGERED, …) |  |
| Notification Period | C |  | INTEGER | Indicates the periodicity of notifications, in units of milliseconds. Shall be present when Notification Method is set to "PERIODIC". |
| Trigger Event Definition | C |  | 7.3.3.25 | Shall be present when Notification Method is set to "EVENT_TRIGGERED". |

#### 7.3.3.25	Trigger Event Definition

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Data Category |  |  |  |  |
| > E2 Node Information |  |  |  |  |
| >> Trigger Event Definition For E2 Node Information | M |  | 7.3.3.28.1.4 |  |
| > RAN Analytics Information |  |  |  |  |
| >> Trigger Event Definition For RAI | M |  | 7.3.3.28.2.4 |  |

#### 7.3.3.26	Subscription ID

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subscription ID | M |  | STRING | Uniquely identifies a SDL data subscription among all the SDL subscriptions created by a consumer. |

#### 7.3.3.27	SDL Update Notify Unit

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Data Category |  |  |  |  |
| > E2 Node Information |  |  |  |  |
| >> SDL Update Notify Unit For E2 Node Information | M |  | 7.3.3.28.1.5 |  |

#### 7.3.3.28	SDL Data Type specific IE Definitions

7.3.3.28.1		E2 Node Information related IEs

7.3.3.28.1.1	SDL Data Type ID

To construct the SDL Data Type ID IE for E2 Node Information,

-	the namespace field is "ORAN";

-	the name field is the text value of the Information Type IE in clause 7.3.3.28.1.6, i.e., "E2NodeList", "E2NodeComponentList", "E2NodeRanFunctionList", etc.;

-	the version field is "1.0.0".

EXAMPLE:	"ORAN: E2NodeList:1.0.0"

7.3.3.28.1.2	SDL Data Description IE for E2 Node Information

| IE Name | Presence | Range | Data type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Type | M |  | 7.3.3.28.1.6 |  |
| > Filter List |  | 0..1 |  | Specifies the subset of the data to be subscribed for the Information Type. Multiple filters can be specified, in which case, a logical AND shall be performed across all filters. |
| >> Filter Item |  | 0..<maxFilterID> |  |  |
| >>> Filter Definition | M |  | 7.3.3.28.1.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofFilters | Maximum number of Filter Items to be contained in the Filter List. Value is 256. |

7.3.3.28.1.3	SDL Data Unit IE for E2 Node Information

| IE Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Unit | M |  | 7.3.3.28.1.11 |  |

7.3.3.28.1.4	Trigger Event Definition IE For E2 Node Information

| IE Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notify On Data Change Flag | O |  | ENUMERATED (TRUE, ...) | When the IE is present and set to "TRUE", the notifications shall be triggered when the E2 Node information described by the SDL Data Description IE changes since the last nofication associated with the same subscription. |

7.3.3.28.1.5	SDL Update Notify IE For E2 Node Information

| IE Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Unit |  |  | 7.3.3.28.1.12 |  |

7.3.3.28.1.6	Information Type

The IE indicates the type of E2 Node information the xApp requests.

| IE Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Type | M |  | ENUMERATED (E2NodeList, E2NodeComponentConfig, E2NodeRanFunction, …) |  |

7.3.3.28.1.7	Filter Definition

The IE defnes a filter on the E2 Node information that the xApp requests. A Filter Definition is satisfied when the logical statement “X p Y” is True, where:

-	X is the filter type, represented by the Filter Type IE;

-	p is the filter operator, represented by the Filter Condition IE;

-	Y is the set of the target values, represented by the Filter Condition Value IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Filter Type | M |  | 7.3.3.28.1.8 |  |
| Filter Condition | M |  | 7.3.3.28.1.9 |  |
| Filter Condition Value | M |  | 7.3.3.28.1.10 |  |

7.3.3.28.1.8	Filter Type

The IE defines the filtering condition type to identify the E2 Node information the xApp request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Filter Type | M |  | ENUMERATED (E2NodeID, ComponentType, ServiceModel, …) | Valid values of Filter Type IE are depending on the type of E2 Node Information indicated in the Information Type IE. See table 7.3.3.28.1.8-1 for the valid settings. |

Table 7.3.3.28.1.8-1: Valid Filter Type IE settings

| Filter Type | Information Type |  |  |
| --- | --- | --- | --- |
|  | E2NodeList | E2NodeComponentConfig | E2NodeRanFunction |
| E2NodeID | NO | YES | YES |
| ComponentType | YES | YES | NO |
| ServiceModel | YES | NO | YES |

7.3.3.28.1.9	Filter Condition

The IE defines the filtering condition to identify the E2 Node information the xApp requests.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Filter Condition | M |  | ENUMERATED (ContainedIn, NotContainedIn, Equals, GreanterThan, LessThan, …) |  |

7.3.3.28.1.10	Filter Condition Value

The IE defines the filtering condition value to identify the information the xApp requests.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Filter Condition Value |  |  |  |  |
| >E2NodeID |  |  |  |  |
| >>E2 Node ID List |  | 1 |  |  |
| >>>E2 Node Item |  | 1..65536 |  |  |
| >>>>E2 Node ID | M |  | 7.3.3.14 |  |
| >>>>E2 Node State | O |  | 7.3.3.28.1.13 |  |
| >ComponentType |  |  |  |  |
| >>E2 Node Component Type List |  | 1 |  |  |
| >>>E2 Node Component Type Item |  | 1..65536 |  |  |
| >>>>E2 Node Component Type | M |  | E2 Node Component Interface Type IE, clause 9.2.26 of [3] |  |
| >ServiceModel |  |  |  |  |
| >>RAN Function OID List |  | 1 |  |  |
| >>>RAN Function OID Item |  | 1..65536 |  |  |
| >>>>RAN function OID | M |  | RAN Function OID IE, clause 9.2.31 of [3] |  |

7.3.3.28.1.11	Information Unit

The IE contains the E2 Node information that the Near-RT RIC platform sends to the xApp.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Information Unit | M |  | Contents depend on Information Type. See following table for allowed settings. |  |
| >E2 Node List Unit |  |  |  |  |
| >>E2 Node ID | M |  | 10.1.6 |  |
| >>E2 Node State | M |  | 7.3.3.28.1.13 |  |
| >E2 Node Component Config Unit |  |  |  |  |
| >>E2 Node ID | M |  | 10.1.6 |  |
| >>E2 Node Component Config List |  | 1..<maxofE2nodeComponents> |  | Value of "maxofE2nodeComponents" defined in 9.1.2.2 of [3]. |
| >>>E2 Node Component Type | M |  | E2 Node Component Interface Type IE, clause 9.2.26 of [3] |  |
| >>>E2 Node Component ID | O |  | Clause 9.2.32 of [3] |  |
| >>>E2 Node Component Configuration | M |  | Clause 9.2.27 of [3] |  |
| >>> Last Modified E2 Node Version | M |  | INTEGER | The Near-RT RIC platform maintains a version number for the E2 Node Information of each E2 Node. Incremented whenever any RAN Funciton or Component config changes. This is the version number when the E2 Node Information changed last time. |
| > Ran Function Unit |  |  |  |  |
| >>E2 Node ID | M |  | 10.1.6 |  |
| >>RAN Function List |  | 1..<maxofRANfunctionID> |  | Value of "maxofRANfunctionIDs" defined in 9.1.2.2 of [3]. |
| >>> RAN Function ID | M |  | 10.1.7 |  |
| >>> RAN Function Definition | M |  | Clause 9.2.23 of [3]. |  |
| >>> RAN Function OID | M |  | Clause 9.2.31 of [3]. |  |
| >>> Last Modified E2 Node Version | M |  | INTEGER | The Near-RT RIC platform maintains a version number for the E2 Node Information of each E2 Node. Incremented whenever any RAN Funciton or Component config changes. This is the version number when the E2 Node Information changed last time. |

| Information Type | Information Unit |
| --- | --- |
| E2NodeList | E2 Node List Unit |
| E2NodeComponentConfig | E2 Node Component Config Unit |
| E2NodeRanFunction | Ran Function Unit |

7.3.3.28.1.12	Notification Unit

The IE contains the summary of E2 Node Information updates sent to the xApp.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Node ID | M |  | 10.1.6 |  |
| Current E2 Node Version | M |  | INTEGER | The Near-RT RIC platform maintains the version number for E2 Node Information of each E2 Node. Incremented whenever any RAN Funciton or Component config changes. This is the current version number of E2 Node Information for the E2 Node. |
| E2 Node State | O |  | 7.3.3.28.1.13 |  |

7.3.3.28.1.13	E2 Node State

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Node State | M |  | ENUMERATED (CONNECTED, RESET, DISCONNECTED, …) |  |

| E2 Node State | Meaning |
| --- | --- |
| CONNECTED | E2 Node has completed E2 Setup procedure. Near-RT RIC platform has at least one validated E2NodeComponentConfig and E2NodeRanFunction record available for xApps using SDL API. xApp may use E2 related API to request E2 Subscription and E2 Control procedures |
| RESET | E2 connection to E2 Node has been reset using E2AP Reset procedure. Near-RT RIC platform has at least one validated E2NodeComponentConfig and E2NodeRanFunction record available for xApps using SDL API. Previous E2 Subscriptions have been deleted E2 related API services not available |
| DISCONNECTED | E2 connection to E2 Node is not available xApp shall delete records for E2 Node |

7.3.3.28.2		RAN Analytics Information related IEs

7.3.3.28.2.1	SDL Data Type ID

To construct the SDL Data Type ID IE for RAN Analytics Information,

-	the namespace field is "ORAN";

-	the name and version fields are the RAI Type ID and the Version defined in Table 6.1-1 of Y1TD [11], respectively.

EXAMPLE:	"ORAN:RAN performance analytics:1.0.0"

7.3.3.28.2.2	SDL Data Description For RAI

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target Entitiy List |  | 1 |  | Represents the "targetEntities" attribute in clause 6.2.2.5.2.2 and 6.3.2.2.2.3.1 of Y1AP [10]. |
| > Target Entity Item |  | 1..<maxnoofTargetEntities> |  |  |
| >> Target Entity | M |  | TargetEntity, clause 6 of Y1TD [11] |  |
| Filter Paramters | O |  | FilterParameters, clause 6 of Y1TD [11] | Represents the "filterParameters" attribute in clause 6.2.2.5.2.2 and 6.3.2.2.2.3.1 of Y1AP [10]. |
| Expected Validity Period Relative List |  | 0..1 |  | Represents the "expectedValidityPeriodsRelative" attribute in clause 6.2.2.5.2.2 of Y1AP [10]. |
| > Expected Validity Period Relative Item |  | 1..<maxnoofValidityPeriods> |  |  |
| >> Expected Validity Period Relative | M |  | ValidityPeriodRelative, clause 6.2.2.5.2.6 of Y1AP [10] |  |
| Expected Validity Period List |  | 0..1 |  | Represents the "expectedValidity Periods" attribute in clause 6.3.2.2.2.3.1 of Y1AP [10]. |
| > Expected Validity Period Item |  | 1..<maxnoofValidityPeriods> |  |  |
| >> Expected Validity Period | M |  | ValidityPeriod, clause 6.2.2.5.2.7 of Y1AP [10] |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTargetEntities | Maximum number of target entities that can be carried in a target entity list. Value is 2048. |
| maxnoofValidityPeriods | Maximum number of validity periods that can be carried in a validity period list. Value is 512. |

7.3.3.28.2.3	SDL Data Unit For RAI

| IE Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAI Report | M |  | RaiReport, clause 6.2.2.5.2.5 of Y1AP [10] | Represents a single RAI report in clause 6.2 and 6.3 of Y1AP [10]. |

7.3.3.28.2.4	Trigger Event Definition For RAI

| IE Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Event Trigger | M |  | NotificationTriggerEvent, clause 6 of Y1TD [11] | Represents the "notificationTriggerEvent" attribute in clause 6.2.2.5.2.3 of Y1AP [10]. |

7.3.4	Message and Information Element Abstract Syntax

#### 7.3.4.1	Solution 1: SCTP with Protocol Buffers

##### 7.3.4.1.1	Usage of SCTP with Protocol Buffers

This solution uses Protocol Buffers as message encoding, and SCTP as the transport protocol.

The PDU on the SCTP connection uses the structure as defined in clause 10.1.8 where its API Payload IE serves as a container for individual API messages in clause 7.3.4.1.2 which can be SdlInformationApiPayload or XappDataSharingApiPayload.

Table 7.3.4.1.1-1 lists the SDL APIs abstract syntax included in this document.

Table 7.3.4.1.1-1: List of SDL APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| SDL_Information_API | 7.3.4.1.2.1 | 2.0.0 | sdl_information_api_ver_2_0_0.proto |
| Xapp_Data_Sharing_API | 7.3.4.1.2.2 | 1.0.0 | xapp_data_sharing_api_ver_1_0_0.proto |
| SDL API common information elements | 7.3.4.1.3 | 2.0.0 | sdl_apis_common_ies_ver_2_0_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

7.3.4.1.2	API Definitions

7.3.4.1.2.1	SDL_Information_API

syntax = "proto3";

package ricapi.sdl_information_api.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "sdl_apis_common_ies_ver_2_0_0.proto";

// top-level PDU

message SdlInformationApiPayload {

oneof procedure_type {

SdlSubscribeInformationProc sdl_subscribe_information_proc = 1;

SdlInformationFetchProc sdl_information_fetch_proc = 2;

SdlInformationPushProc sdl_information_push_proc = 3;

SdlInformationUpdateNotifyProc sdl_information_update_notify_proc = 4;

}

}

// ---------------- procedures of SDL_Information_API ----------------

message SdlSubscribeInformationProc {

oneof type_of_message {

SubscribeInformationRequest subscribe_information_request = 1;

SubscribeInformationResponse subscribe_information_response = 2;

SubscribeInformationFailure subscribe_information_failure = 3;

}

}

message SdlInformationFetchProc {

oneof type_of_message {

InformationFetchRequest information_fetch_request = 1;

InformationFetchResponse information_fetch_response = 2;

InformationFetchFailure information_fetch_failure = 3;

}

}

message SdlInformationPushProc {

oneof type_of_message {

InformationPush information_push = 1;

}

}

message SdlInformationUpdateNotifyProc {

oneof type_of_message {

InformationUpdateNotify information_update_notify = 1;

}

}

// ---------------- messages of SDL_Information_API ----------------

// -------- messages of SDL Subscribe Information procedure --------

message SubscribeInformationRequest {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

optional ricapi.sdl_apis_common_ies.v2.DeliveryMethod delivery_method = 3;

optional ricapi.sdl_apis_common_ies.v2.SdlDataDescription sdl_data_description = 4;

optional ricapi.near_rt_ric_apis_common_ies.v2.EndpointInformation notification_destination = 5;

optional ricapi.sdl_apis_common_ies.v2.NotificationCriteria notification_criteria = 6;

}

message SubscribeInformationResponse {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionId subscription_id = 2;

}

message SubscribeInformationFailure {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionRejectCause subscription_reject_cause = 2;

optional uint32 time_to_wait = 3;

}

// -------- messages of SDL Information Fetch procedure --------

message InformationFetchRequest {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

optional ricapi.sdl_apis_common_ies.v2.SdlDataDescription sdl_data_description = 3;

}

message InformationFetchResponse {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

repeated SdlDataBlockItem sdl_data_block = 3;

message SdlDataBlockItem {

optional ricapi.sdl_apis_common_ies.v2.SdlDataUnit sdl_data_unit = 1;

}

}

message InformationFetchFailure {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.FetchFailureCause fetch_failure_cause = 2;

}

// -------- messages of SDL Information Push procedure --------

message InformationPush {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 4;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionId subscription_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

repeated SdlDataBlockItem sdl_data_block = 3;

message SdlDataBlockItem {

optional ricapi.sdl_apis_common_ies.v2.ModificationType modification_type = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataUnit sdl_data_unit = 2;

}

}

// -------- messages of SDL Information Update Notify procedure --------

message InformationUpdateNotify {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 4;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionId subscription_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

repeated SdlUpdateNotifyBlockItem sdl_update_notify_block = 3;

message SdlUpdateNotifyBlockItem {

optional ricapi.sdl_apis_common_ies.v2.ModificationType modification_type = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlUpdateNotifyUnit sdl_update_notify_unit = 2;

}

}

7.3.4.1.2.2	Xapp_Data_Sharing_API

syntax = "proto3";

package ricapi.xapp_data_sharing_api.v1;

import "sdl_apis_common_ies_ver_2_0_0.proto";

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

// top-level PDU

message XappDataSharingApiPayload {

oneof procedure_type {

XappSharedDataFetchProc xapp_shared_data_fetch_proc = 1;

XappSharedDataSubscribeProc xapp_shared_data_subscribe_proc = 2;

XappSharedDataSubscriptionDeleteProc xapp_shared_data_subscription_delete_proc = 3;

XappSharedDataPushProc xapp_shared_data_push_proc = 4;

}

}

// ---------------- procedures of Xapp_Data_Sharing_API ----------------

message XappSharedDataFetchProc {

oneof type_of_message {

XappSharedDataFetchRequest xapp_shared_data_fetch_request = 1;

XappSharedDataFetchResponse xapp_shared_data_fetch_response = 2;

XappSharedDataFetchFailure xapp_shared_data_fetch_failure = 3;

}

}

message XappSharedDataSubscribeProc {

oneof type_of_message {

XappSharedDataSubscriptionRequest xapp_shared_data_subscription_request = 1;

XappSharedDataSubscriptionResponse xapp_shared_data_subscription_response = 2;

XappSharedDataSubscriptionFailure xapp_shared_data_subscription_failure = 3;

}

}

message XappSharedDataSubscriptionDeleteProc {

oneof type_of_message {

XappSharedDataSubscriptionDeleteRequest xapp_shared_data_subscription_delete_request = 1;

XappSharedDataSubscriptionDeleteResponse xapp_shared_data_subscription_delete_response = 2;

XappSharedDataSubscriptionDeleteFailure xapp_shared_data_subscription_delete_failure = 3;

}

}

message XappSharedDataPushProc {

oneof type_of_message {

XappSharedDataPush xapp_shared_data_push = 1;

}

}

// ---------------- messages of Xapp_Data_Sharing_API ----------------

// -------- messages of Xapp Shared Data Fetch procedure --------

message XappSharedDataFetchRequest {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

optional ricapi.sdl_apis_common_ies.v2.SdlDataDescription sdl_data_description = 3;

}

message XappSharedDataFetchResponse {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

repeated SdlDataBlockItem sdl_data_block = 3;

message SdlDataBlockItem {

optional ricapi.sdl_apis_common_ies.v2.SdlDataUnit sdl_data_unit = 1;

}

}

message XappSharedDataFetchFailure {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.FetchFailureCause fetch_failure_cause = 2;

}

// -------- messages of Xapp Shared Data Subscribe procedure --------

message XappSharedDataSubscriptionRequest {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

optional ricapi.sdl_apis_common_ies.v2.SdlDataDescription sdl_data_description = 3;

optional ricapi.sdl_apis_common_ies.v2.DeliveryMethod delivery_method = 4;

optional ricapi.near_rt_ric_apis_common_ies.v2.EndpointInformation notification_destination = 5;

optional ricapi.sdl_apis_common_ies.v2.NotificationCriteria notification_criteria = 6;

}

message XappSharedDataSubscriptionResponse {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionId subscription_id = 2;

}

message XappSharedDataSubscriptionFailure {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionRejectCause subscription_reject_cause = 2;

}

// -------- messages of Xapp Shared Data Push procedure --------

message XappSharedDataPush {

optional ricapi.sdl_apis_common_ies.v2.SubscriptionId subscription_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataTypeId sdl_data_type_id = 2;

repeated SdlDataBlockItem sdl_data_block = 3;

message SdlDataBlockItem {

optional ricapi.sdl_apis_common_ies.v2.ModificationType modification_type = 1;

optional ricapi.sdl_apis_common_ies.v2.SdlDataUnit sdl_data_unit = 2;

}

}

// -------- messages of Xapp Shared Data Subscription Delete procedure --------

message XappSharedDataSubscriptionDeleteRequest {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionId subscription_id = 2;

}

message XappSharedDataSubscriptionDeleteResponse {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

}

message XappSharedDataSubscriptionDeleteFailure {

optional ricapi.near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional ricapi.sdl_apis_common_ies.v2.SubscriptionRejectCause subscription_reject_cause = 2;

}

7.3.4.1.3	Information Element Definitions

syntax = "proto3";

package ricapi.sdl_apis_common_ies.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

// ----------------------------- Information Elements -----------------------------

// --------------A--------------

// From E2AP Clause 9.2.32

message AmfName {

optional string value = 1;

}

// --------------D--------------

enum DeliveryMethod {

DELIVERY_METHOD_UPDATE_NOTIFY = 0;

DELIVERY_METHOD_PUSH_DELTA = 1;

DELIVERY_METHOD_PUSH_FULL = 2;

}

// --------------E--------------

// From E2AP Clause 9.2.27

message E2NodeComponentConfiguration {

optional bytes e2_node_component_request_part = 1;

optional bytes e2_node_component_response_part = 2;

}

// From E2AP Clause 9.2.32

message E2NodeComponentId {

oneof e2_node_component_interface_type {

NgInterfaceType ng = 1;

XnInterfaceType xn = 2;

E1InterfaceType e1 = 3;

F1InterfaceType f1 = 4;

W1InterfaceType w1 = 5;

S1InterfaceType s1 = 6;

X2InterfaceType x2 = 7;

}

message NgInterfaceType {

optional AmfName amf_name = 1;

}

message XnInterfaceType {

optional GlobalNgRanNodeId global_ng_ran_node_id = 1;

}

message E1InterfaceType {

optional near_rt_ric_apis_common_ies.v2.GnbCuUpId gnb_cu_up_id = 1;

}

message F1InterfaceType {

optional near_rt_ric_apis_common_ies.v2.GnbDuId gnb_du_id = 1;

}

message W1InterfaceType {

optional near_rt_ric_apis_common_ies.v2.NgEnbDuId ng_enb_du_id = 1;

}

message S1InterfaceType {

optional MmeName mme_name = 1;

}

message X2InterfaceType {

optional near_rt_ric_apis_common_ies.v2.GlobalEnbId global_enb_id = 1;

optional near_rt_ric_apis_common_ies.v2.GlobalEnGnbId global_en_gnb_id = 2;

}

}

// From E2AP Clause 9.2.26

enum E2NodeComponentType {

E2_NODE_COMPONENT_TYPE_NG = 0;

E2_NODE_COMPONENT_TYPE_XN = 1;

E2_NODE_COMPONENT_TYPE_E1 = 2;

E2_NODE_COMPONENT_TYPE_F1 = 3;

E2_NODE_COMPONENT_TYPE_W1 = 4;

E2_NODE_COMPONENT_TYPE_S1 = 5;

E2_NODE_COMPONENT_TYPE_X2 = 6;

}

enum E2NodeState {

E2_NODE_STATE_CONNECTED = 0;

E2_NODE_STATE_RESET = 1;

E2_NODE_STATE_DISCONNECTED = 2;

}

// --------------F--------------

enum FetchFailureCause {

FETCH_FAILURE_CAUSE_UNSPECIFIED = 0;

FETCH_FAILURE_CAUSE_INVALID_FILTER = 1;

FETCH_FAILURE_CAUSE_INFORMATION_TYPE_NOT_SUPPORTED = 2;

FETCH_FAILURE_CAUSE_NO_INFORMATION_PRESENT = 3;

}

enum FilterCondition {

FILTER_CONDITION_CONTAINTED_IN = 0;

FILTER_CONDITION_NOT_CONTAINTED_IN = 1;

FILTER_CONDITION_EQUALS = 2;

FILTER_CONDITION_GREATER_THAN = 3;

FILTER_CONDITION_LESS_THAN = 4;

}

message FilterConditionValue {

oneof filter_condition_value_type {

E2NodeIdList e2_node_id_list = 1;

E2NodeComponentTypeList e2_node_component_type_list = 2;

RanFunctionOidList ran_function_oid_list = 3;

}

message E2NodeIdList {

repeated E2NodeIdItem list = 1;

message E2NodeIdItem {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId e2_node_id = 1;

optional E2NodeState e2_node_state = 2;

}

}

message E2NodeComponentTypeList {

repeated E2NodeComponentTypeItem list = 1;

message E2NodeComponentTypeItem {

optional E2NodeComponentType e2_node_component_type = 1;

}

}

message RanFunctionOidList {

repeated RanFunctionOidItem list = 1;

message RanFunctionOidItem {

optional RanFunctionOid ran_function_oid = 1;

}

}

}

message FilterDefinition {

optional FilterType filter_type = 1;

optional FilterCondition filter_condition = 2;

optional FilterConditionValue filter_condition_value = 3;

}

enum FilterType {

FILTER_TYPE_E2_NODE_ID = 0;

FILTER_TYPE_COMPONENT_TYPE = 1;

FILTER_TYPE_SERVICE_MODEL = 2;

}

// --------------G--------------

// From E2AP Clause 9.2.32

message GlobalNgRanNodeId {

oneof ng_ran_node_choice {

near_rt_ric_apis_common_ies.v2.GlobalGnbId global_gnb_id = 1;

near_rt_ric_apis_common_ies.v2.GlobalNgEnbId global_ng_enb_id = 2;

}

}

// --------------I--------------

enum InformationType {

INFORMATION_TYPE_E2NODELIST = 0;

INFORMATION_TYPE_E2NODECOMPONENTCONFIG = 1;

INFORMATION_TYPE_E2NODERANFUNCTION = 2;

}

message InformationUnit {

oneof information_unit_type {

E2NodeListUnit e2_node_list_unit = 1;

E2NodeComponentConfigUnit e2_node_component_config_unit = 2;

RanFunctionUnit ran_function_unit = 3;

}

message E2NodeListUnit {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId e2_node_id = 1;

optional E2NodeState e2_node_state = 2;

}

message E2NodeComponentConfigUnit {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId e2_node_id = 1;

repeated E2NodeComponentConfigItem e2_node_component_config_list = 2;

message E2NodeComponentConfigItem {

optional E2NodeComponentType e2_node_component_type = 1;

optional E2NodeComponentId e2_node_component_id = 2;

optional E2NodeComponentConfiguration e2_node_component_configuration = 3;

optional uint32 last_modified_e2_node_version = 4;

}

}

message RanFunctionUnit {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId e2_node_id = 1;

repeated RanFunctionItem ran_function_list = 2;

message RanFunctionItem {

optional near_rt_ric_apis_common_ies.v2.RanFunctionId ran_function_id = 1;

optional RanFunctionDefinition ran_function_definition = 2;

optional RanFunctionOid ran_function_oid = 3;

optional uint32 last_modified_e2_node_version = 4;

}

}

}

// --------------M--------------

enum ModificationType {

TYPE_ADDED = 0;

TYPE_MODIFIED = 1;

TYPE_REMOVED = 2;

}

// From E2AP Clause 9.2.32

message MmeName {

optional string value = 1;

}

// --------------N--------------

message NotificationCriteria {

optional NotificationMethod notification_method = 1;

optional uint32 notification_period = 2;

optional TriggerEventDefinition trigger_event_definition = 3;

enum NotificationMethod {

NOTIFICATION_METHOD_PERIODIC = 0;

NOTIFICATION_METHOD_EVENT_TRIGGERED = 1;

}

}

message NotificationUnit {

optional near_rt_ric_apis_common_ies.v2.GlobalE2NodeId e2_node_id = 1;

optional uint32 current_e2_node_version = 2;

optional E2NodeState e2_node_state = 3;

}

// --------------R--------------

// From E2AP Clause 9.2.23

message RanFunctionDefinition {

optional bytes value = 1;

}

// From E2AP Clause 9.2.31

message RanFunctionOid {

optional string value = 1;

}

// --------------S--------------

message SdlDataDescription {

oneof data_category {

SdlDataDescriptionForE2NodeInformation sdl_data_description_for_e2_node_information = 1;

SdlDataDescriptionForRai sdl_data_description_for_rai = 2;

}

}

message SdlDataDescriptionForE2NodeInformation {

optional InformationType information_type = 1;

repeated FilterItem filter_list = 2;

message FilterItem {

optional FilterDefinition filter_definition = 1;

}

}

message SdlDataDescriptionForRai {

repeated TargetEntityItem target_entity_list = 1;

optional bytes filter_parameters = 2;

repeated ExpectedValidityPeriodRelativeItem expected_validity_period_relative_list = 3;

repeated ExpectedValidityPeriodItem expected_validity_period_list = 4;

message TargetEntityItem {

optional bytes target_entity = 1;

}

message ExpectedValidityPeriodRelativeItem {

optional bytes expected_valitity_period_relative = 1;

}

message ExpectedValidityPeriodItem {

optional bytes expected_valitity_period = 1;

}

}

message SdlDataTypeId {

optional string value = 1;

}

message SdlDataUnit {

oneof data_category {

SdlDataUnitForE2NodeInformation sdl_data_unit_for_e2_node_information = 1;

SdlDataUnitForRai sdl_data_unit_for_rai = 2;

}

}

message SdlDataUnitForE2NodeInformation {

optional InformationUnit information_unit = 1;

}

message SdlDataUnitForRai {

optional bytes rai_report = 1;

}

message SdlUpdateNotifyUnit {

oneof data_category {

SdlUpdateNofityUnitForE2NodeInformation sdl_update_notify_unit_for_e2_node_information = 1;

}

}

message SdlUpdateNofityUnitForE2NodeInformation {

optional NotificationUnit notification_unit = 1;

}

message SubscriptionId {

optional string subscription_id = 1;

}

enum SubscriptionRejectCause {

SUBSCRIPTION_REJECT_CAUSE_UNSPECIFIED = 0;

SUBSCRIPTION_REJECT_CAUSE_SDL_REACHED_CAPACITY = 1;

SUBSCRIPTION_REJECT_CAUSE_INFORMATION_TYPE_NOT_SUPPORTED = 2;

SUBSCRIPTION_REJECT_CAUSE_INVALID_FILTER = 3;

}

// --------------T--------------

message TimeToWait {

optional uint32 value = 1;

}

message TriggerEventDefinition {

oneof data_category {

TriggerEventDefinitionForE2NodeInformation trigger_event_definition_for_e2_node_information = 1;

TriggerEventDefinitionForRai trigger_event_definition_for_rai = 2;

}

}

message TriggerEventDefinitionForE2NodeInformation {

optional EnumValue notify_on_data_change = 1;

enum EnumValue {

ENUM_VALUE_TRUE = 0;

}

}

message TriggerEventDefinitionForRai {

optional bytes notification_event_trigger = 1;

}

#### 7.3.4.2	Solution 2: gRPC with Protocol Buffers

##### 7.3.4.2.1	Usage of gRPC with Protocol Buffers

This solution uses Protocol Buffers as message encoding, also as the Interface Definition Language (IDL) for gRPC [5].

Table 7.3.4.2.1-1 lists the mapping of enablement APIs and corresponding gRPC services.

Table 7.3.4.2.1-1: Mapping of SDL APIs and gRPC services

| API Name | gRPC service(s) |
| --- | --- |
| SDL_Information_API | Sdl_Information_API, SDL_Information_API_Callback |
| Xapp_Data_Sharing_API | Xapp_Data_Sharing_API, Xapp_Data_Sharing_API_Callback |

A procedure of an SDL API is mapped to a gRPC method.

Table 7.3.4.1.1-1 lists the SDL APIs abstract syntax included in this document.

Table 7.3.4.2.1-1: List of SDL APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| SDL_Information_API | 7.3.4.2.2.1 | 1.0.0 | sdl_information_api_grpc_ver_1_0_0.proto |
| Xapp_Data_Sharing_API | 7.3.4.2.2.2 | 1.0.0 | xapp_data_sharing_api_grpc_ver_1_0_0.proto |
| SDL API common information elements | 7.3.4.1.3 | 2.0.0 | sdl_apis_common_ies_ver_2_0_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

##### 7.3.4.2.2	API Definitions

###### 7.3.4.2.2.1	SDL_Information_API

syntax = "proto3";

package ricapi.sdl_information_api_grpc.v1;

import "sdl_information_api_ver_2_0_0.proto";

// gRPC service and rpc definition

service Sdl_Information_API {

rpc SdlSubscribeInformationProc(SdlSubscribeInformationInitMsg) returns (SdlSubscribeInformationOutMsg) {};

rpc SdlInformationFetchProc(SdlInformationFetchInitMsg) returns (SdlInformationFetchOutMsg) {};

}

message SdlSubscribeInformationInitMsg {

optional ricapi.sdl_information_api.v2.SubscribeInformationRequest subscribe_information_request = 1;

}

message SdlSubscribeInformationOutMsg {

oneof type_of_message {

ricapi.sdl_information_api.v2.SubscribeInformationResponse subscribe_information_response = 1;

ricapi.sdl_information_api.v2.SubscribeInformationFailure subscribe_information_failure = 2;

}

}

message SdlInformationFetchInitMsg {

optional ricapi.sdl_information_api.v2.InformationFetchRequest information_fetch_request = 1;

}

message SdlInformationFetchOutMsg {

oneof type_of_message {

ricapi.sdl_information_api.v2.InformationFetchResponse information_fetch_response = 1;

ricapi.sdl_information_api.v2.InformationFetchFailure information_fetch_failure = 2;

}

}

service Sdl_Information_API_Callback {

rpc SdlInformationPushProc(SdlInformationPushInitMsg) returns (SdlInformationPushOutMsg) {};

rpc SdlInformationUpdateNotifyProc(SdlInformationUpdateNotifyInitMsg) returns (SdlInformationUpdateNofityNotifyInitMsg) {};

}

message SdlInformationPushInitMsg {

optional ricapi.sdl_information_api.v2.InformationPush information_push = 1;

}

message SdlInformationPushOutMsg {

}

message SdlInformationUpdateNotifyInitMsg {

optional ricapi.sdl_information_api.v2.InformationUpdateNotify information_update_notify = 1;

}

message SdlInformationUpdateNofityNotifyInitMsg {

}

###### 7.3.4.2.2.2	Xapp_Data_Sharing_API

syntax = "proto3";

package ricapi.xapp_data_sharing_api_grpc.v1;

import "xapp_data_sharing_api_ver_1_0_0.proto";

// gRPC service and rpc definition

service Xapp_Data_Sharing_API {

rpc XappSharedDataFetchProc(XappSharedDataFetchInitMsg) returns (XappSharedDataFetchOutMsg) {};

rpc XappSharedDataSubscribeProc(XappSharedDataSubscribeInitMsg) returns (XappSharedDataSubscribeOutMsg) {};

rpc XappSharedDataSubscriptionDeleteProc(XappSharedDataSubscriptionDeleteInitMsg) returns (XappSharedDataSubscriptionDeleteOutMsg) {};

}

message XappSharedDataFetchInitMsg {

optional ricapi.xapp_data_sharing_api.v1.XappSharedDataFetchRequest xapp_shared_data_fetch_request = 1;

}

message XappSharedDataFetchOutMsg {

oneof type_of_message {

ricapi.xapp_data_sharing_api.v1.XappSharedDataFetchResponse xapp_shared_data_fetch_response = 1;

ricapi.xapp_data_sharing_api.v1.XappSharedDataFetchFailure xapp_shared_data_fetch_failure = 2;

}

}

message XappSharedDataSubscribeInitMsg {

optional ricapi.xapp_data_sharing_api.v1.XappSharedDataSubscriptionRequest xapp_shared_data_subscription_request = 1;

}

message XappSharedDataSubscribeOutMsg {

oneof type_of_message {

ricapi.xapp_data_sharing_api.v1.XappSharedDataSubscriptionResponse xapp_shared_data_subscription_response = 1;

ricapi.xapp_data_sharing_api.v1.XappSharedDataSubscriptionFailure xapp_shared_data_subscription_failure = 2;

}

}

message XappSharedDataSubscriptionDeleteInitMsg {

optional ricapi.xapp_data_sharing_api.v1.XappSharedDataSubscriptionDeleteRequest xapp_shared_data_subscription_delete_request = 1;

}

message XappSharedDataSubscriptionDeleteOutMsg {

oneof type_of_message {

ricapi.xapp_data_sharing_api.v1.XappSharedDataSubscriptionDeleteResponse xapp_shared_data_subscription_delete_response = 2;

ricapi.xapp_data_sharing_api.v1.XappSharedDataSubscriptionDeleteFailure xapp_shared_data_subscription_delete_failure = 3;

}

}

service Xapp_Data_Sharing_API_Callback {

rpc XappSharedDataPushProc(XappSharedDataPushInitMsg) returns (XappSharedDataPushOutMsg) {};

}

message XappSharedDataPushInitMsg {

optional ricapi.xapp_data_sharing_api.v1.XappSharedDataPush xapp_shared_data_push = 1;

}

message XappSharedDataPushOutMsg {

}

7.3.5	Message Transfer Syntax

#### 7.3.5.1	Solution 1: SCTP with Protocol Buffers

The message transfer syntax for Solution 1 is serialized Protocol Buffers [4].

#### 7.3.5.2	Solution 2: gRPC with Protocol Buffers

The message transfer syntax for Solution 2 is serialized Protocol Buffers [4].

8	Management API

8.1	Overview

      - 8.1.1	Protocol Stack

In this version of specification, the default protocol stack for the Management APIs is shown on Figure 8.1.1-1.

Note: The security protocols are specified in [6].

Figure 8.1.1-1: Protocol stack for Management APIs

The Management APIs use gRPC [5] with Protocol Buffers [4] as the application layer serialization protocol.

      - 8.1.2	List of Management APIs, procedures and messages

The Management API procedures introduced in clause 9.4 of [2] are categorized under the Management APIs. These APIs include:

xApp Registration API: enabling registration of xApps with the Near-RT RIC platform.

These APIs are summarized in Table 8.1.2-1.

Table 8.1.2-1: List of Management APIs

| API Name | Description |
| --- | --- |
| Mgmt_xApp_Registration_API | Near-RT RIC platform service for xApp registration |

The table 8.1.2-2 lists the API procedures for each Management API.

Table 8.1.2-2: List of Management API procedures

| API Name | API procedure | Communication type |
| --- | --- | --- |
| Mgmt_xApp_Registration_API | xApp Registration | Request/Response |

The table 8.1.2-3 lists the messages for each management API procedure.

Table 8.1.2-3: List of Management API messages

| API Procedure | Initiated by | Initiating Message | Outcome Message |  |
| --- | --- | --- | --- | --- |
|  |  |  | Successful Outcome | Unsuccessful Outcome |
| xApp Registration | xApp | XAPP REGISTRATION REQUEST | XAPP REGISTRATION SUCCESS | XAPP REGISTRATION FAILURE |

8.2	Management API Procedures

      - 8.2.1	xApp Registration Procedure

        - 8.2.1.1	General

This procedure is used by an xApp who wants to register in Near-RT RIC platform.

        - 8.2.1.2	Successful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: xAPP REGISTRATION (Request)

near -> app: xAPP REGISTRATION (Success)

@enduml

Figure 8.2.1.2-1: xApp Registration procedure, successful operation

The xApp initiates the procedure by sending an xApp Registration (Request) message to the Near-RT-RIC platform. The message contains a unique Procedure Transaction ID IE assigned by the initiating xApp, the optional xApp Description IE, and the Required APIs and Data Types IE that specifies the APIs and data types needed for normal operation of the xApp.

At reception of the xApp Registration (Request) message, the Near-RT-RIC platform shall:

-	 Check xApp’s compatibility with the Near-RT RIC platform, using information in the Required APIs and Data Types IE.

-	Assign an xApp ID and create the profile for the xApp.

If the registration request is successfully fulfilled, the Near-RT RIC platform shall send an xApp Registration (Success) message back to the xApp.

In the xApp Registration (Success) message, the Near-RT RIC platform shall include the associated Procedure Transaction ID IE and the xApp ID IE.

        - 8.2.1.3	Unsuccessful Operation

@startuml

Skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam lifelineStrategy solid

participant "xApp" as app

participant "Near-RT RIC Platform" as near

app -> near: xAPP REGISTRATION (Request)

near -> app: xAPP REGISTRATION (Failure)

@enduml

Figure 8.2.1.3-1: xApp Registration procedure, unsuccessful operation

If the registration request fails, the Near-RT RIC platform shall send an xApp Registration (Failure) message back to the xApp. The message shall include the associated Procedure Transaction ID IE replicated from the corresponding request and appropriate cause.

        - 8.2.1.4	Abnormal Conditions

Not applicable.

8.3	Elements for Management API Communication

8.3.1	General

8.3.2	Message Functional Definition and Content

8.3.2.1	xAPP REGISTRATION REQUEST

This message is sent by an xApp to the Near-RT RIC platform for xApp Registration.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 8.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value assigned by the initiating xApp. | YES | Reject |
| xApp Description | O |  | 8.3.3.2 |  | YES | Ignore |
| Required APIs and Data Types | M |  |  |  | YES | Reject |
| > List of Required APIs |  | 0.. <maxnoofAPIs> |  | The API names and versions required by the xApp | - | - |
| >> API Name | M |  | 10.1.3 |  | - | - |
| >> API Version | M |  | 10.1.4 |  | - | - |
| > List of A1 Policy Types |  | 0.. <maxnoofA1PolicyTypes> |  | The A1 Policy Types required by the xApp | - | - |
| >> A1 Policy Type ID | M |  | 8.3.3.3 |  | - | - |
| > List of E2 Service Models |  | 0.. <maxnoofE2SMs> |  | The E2 Service Models required by the xApp | - | - |
| >> E2 Service Model Name | M |  | 8.3.3.4 |  | - | - |
| >> E2 Service Model Version | M |  | 8.3.3.5 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIs | Maximum no. of APIs supported by the xApp. Value is 1024. |
| maxnoofA1PolicyTypes | Maximum no. of A1 Policy Types supported by the xApp. Value is 1024. |
| maxnoofE2SMs | Maximum no. of E2SMs supported by the xApp. Value is 1024. |

8.3.2.2	xAPP REGISTRATION SUCCESS

This message is sent by the Near-RT RIC platform after the xApp’s registration request is successfully accepted.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 8.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| xApp ID | M |  | 10.1.2 |  | YES | Reject |

8.3.2.3	xAPP REGISTRATION FAILURE

This message is sent by the Near-RT RIC platform after the xApp’s registration request is failed.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 8.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 | Value replicated from corresponding request. | YES | Reject |
| Cause | M |  | 8.3.3.6 |  | YES | Reject |

8.3.2.4	xAPP CONFIGURATION REQUEST

8.3.2.5	xAPP CONFIGURATION RESPONSE

8.3.2.6	xAPP CONFIGURATION FAILURE

8.3.3	Information Element Definitions

#### 8.3.3.1	Message Type

This IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Type | M |  | CHOICE (xApp Registration, …) |  |
| >Type of Message | M |  | CHOICE (Request, Success, Failure, …) | Refer to Table 8.1.2-3 for applicable choices for each procedure type |

#### 8.3.3.2	xApp Description

This IE provides generic information related to the xApp such as its name, vendor and purpose.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| xApp Description | M |  | STRING |  |

#### 8.3.3.3	A1 Policy Type ID

Defined in 10.1.5.

#### 8.3.3.4	E2 Service Model Name

This IE specifies the name of E2 Service Model supported by the xApp.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Service Model Name | M |  | STRING | The E2SM short name as in Table 5-1 of [8] |

#### 8.3.3.5	E2 Service Model Version

This IE is used to indicate the version of E2 Service Model supported by the xApp.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E2 Service Model Version | M |  | STRING |  |

#### 8.3.3.6	Cause

This IE is used to indicate the reason for a particular event for the Management APIs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | ENUMERATED (Unspecified, API Not Supported, A1 Policy Type Not Supported, E2 Service Model Not Supported, …) |  |

8.3.4	Message and Information Element Abstract Syntax

8.3.4.1	Solution 1: gRPC with Protocol Buffers

8.3.4.1.1	Usage of gRPC with Protocol Buffers

This solution uses Protocol Buffers as message encoding, and also as the Interface Definition Language (IDL) for gRPC [5].

Table 8.3.4.1.1-1 lists the mapping of management APIs and corresponding gRPC services.

Table 8.3.4.1.1-1: Mapping of management APIs and gRPC services

| API Name | gRPC service(s) |
| --- | --- |
| Mgmt_Xapp_Registration_API | Mgmt_Xapp_Registration_API |

A procedure of an Management API is mapped to a gRPC method.

Table 8.3.4.1.1-2 lists the Management APIs abstract syntax included in this document.

Table 8.3.4.1.1-2: List of Management APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| Mgmt_Xapp_Registration_API | 8.3.4.1.2.1 | 1.0.0 | mgmt_xapp_registration_api_ver_1_0_0.proto |
| Management API common information elements | 8.3.4.1.3 | 1.0.0 | mgmt_apis_common_ies_ver_1_0_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

8.3.4.1.2	API Definitions

8.3.4.1.2.1	Mgmt_Xapp_Registration_API

syntax = "proto3";

package ricapi.mgmt_xapp_registration_api.v1;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "mgmt_apis_common_ies_ver_1_0_0.proto";

// gRPC service and rpc definition

service mgmt_Xapp_Registration_API {

rpc XappRegistrationProc(XappRegistrationInitMsg) returns (XappRegistrationOutMsg) {};

}

message XappRegistrationInitMsg {

optional XappRegistrationRequest xapp_registration_request = 1;

}

message XappRegistrationOutMsg {

oneof type_of_message {

XappRegistrationSuccess xapp_registration_success = 1;

XappRegistrationFailure xapp_registration_failure = 2;

}

}

// ---------------- messages of Mgmt_Xapp_Registration_API ----------------

// -------- messages of xApp Registration procedure --------

message XappRegistrationRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional mgmt_apis_common_ies.v1.XappDescription xapp_description = 2;

optional RequiredApisAndDataTypes required_apis_and_data_types = 3;

message RequiredApisAndDataTypes {

repeated RequiredApiItem list_of_required_apis = 1;

repeated A1PolicyTypeItem list_of_a1_policy_types = 2;

repeated E2ServiceModelItem list_of_e2_service_models = 3;

message RequiredApiItem {

optional near_rt_ric_apis_common_ies.v2.ApiName api_name = 1;

optional near_rt_ric_apis_common_ies.v2.ApiVersion api_version = 2;

}

message A1PolicyTypeItem {

optional near_rt_ric_apis_common_ies.v2.A1PolicyTypeId a1_policy_type_id = 1;

}

message E2ServiceModelItem {

optional mgmt_apis_common_ies.v1.E2ServiceModelName e2_service_model_name = 1;

optional mgmt_apis_common_ies.v1.E2ServiceModelVersion e2_service_model_version = 2;

}

}

}

message XappRegistrationSuccess {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 2;

}

message XappRegistrationFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

optional mgmt_apis_common_ies.v1.Cause cause = 2;

}

8.3.4.1.3	Information Element Definitions

syntax = "proto3";

package ricapi.mgmt_apis_common_ies.v1;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

// ----------------------------- Information Elements -----------------------------

// --------------C--------------

enum Cause {

UNSPECIFIED = 0;

API_NOT_SUPPORTED = 1;

A1_POLICY_TYPE_NOT_SUPPORTED = 2;

E2_SERVICE_MODEL_NOT_SUPPORTED = 3;

}

// --------------E--------------

message E2ServiceModelName {

optional string value = 1;

}

message E2ServiceModelVersion {

optional string value = 1;

}

// --------------X--------------

message XappDescription {

optional string value = 1;

}

8.3.5	Message Transfer Syntax

8.3.5.1	Solution 1: gRPC with Protocol Buffers

The message transfer syntax for Solution 1 is serialized Protocol Buffers [4].

9	Enablement API

9.1	Overview

9.1.1	Protocol Stack

In this version of specification, the default protocol stack for the Enablement APIs is shown on Figure 9.1.1-1.

Note: The security protocols are specified in [6].

Figure 9.1.1-1: Protocol stack for Enablement APIs

The Enablement APIs use gRPC [5] with Protocol Buffers [4] as the application layer serialization protocol.

9.1.2	List of Enablement APIs, procedures and messages

The enablement API procedures introduced in clause 9.5 of [2] are categorized under the enablement APIs. These APIs include:

Discovery API: enabling discovery of the Near-RT RIC APIs by xApps.

Events API: enabling subscription/un-subscription to API-related events and event notifications.

Registration API: enabling registration of the Near-RT RIC APIs by xApps.

These APIs are summarized in Table 9.1.2-1.

Table 9.1.2-1: List of Enablement APIs

| API Name | Description |
| --- | --- |
| Enabl_Discovery_API | Near-RT RIC platform service for API discovery |
| Enabl_Events_API | Near-RT RIC platform service for API related events |
| Enabl_Registration_API | Near-RT RIC platform service for API registration |

The table 9.1.2-2 lists the API procedures for each enablement API.

Table 9.1.2-2: List of enablement API procedures

| API Name | API procedure | Communication type |
| --- | --- | --- |
| Enabl_Discovery_API | API Discovery | Request/Response |
| Enabl_Events_API | API Event Subcription | Request/Response |
|  | API Event Subcription Delete | Request/Response |
|  | API Event Notification | Notify |
| Enabl_Registration_API | API Registration | Request/Response |
|  | API Registration Update | Request/Response |
|  | API Deregistration | Request/Response |

The table 9.1.2-3 lists the messages for each enablement API procedure.

Table 9.1.2-3: List of enablement API messages

| API Procedure | Initiated by | Initiating Message | Outcome Message |  |
| --- | --- | --- | --- | --- |
|  |  |  | Successful Outcome | Unsuccessful Outcome |
| API Discovery | xApp | API DISCOVERY REQUEST | API DISCOVERY RESPONSE | API DISCOVERY FAILURE |
| API Event Subscription | xApp | API Event Subscription REQUEST | API Event Subscription RESPONSE | API Event Subscription FAILURE |
| API Event Subscription Delete | xApp | API Event Subscription DELETE REQUEST | API Event Subscription DELETE RESPONSE | API Event Subscription DELETE FAILURE |
| API Event Notification | Near-RT RIC platform | API Event NOTIFICATION | API Event NOTIFICATION ACK | - |
| API Registration | xApp | API REGISTRATION REQUEST | api REGISTRATION RESPONSE | API REGISTRATION FAILURE |
| API Registration Update | xApp | API REGISTRATION UPDATE REQUEST | API REGISTRATION UPDATE RESPONSE | API REGISTRATION UPDATE FAILURE |
| API Deregistration | xApp | API DEREGISTRATION REQUEST | API DEREGISTRATION RESPONSE | API DEREGISTRATION FAILURE |

9.2	Enablement API Procedures

9.2.1	Enabl_Discovery_API Procedures

9.2.1.1	General

The Enabl_Discovery_API allows an xApp to discover service APIs available at the Near-RT RIC platform.

The API procedure defined for Enabl_Discovery_API is shown in table 9.2.1.1-1.

Table 9.2.1.1-1: Procedure of the Enabl_Discovery_API

| API Procedure | Description | Initiated by |
| --- | --- | --- |
| API Discovery | This procedure is used by an xApp to discover Near-RT RIC APIs available at the Near-RT RIC platform. | xApp |

9.2.1.2	API Discovery procedure

9.2.1.2.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API DISCOVERY REQUEST

RICP -> xApp  : 2. API DISCOVERY RESPONSE

@enduml

Figure 9.2.1.2.1-1: API Discovery procedure, successful operation

To discover Near-RT RIC APIs available at the API Enablement function, the xApp shall send an API DISCOVERY REQUEST message with the xApp ID IE and the Query Parameters IE to the Near-RT RIC platform.

Upon receiving the message, the Near-RT RIC platform shall:

1.	verify the identity of the xApp and check if it is authorized to discover the Near-RT RIC APIs;

2.	if the xApp is authorized to discover the Near-RT RIC APIs, the Near-RT RIC platform shall:

a.	search the API registry for APIs matching the query criteria in the Query Parameters IE;

b.	apply the discovery policy, if any, on the search results and filter the search results to obtain the list of service API description;

c.	return the filtered search results in the API DISCOVERY RESPONSE message.

9.2.1.2.2 	Unsuccessful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API DISCOVERY REQUEST

RICP -> xApp  : 2. API DISCOVERY FAILURE

@enduml

Figure 9.2.1.2.2-1: API Discovery procedure, unsuccessful operation

If the Near-RT RIC platform rejects the request it shall respond with the API DISCOVERY FAILURE message with a proper cause. If the message contains the Time to Wait IE the xApp shall wait at least the indicated time before reinitiating the request to the Near-RT RIC platform.

9.2.1.2.3	Abnormal Conditions

Not applicable.

9.2.2	Enabl_Events_API Procedures

9.2.2.1	General

The Enabl_Events_API allows an xApp to subscribe to and unsubscribe from events related to Near-RT RIC APIs and to receive notifications of the events.

Such events includes:

Availability of Near-RT RIC API: event related to availability of Near-RT RIC APIs (e.g. active, inactive)

Update of Near-RT RIC API: event related to change in Near-RT RIC API information

The API procedures defined for the Enabl_Events_API are shown in table 9.2.2.1-1.

Table 9.2.2.1-1: Operations of the Enabl_Events_API

| API Procedure | Description | Initiated by |
| --- | --- | --- |
| API Event Subscription | This procedure is used by xApp to subscribe for receiving Near-RT RIC API related events. | xApp |
| API Event Subscription Delete | This procedure is used by xApp to unsubscribe from receiving Near-RT RIC API related events. | xApp |
| API Event Notification | This procedure is used by Near-RT RIC to send a notification of Near-RT RIC API related event to an xApp. | Near-RT RIC platform |

9.2.2.2	API Event Subscription procedure

9.2.2.2.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API EVENT SUBSCRIPTION REQUEST

RICP -> xApp  : 2. API EVENT SUBSCRIPTION RESPONSE

@enduml

Figure 9.2.2.2.1-1: API Event Subscription procedure, successful operation

To subscribe to Near-RT RIC API related events, the xApp shall send an API EVENT SUBSCRIPTION REQUEST message to the Near-RT RIC platform. The message shall include the xApp ID IE, the List of Events IE, the Event Notification Requirements IE and the Notification Destination IE.

The xApp may also include additional event filter(s) in the List of Event Filters IE. Such event filters include:

-	API ID(s) in the API ID Filter IE, if the event type is NEAR_RT_RIC_API_AVAILABLE, NEAR_RT_RIC_API_UNAVAILABLE or NEAR_RT_RIC_API_UPDATE;

Upon receiving the message, the API Enablement function shall:

1.	verify the identity of the xApp and check if the xApp is authorized to subscribe to the Near-RT RIC API related events specified in the request message;

2.	if the xApp is authorized to subscribe to the Near-RT RIC API related events, the Near-RT RIC platform shall prepare necessary resource for the subscription and associated notifications, according to the contents in the List of Events IE and the Event Notification Requirements IE, and send an API EVENT SUBSCRIPTION RESPONSE message back to the xApp. The response message shall include the assigned subscription ID in the Subscription ID IE.

9.2.2.2.2	Unsuccessful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP : 1. API EVENT SUBSCRIPTION REQUEST

RICP -> xApp  : 2. API EVENT SUBSCRIPTION FAILURE

@enduml

Figure 9.2.2.2.2-1: API Event Subscription procedure, unsuccessful operation

If the Near-RT RIC platform rejects the request it shall respond with the API EVENT SUBSCRIPTION FAILURE message with proper cause. If the message contains Time to Wait IE the xApp shall wait at least the indicated time before reinitiating the request to the Near-RT RIC platform.

9.2.2.2.3	Abnormal conditions

If the Near-RT RIC platform receives an API EVENT SUBSCRIPTION REQUEST message containing Event Type  IE that is not supported by the Near-RT RIC platform, or the optional filter definition is invalid, the Near-RT RIC platform shall send the API EVENT SUBSCRIPTION FAILURE message to the xApp with an appropriate cause value.

9.2.2.3	API Event Subscription Delete procedure

9.2.2.3.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP : 1. API EVENT SUBSCRIPTION DELETE REQUEST

RICP -> xApp  : 2. API EVENT SUBSCRIPTION DELETE RESPONSE

@enduml

Figure 9.2.2.3.1-1: API Event Subscription Delete procedure, successful operation

To unsubscribe from Near-RT RIC API related events, the xApp shall send an API EVENT SUBSCRIPTION DELETE REQUEST message to the Near-RT RIC platform with the xApp ID IE and the Subscription ID IE.

Upon receiving the message, the Near-RT RIC platform shall:

1.	verify the identity of the xApp and check if the xApp is authorized to unsubscribe from the Near-RT RIC API related events associated with the subscription ID; and

2.	if the xApp is authorized to unsubscribe from the Near-RT RIC API related events, the Near-RT RIC platform shall delete the resource for the subscription and reply API EVENT SUBSCRIPTION DELETE RESPONSE to the xApp.

9.2.2.3.2	Unsuccessful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP : 1. API EVENT SUBSCRIPTION DELETE REQUEST

RICP -> xApp  : 2. API EVENT SUBSCRIPTION DELETE FAILURE

@enduml

Figure 9.2.2.3.2-1: API Event Subscription Delete procedure, unsuccessful operation

If the Near-RT RIC platform rejects the request it shall respond with the API EVENT SUBSCRIPTION DELETE FAILURE message with proper cause. If the message contains Time To Wait IE the xApp shall wait at least the indicated time before reinitiating the request to the Near-RT RIC platform.

9.2.2.3.3	Abnormal conditions

Not applicable.

9.2.2.4	API Event Notification procedure

9.2.2.4.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

RICP -> xApp   : 1. API EVENT NOTIFICATION

RICP <-- xApp  : 2. API EVENT NOTIFICATION ACK

@enduml

Figure 9.2.2.4.1-1: API Event Notification procedure, successful operation

To notify a Near-RT RIC API related event, the Near-RT RIC platform shall send an API EVENT NOTIFICATION  message using the Notification Destination IE received in the API Event Subscription procedure.

The Near-RT RIC platform may include details of the event in the Event Detail IE. Such details include:

-	if the event type is Near-RT_RIC_API_AVAILABLE or Near-RT_RIC_API_UNAVAILABLE, the API IDs in the List of API IDs IE;

-	if the event is Near-RT_RIC_API_UPDATE, the updated API information in the List of API Information IE;

Upon receiving the message, the xApp may reply an API EVENT NOTIFICATION ACK message.

9.2.2.4.2	Unsuccessful Operation

Not applicable.

9.2.2.4.3	Abnormal conditions

Not applicable.

### 9.2.3	Enabl_Registration_API Procedures

#### 9.2.3.1	General

The Enabl_Registration_API allows an xApp to register its service APIs with the Near-RT RIC platform.

The API procedure defined for Enabl_Registration_API is shown in table 9.2.3.1-1.

Table 9.2.3.1-1: Procedure of the Enabl_Discovery_API

| API Procedure | Description | Initiated by |
| --- | --- | --- |
| API Registration | This procedure is used by an xApp to register its Near-RT RIC APIs. | xApp |
| API Registration Update | This procedure is used by an xApp to update its registered Near-RT RIC APIs. | xApp |
| API Deregistration | This procedure is used by an xApp to deregiter its registered Near-RT RIC APIs. | xApp |

#### 9.2.3.2	API Registration procedure

##### 9.2.3.2.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API REGISTRATION REQUEST

RICP -> xApp  : 2. API REGISTRATION RESPONSE

@enduml

Figure 9.2.3.2.1-1: API Registration procedure, successful operation

To register Near-RT RIC APIs with the Near-RT RIC platform, the xApp shall send an API REGISTRATION REQUEST message. The message shall include:

-	the List of API Information to Register IE, which contain the API information of the Near-RT RIC APIs to be registered.

Upon receiving the message, the Near-RT RIC platform shall verify the authorization of the xApp, assign an API ID for each successfully registered API, and store the associated API information. The Near-RT RIC platform shall reply with an API REGISTRATION RESPONSE message. The message shall include:

-	the List of API Information Successfully Registered IE, which contains the API information of the successfully registered APIs;

-	the List of API Information Failed To Register IE if any API is not successfully registered, which contains the corresponding API information and cause.

##### 9.2.3.2.2	Unsuccessful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API REGISTRATION REQUEST

RICP -> xApp  : 2. API REGISTRATION FAILURE

@enduml

Figure 9.2.3.2.2-1: API Registration procedure, unsuccessful operation

If the Near-RT RIC platform rejects the request it shall respond with the API REGISTRATION FAILURE message. The message shall include the List of API Information Failed To Register IE, which contains the corresponding API information and cause.

##### 9.2.3.2.3	Abnormal Conditions

Not applicable.

#### 9.2.3.3	API Registration Update procedure

##### 9.2.3.3.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API REGISTRATION UPDATE REQUEST

RICP -> xApp  : 2. API REGISTRATION UPDATE RESPONSE

@enduml

Figure 9.2.3.3.1-1: API Registration Update procedure, successful operation

To update its Near-RT RIC APIs that have been registered at the Near-RT RIC platform, the xApp shall send an API REGISTRATION UPDATE REQUEST message. The message shall include:

-	the List of API Information to Update IE, which contain the API information of the Near-RT RIC APIs to be updated.

Upon receiving the message, the Near-RT RIC platform shall verify the authorization of the xApp, and update the stored API information with the new API information associated with the same API ID. The Near-RT RIC platform shall reply with an API REGISTRATION UPDATE RESPONSE message. The message shall include:

-	the List of API IDs Successfully Updated IE, which contains the API IDs of the successfully updated APIs;

-	the List of API IDs Failed To Update IE if any API is not successfully updated, which contains the corresponding API ID and cause.

##### 9.2.3.3.2	Unsuccessful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API REGISTRATION UPDATE REQUEST

RICP -> xApp  : 2. API REGISTRATION UPDATE FAILURE

@enduml

Figure 9.2.3.3.2-1: API Registration Update procedure, unsuccessful operation

If the Near-RT RIC platform rejects the request it shall respond with the API REGISTRATION UPDATE FAILURE message. The message shall include the List of API IDs Failed To Update IE, which contains the API IDs that are not updated and the corresponding cause.

##### 9.2.3.3.3	Abnormal Conditions

Not applicable.

#### 9.2.3.4	API Deregistration procedure

##### 9.2.3.4.1	Successful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API DEREGISTRATION REQUEST

RICP -> xApp  : 2. API DEREGISTRATION RESPONSE

@enduml

Figure 9.2.3.4.1-1: API Deregistration procedure, successful operation

To deregister its Near-RT RIC APIs from the Near-RT RIC platform, the xApp shall send an API DEREGISTRATION REQUEST message. The message shall include the List of API IDs To Deregister IE which indicates the API IDs of the APIs to be deregistered.

Upon receiving the message, the Near-RT RIC platform shall verify the authorization of the xApp, and delete the stored API information associated with the API IDs in the request. The Near-RT RIC platform shall reply with an API DEREGISTRATION RESPONSE message. The message shall include:

-	the List of API IDs Successfully Deregistered IE, which contains the API IDs of the successfully deregistered APIs;

-	the List of API IDs Failed To Deregister IE if any API is not successfully deregistered, which contains the corresponding API IDs and cause.

##### 9.2.3.4.2	Unsuccessful Operation

@startuml

skin rose

skinparam ParticipantPadding 50

skinparam BoxPadding 10

skinparam defaultFontSize 12

skinparam lifelineStrategy solid

Participant xApp as "xApp"

Participant RICP as “Near-RT RIC Platform”

xApp -> RICP  : 1. API DEREGISTRATION REQUEST

RICP -> xApp  : 2. API DEREGISTRATION FAILURE

@enduml

Figure 9.2.3.4.2-1: API Deregistration procedure, unsuccessful operation

If the Near-RT RIC platform rejects the request it shall respond with the API DEREGISTRATION FAILURE message. The message shall include the List of API IDs Failed To Deregister IE, which contains the API IDs that are not deregistered and the corresponding cause.

##### 9.2.3.4.3	Abnormal Conditions

Not applicable.

9.3	Elements for Enablement API Communication

9.3.1	General

9.3.2	Message Functional Definition and Content

9.3.2.1	API DISCOVERY REQUEST

This message is sent by an xApp to the Near-RT RIC platform to discover the available Near-RT RIC APIs.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Query Parameters |  | 0..1 |  |  |  |  |
| >API Name | O |  | 9.3.3.6 |  |  |  |
| >API Version | O |  | 9.3.3.7 |  |  |  |
| >Protocol | O |  | 9.3.3.8 |  |  |  |
| >Data Format | O |  | 9.3.3.9 |  |  |  |
| >API Category | O |  | 9.3.3.10 |  |  |  |

9.3.2.2	API DISCOVERY RESPONSE

This message is sent by Near-RT RIC platform to the xApp to carry the information of discovered Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| List of API Information |  | 0.. <maxnoofAPIInformation> |  | Description of the Near-RT RIC APIs as published by the provider(s). |  |  |
| >API Information | M |  | 9.3.3.2 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIInformation | Maximum no. of API information items supported in an API list. Value is 1024. |

9.3.2.3	API DISCOVERY FAILURE

This message is sent by Near-RT RIC platform to the xApp for the case of failure of the discovery of the available near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Cause | M |  | 9.3.3.4 |  |  |  |
| Time to Wait | O |  | 9.3.3.5 |  |  |  |

9.3.2.4	API EVENT SUBSCRIPTION REQUEST

This message is sent by an xApp to the Near-RT RIC platform to subscribe for events related to Near-RT RIC APIs.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| List of Events |  | 1.. <maxnoofAPIEventFilter> |  |  |  |  |
| >Event Type | M |  | 9.3.3.13 |  |  |  |
| >API ID Filter |  | 0.. <maxnoofAPIIDFiliters> |  | API identifiers that the event subscriber wants to know in the interested event. |  |  |
| >>API ID | M |  | 9.3.3.11 |  |  |  |
| Event Notification Requirements |  | 1 |  | Represents the reporting requirements of the event subscription. |  |  |
| >Notification Method | M |  | ENUMERATED (On Event, Periodic, ...) |  |  |  |
| >Notification Period | O |  | INTEGER (1.. 300, ...) | The duration between notifications expressed in unit of second. Shall be present when the Notification Method IE is set to " Periodic". |  |  |
| Notification Destination | M |  | 10.1.9 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIEventFilter | Maximum no. of API event filters supported in the list. Value is 64. |
| maxnoofAPIIDFiliters | Maximum no. of API ID filters supported in the list. Value is 1024. |

9.3.2.5	API EVENT SUBSCRIPTION RESPONSE

This message is sent by Near-RT RIC platform to the xApp as successful response to API event subscription request.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Subscription ID | M |  | 9.3.3.3 |  |  |  |

9.3.2.6	API EVENT SUBSCRIPTION FAILURE

This message is sent by Near-RT RIC platform to the xApp for the case of failure of the API event subscription request.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Cause | M |  | 9.3.3.4 |  |  |  |
| Time to Wait | O |  | 9.3.3.5 |  |  |  |

9.3.2.7	API EVENT SUBSCRIPTION DELETE REQUEST

This message is sent by an xApp to the Near-RT RIC platform to unsubscribe for events related to Near-RT RIC APIs.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Subscription ID | M |  | 9.3.3.3 |  |  |  |

9.3.2.8	API EVENT SUBSCRIPTION DELETE RESPONSE

This message is sent by Near-RT RIC platform to the xApp as successful response to API event subscription delete request.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Subscription ID | M |  | 9.3.3.3 |  |  |  |

9.3.2.9	API EVENT SUBSCRIPTION DELETE FAILURE

This message is sent by Near-RT RIC platform to the xApp for the case of failure of the API event subscription delete request.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| xApp ID | M |  | 10.1.2 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Cause | M |  | 9.3.3.4 |  |  |  |
| Time to Wait | O |  | 9.3.3.5 |  |  |  |

9.3.2.10	API EVENT NOTIFICATION

This message is sent to a subscriber xApp from the Near-RT RIC platform to notify on an API event (based on subscription).

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| Subscription ID | M |  | 9.3.3.4 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |
| Event Type | M |  | 9.3.3.13 |  |  |  |
| Event Detail |  | 0.. 1 |  |  |  |  |
| >List of API IDs |  | 0.. <maxnoofAPIIDs> |  | The API IDs which are available/unavailable in the relevant event. |  |  |
| >>API ID | M |  | 9.3.3.11 |  |  |  |
| >List of API Information |  | 0.. <maxnoofAPIInformation> |  | Description of the Near-RT RIC APIs which are to be updated. |  |  |
| >>API Information | M |  | 9.3.3.2 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIIDs | Maximum no. of API IDs supported in an API list. Value is 1024. |
| maxnoofAPIInformation | Maximum no. of API information items supported in an API list. Value is 1024. |

9.3.2.11	API EVENT NOTIFICATION ACK

This message is optionally sent by the xApp to Near-RT RIC platform as acknowledgement to an API event notification.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  |  |  |
| Subscription ID | M |  | 9.3.3.3 |  |  |  |
| Procedure Transaction ID | M |  | 10.1.1 |  |  |  |

#### 9.3.2.12	API REGISTRATION REQUEST

This message is sent by an xApp to the Near-RT RIC platform to register its Near-RT RIC APIs.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API Information To Register |  | 1..<maxnoofAPIInformation> |  |  | YES | Reject |
| > API Information | M |  | 9.3.3.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIInformation | Maximum no. of API information items supported in an API list. Value is 1024. |

#### 9.3.2.13	API REGISTRATION RESPONSE

This message is sent by Near-RT RIC platform to the xApp to indicate success of the registration of Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API Information Successfully Registered |  | 1..<maxnoofAPIInformation> |  | The list of successfully registered APIs | YES | Reject |
| >API Information | M |  | 9.3.3.2 |  | - | - |
| List of API Information Failed To Register |  | 0..<maxnoofAPIInformation> |  | The list of APIs that are not successfully registered | YES | Reject |
| >API Information | M |  | 9.3.3.2 |  | - | - |
| >Cause | M |  | 9.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIInformation | Maximum no. of API information items supported in an API list. Value is 1024. |

#### 9.3.2.14	API REGISTRATION FAILURE

This message is sent by Near-RT RIC platform to the xApp to indicate failure of the registration of Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API Information Failed To Register |  | 1..<maxnoofAPIInformation> |  | The list of APIs that are not successfully registered. | YES | Reject |
| > API Information | M |  | 9.3.3.2 |  | - | - |
| > Cause | M |  | 9.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIInformation | Maximum no. of API information items supported in an API list. Value is 1024. |

#### 9.3.2.15	API REGISTRATION UPDATE REQUEST

This message is sent by an xApp to the Near-RT RIC platform to update its registered Near-RT RIC APIs.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API Information To Update |  | 1..<maxnoofAPIInformation> |  |  | YES | Reject |
| > API Information | M |  | 9.3.3.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIInformation | Maximum no. of API information items supported in an API list. Value is 1024. |

#### 9.3.2.16	API REGISTRATION UPDATE RESPONSE

This message is sent by Near-RT RIC platform to the xApp to indicate success of the registration of Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API IDs Successfully Updated |  | 1..<maxnoofAPIIDs> |  | The list of successfully updated APIs | YES | Reject |
| >API ID | M |  | 9.3.3.11 |  | - | - |
| List of API IDs Failed To Update |  | 0..<maxnoofAPIIDs> |  | The list of APIs that are not successfully updated. | YES | Reject |
| >API ID | M |  | 9.3.3.11 |  | - | - |
| >Cause | M |  | 9.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIIDs | Maximum no. of API IDs supported in an API list. Value is 1024. |

#### 9.3.2.17	API REGISTRATION UPDATE FAILURE

This message is sent by Near-RT RIC platform to the xApp to indicate failure of the registration of Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API IDs Failed to Update |  | 1..<maxnoofAPIIDs> |  | The list of APIs that are not successfully updated. | YES | Reject |
| > API ID | M |  | 9.3.3.11 |  | - | - |
| > Cause | M |  | 9.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIIDs | Maximum no. of API IDs supported in an API list. Value is 1024. |

#### 9.3.2.18	API DEREGISTRATION REQUEST

This message is sent by an xApp to the Near-RT RIC platform to deregister its Near-RT RIC APIs.

Direction: xApp -> Near-RT RIC platform

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API IDs To Deregister |  | 1..<maxnoofAPIIDs> |  |  | YES | Reject |
| > API ID | M |  | 9.3.3.11 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIIDs | Maximum no. of API IDs supported in an API list. Value is 1024. |

#### 9.3.2.19	API DEREGISTRATION RESPONSE

This message is sent by Near-RT RIC platform to the xApp to indicate success of the deregistration of Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API IDs Successfully Deregistered |  | 1..<maxnoofAPIIDs> |  | The list of successfully deregistered APIs | YES | Reject |
| >API ID | M |  | 9.3.3.11 |  | - | - |
| List of API IDs Failed To Deregister |  | 0..<maxnoofAPIIDs> |  | The list of APIs that are not successfully deregistered | YES | Reject |
| >API ID | M |  | 9.3.3.11 |  | - | - |
| >Cause | M |  | 9.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIIDs | Maximum no. of API IDs supported in an API list. Value is 1024. |

#### 9.3.2.20	API DEREGISTRATION FAILURE

This message is sent by Near-RT RIC platform to the xApp to indicate failure of the deregistration of Near-RT RIC APIs.

Direction: Near-RT RIC platform -> xApp

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.3.1 |  | YES | Reject |
| Procedure Transaction ID | M |  | 10.1.1 |  | YES | Reject |
| List of API IDs Failed To Deregister |  | 1..<maxnoofAPIInformation> |  | The list of APIs that are not successfully deregistered. | YES | Reject |
| > API ID | M |  | 9.3.3.11 |  | - | - |
| > Cause | M |  | 9.3.3.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofAPIIDs | Maximum no. of API IDs supported in an API list. Value is 1024. |

9.3.3	Information Element Definitions

9.3.3.1	Message Type

This IE identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Type | M |  | CHOICE (API Discovery, API Event Subscription, API Event Subscription Delete, API Event Notification, API Registration, API Registration Update, API Deregistration, …) |  |
| >Type of Message | M |  | CHOICE (Request, Response, Failure, …) | Refer to Table 9.1.2-3 for applicable choices for each procedure type |

9.3.3.2	API Information

This IE indicates the information of a Near-RT RIC API.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| API Name | M |  | 9.3.3.6 |  |
| API ID | C |  | 9.3.3.11 | Assigned by the Near-RT RIC platform. Shall be present when the API Information is associated with a registered API, or used to update a registered API. |
| API Category | M |  | 9.3.3.10 |  |
| Description | O |  | STRING | Text description of the API |
| List of API Profiles |  | 1.. <maxnoofAPIProfiles> |  |  |
| >API Version | M |  | 9.3.3.7 |  |
| >Protocol | O |  | 9.3.3.8 |  |
| >Data Format | O |  | 9.3.3.9 |  |
| >Endpoint Information | O |  | 10.1.9 |  |
| >List of Supported Data Types |  | 0..<maxnoofSupportedDataTypes> |  | Shall be present for SDL APIs. |
| >> Data Type ID | M |  | STRING | For the API information of SDL APIs, this IE is populated with the SDL Data Type ID specified in clause 7.3.3.21. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSupportedDataTypes | Maximum no. of supported data types in an API profile. Value is 1024. |

9.3.3.3	Subscription ID

The IE uniquely identifies a subscription to API-related event(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subscription ID | M |  | STRING | Identifier of the subscription resource to which the notification is related |

9.3.3.4	Cause

This IE is used to indicate the reason for a particular event for the Enablement APIs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | ENUMERATED (Unspecified, System busy, Duplicate API Information, Unknown API ID, Invalid Data Type, Unknown Subscription ID,...) |  |

The meaning of the different cause values is described in the following table.

| Cause value | Meaning |
| --- | --- |
| Unspecified | Sent when none of the below cause values applies. |
| System busy | The system cannot process the request due to temporary overload. |
| Duplicate API Information | The same API information has been registered. |
| Unknown API ID | The API ID is unknown. |
| Invalid Data Type | The supported data type in the API information is invalid. |
| Unknown Subscription ID | The Subscription ID for API event is unknown. |

9.3.3.5	Time to Wait

This IE specifies the time that xApp shall wait before reinitiating a request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time to Wait | M |  | INTEGER (1.. 300, ...) | Unit in second |

9.3.3.6	API Name

Defined in 10.1.3.

9.3.3.7	API Version

Defined in 10.1.4.

9.3.3.8	Protocol

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Protocol | M |  | ENUMERATED (gRPC, SCTP, HTTP REST, …) | Protocol used by the Near-RT RIC API |

9.3.3.9	Data Format

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Format | M |  | ENUMERATED (Proto3, JSON, ASN.1, …) | Data format used by the Near-RT RIC API |

9.3.3.10	API Category

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| API Category | M |  | ENUMERATED (E2 related APIs, SDL APIs, A1 related APIs, …) | The Near-RT RIC API category to which the API belongs |

9.3.3.11	API ID

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| API ID | M |  | STRING | API identifier assigned by the Near-RT RIC platform to the published Near-RT RIC API |

9.3.3.12	Void

9.3.3.13	Event Type

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (NEAR_RT_RIC_API_AVAILABLE, NEAR_RT_RIC_API_UNAVAILABLE, NEAR_RT_RIC_API_API_UPDATE, ...) |  |

Table 9.3.3.13-1: Meaning of Event Types

| value | Description |
| --- | --- |
| NEAR_RT_RIC_API_AVAILABLE | Events related to the availability of APIs after the APIs are published. |
| NEAR_RT_RIC_API_UNAVAILABLE | Events related to the unavailability of APIs after the APIs are unpublished. |
| NEAR_RT_RIC_API_API_UPDATE | Events related to change in API information |

9.3.4	Message and Information Element Abstract Syntax

9.3.4.1	Solution 1: gRPC with Protocol Buffers

9.3.4.1.1	Usage of gRPC with Protocol Buffers

This solution uses Protocol Buffers as message encoding, and also as the Interface Definition Language (IDL) for gRPC [5].

Table 9.3.4.1.1-1 lists the mapping of enablement APIs and corresponding gRPC services.

Table 9.3.4.1.1-1: Mapping of enablement APIs and gRPC services

| API Name | gRPC service(s) |
| --- | --- |
| Enabl_Discovery_API | Enabl_Discovery_API |
| Enabl_Events_API | Enabl_Events_API, Enabl_Events_API_Notification |
| Enabl_Registration_API | Enabl_Registration_API |

A procedure of an Enablement API is mapped to a gRPC method.

Table 9.3.4.1.1-2 lists the Enablement APIs abstract syntax included in this document.

Table 9.3.4.1.1-2: List of Enbalement APIs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| Enabl_Discovery_API | 9.3.4.1.2.1 | 2.0.0 | enabl_discovery_api_ver_2_0_0.proto |
| Enabl_Events_API | 9.3.4.1.2.2 | 2.0.0 | enabl_events_api_ver_2_0_0.proto |
| Enabl_Registration_API | 9.3.4.1.2.3 | 1.0.0 | enabl_registration_api_ver_1_0_0.proto |
| Enablement API common information elements | 9.3.4.1.3 | 2.0.0 | enabl_apis_common_ies_ver_2_0_0.proto |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

9.3.4.1.2	API Definitions

9.3.4.1.2.1	Enabl_Discovery_API

syntax = "proto3";

package ricapi.enabl_discovery_api.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "enabl_apis_common_ies_ver_2_0_0.proto";

// gRPC service and rpc definition

service Enabl_Discovery_API {

rpc ApiDiscoveryProc(ApiDiscoveryInitMsg) returns (ApiDiscoveryOutMsg) {};

}

message ApiDiscoveryInitMsg {

optional ApiDiscoveryRequest api_discovery_request = 1;

}

message ApiDiscoveryOutMsg {

oneof type_of_message {

ApiDiscoveryResponse api_discovery_response = 1;

ApiDiscoveryFailure api_discovery_failure = 2;

}

}

// ---------------- messages of Enabl_Discovery_API ----------------

// -------- messages of API Discovery procedure --------

message ApiDiscoveryRequest {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 3;

optional QueryParameters qps = 2;

message QueryParameters {

optional near_rt_ric_apis_common_ies.v2.ApiName api_name = 1;

optional near_rt_ric_apis_common_ies.v2.ApiVersion api_version = 2;

optional enabl_apis_common_ies.v2.Protocol protocol = 3;

optional enabl_apis_common_ies.v2.DataFormat data_format = 4;

optional enabl_apis_common_ies.v2.ApiCategory api_category = 5;

};

};

message ApiDiscoveryResponse {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 3;

repeated enabl_apis_common_ies.v2.ApiInformation list_of_api_information = 2;

};

message ApiDiscoveryFailure {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 4;

optional enabl_apis_common_ies.v2.Cause cause = 2;

optional uint32 time_to_wait = 3;

};

9.3.4.1.2.2	Enabl_Events_API

syntax = "proto3";

package ricapi.enabl_events_api.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "enabl_apis_common_ies_ver_2_0_0.proto";

// gRPC service and rpc definition

service Enabl_Events_API {

rpc ApiEventSubscriptionProc(ApiEventSubscriptionInitMsg) returns (ApiEventSubscriptionOutMsg) {};

rpc ApiEventSubscriptionDeleteProc(ApiEventSubscriptionDeleteInitMsg) returns (ApiEventSubscriptionDeleteOutMsg) {};

}

message ApiEventSubscriptionInitMsg {

optional ApiEventSubscriptionRequest api_event_subscription_request = 1;

}

message ApiEventSubscriptionOutMsg {

oneof type_of_message {

ApiEventSubscriptionResponse api_event_subscription_response = 1;

ApiEventSubscriptionFailure api_event_subscription_failure = 2;

}

}

message ApiEventSubscriptionDeleteInitMsg {

optional ApiEventSubscriptionDeleteRequest api_event_subscription_delete_request = 1;

}

message ApiEventSubscriptionDeleteOutMsg {

oneof type_of_message {

ApiEventSubscriptionDeleteResponse api_event_subscription_delete_response = 1;

ApiEventSubscriptionDeleteFailure api_event_subscription_delete_failure = 2;

}

}

service Enabl_Events_API_Notification {

rpc ApiEventNotificationProc(ApiEventNotificationInitMsg) returns (ApiEventNotificationOutMsg) {};

}

message ApiEventNotificationInitMsg {

optional ApiEventNotification api_event_notification = 1;

}

message ApiEventNotificationOutMsg {

oneof type_of_message {

ApiEventNotificationACK api_event_notification_ack = 1;

}

}

// ---------------- messages of Enabl_Events_API ----------------

// -------- messages of API Event Subscription procedure --------

message ApiEventSubscriptionRequest {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 5;    repeated EventItem list_of_events = 2;

optional EventNotificationRequirements event_notification_requirements = 3;

optional near_rt_ric_apis_common_ies.v2.EndpointInformation notification_destination = 4;

message EventItem {

optional enabl_apis_common_ies.v2.EventType event_type = 1;

repeated ApiIdFilterItem api_id_filter = 2;

message ApiIdFilterItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

}

}

message EventNotificationRequirements {

optional NotificationMethod notification_method = 1;

optional uint32 notification_period = 2;

enum NotificationMethod {

NOTIFICATION_METHOD_ON_EVENT = 0;

NOTIFICATION_METHOD_PERIODIC = 1;

}

}

};

message ApiEventSubscriptionResponse {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 3;

optional enabl_apis_common_ies.v2.SubscriptionId subscription_id = 2;

}

message ApiEventSubscriptionFailure {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 4;

optional enabl_apis_common_ies.v2.Cause cause = 2;

optional uint32 time_to_wait = 3;

}

// -------- messages of API Event Subscription Delete procedure --------

message ApiEventSubscriptionDeleteRequest {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 3;

optional enabl_apis_common_ies.v2.SubscriptionId subscription_id = 2;

};

message ApiEventSubscriptionDeleteResponse {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 3;

optional enabl_apis_common_ies.v2.SubscriptionId subscription_id = 2;

}

message ApiEventSubscriptionDeleteFailure {

optional near_rt_ric_apis_common_ies.v2.XappId xapp_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 4;

optional enabl_apis_common_ies.v2.Cause cause = 2;

optional uint32 time_to_wait = 3;

}

// -------- messages of API Event Notification procedure --------

message ApiEventNotification {

optional enabl_apis_common_ies.v2.SubscriptionId subscription_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 4;

optional enabl_apis_common_ies.v2.EventType event_type = 2;

optional EventDetail event_detail = 3;

message EventDetail {

repeated enabl_apis_common_ies.v2.ApiId list_of_api_ids = 1;

repeated enabl_apis_common_ies.v2.ApiInformation list_of_api_information = 2;

};

};

message ApiEventNotificationACK {

optional enabl_apis_common_ies.v2.SubscriptionId subscription_id = 1;

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 2;

};

###### 9.3.4.1.2.3	Enabl_Registration_API

syntax = "proto3";

package ricapi.enabl_registration_api_v1;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

import "enabl_apis_common_ies_ver_2_0_0.proto";

// gRPC service and rpc definition

service Enabl_Registration_API {

rpc ApiRegistrationProc(ApiRegistrationInitMsg) returns (ApiRegistrationOutMsg) {};

rpc ApiRegistrationUpdateProc(ApiRegistrationUpdateInitMsg) returns (ApiRegistrationUpdateOutMsg) {};

rpc ApiDeregistrationProc(ApiDeregistrationInitMsg) returns (ApiDeregistrationOutMsg) {};

}

message ApiRegistrationInitMsg {

optional ApiRegistrationRequest api_registration_request = 1;

}

message ApiRegistrationOutMsg {

oneof type_of_message {

ApiRegistrationResponse api_registration_response = 1;

ApiRegistrationFailure api_registration_failure = 2;

}

}

message ApiRegistrationUpdateInitMsg {

optional ApiRegistrationUpdateRequest api_registration__update_request = 1;

}

message ApiRegistrationUpdateOutMsg {

oneof type_of_message {

ApiRegistrationUpdateResponse api_registration_update_response = 1;

ApiRegistrationUpdateFailure api_registration_update_failure = 2;

}

}

message ApiDeregistrationInitMsg {

optional ApiDeregistrationRequest api_deregistration_request = 1;

}

message ApiDeregistrationOutMsg {

oneof type_of_message {

ApiDeregistrationResponse api_deregistration_response = 1;

ApiDeregistrationFailure api_deregistration_failure = 2;

}

}

// ---------------- messages of Enabl_Registration_API ----------------

// -------- messages of API Registration procedure --------

message ApiRegistrationRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiInformationToRegisterItem list_of_api_information_to_register = 2;

message ApiInformationToRegisterItem {

optional enabl_apis_common_ies.v2.ApiInformation api_information = 1;

}

}

message ApiRegistrationResponse {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiInformationSuccessfullyRegisteredItem list_of_api_information_successfully_registered = 2;

repeated ApiInformationFailedToRegisterItem list_of_api_information_failed_to_register = 3;

message ApiInformationSuccessfullyRegisteredItem {

optional enabl_apis_common_ies.v2.ApiInformation api_information = 1;

}

message ApiInformationFailedToRegisterItem {

optional enabl_apis_common_ies.v2.ApiInformation api_information = 1;

optional enabl_apis_common_ies.v2.Cause cause = 2;

}

}

message ApiRegistrationFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiInformationFailedToRegisterItem list_of_api_information_failed_to_register = 2;

message ApiInformationFailedToRegisterItem {

optional enabl_apis_common_ies.v2.ApiInformation api_information = 1;

optional enabl_apis_common_ies.v2.Cause cause = 2;

}

}

// -------- messages of API Registration Update procedure --------

message ApiRegistrationUpdateRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiInformationToUpdateItem list_of_api_information_to_update = 2;

message ApiInformationToUpdateItem {

optional enabl_apis_common_ies.v2.ApiInformation api_information = 1;

}

}

message ApiRegistrationUpdateResponse {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiIdSuccessfullyUpdatedItem list_of_api_ids_successfully_updated = 2;

repeated ApiIdFailedToUpdateItem list_of_api_ids_failed_to_update = 3;

message ApiIdSuccessfullyUpdatedItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

}

message ApiIdFailedToUpdateItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

optional enabl_apis_common_ies.v2.Cause cause = 2;

}

}

message ApiRegistrationUpdateFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiIdFailedToUpdateItem list_of_api_ids_failed_to_update = 2;

message ApiIdFailedToUpdateItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

optional enabl_apis_common_ies.v2.Cause cause = 2;

}

}

// -------- messages of API Deregistration procedure --------

message ApiDeregistrationRequest {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiIdToDeregisterItem list_of_api_ids_to_deregister = 2;

message ApiIdToDeregisterItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

}

}

message ApiDeregistrationResponse {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiIdSuccessfullyDeregisteredItem list_of_api_ids_successfully_deregistered = 2;

repeated ApiIdFailedToDeregisterItem list_of_api_ids_failed_to_deregister = 3;

message ApiIdSuccessfullyDeregisteredItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

}

message ApiIdFailedToDeregisterItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

optional enabl_apis_common_ies.v2.Cause cause = 2;

}

}

message ApiDeregistrationFailure {

optional near_rt_ric_apis_common_ies.v2.ProcedureTransactionId procedure_transaction_id = 1;

repeated ApiIdFailedToDeregisterItem list_of_api_ids_failed_to_deregister = 2;

message ApiIdFailedToDeregisterItem {

optional enabl_apis_common_ies.v2.ApiId api_id = 1;

optional enabl_apis_common_ies.v2.Cause cause = 2;

}

}

9.3.4.1.3	Information Element Definitions

syntax = "proto3";

package ricapi.enabl_apis_common_ies.v2;

import "near_rt_ric_apis_common_ies_ver_2_0_0.proto";

// ----------------------------- Information Elements -----------------------------

// --------------A--------------

enum ApiCategory {

APICATEGORY_E2_RELATED = 0;

APICATEGORY_SDL = 1;

APICATEGORY_A1_RELATED = 2;

};

message ApiId {

optional string value = 1;

}

message ApiInformation {

optional near_rt_ric_apis_common_ies.v2.ApiName api_name = 1;

optional ApiId api_id = 2;

optional ApiCategory api_category = 3;

optional string description = 4;

repeated ApiProfile list_of_api_profiles = 5;

message ApiProfile {

optional near_rt_ric_apis_common_ies.v2.ApiVersion api_version = 1;

optional Protocol protocol = 2;

optional DataFormat data_format = 3;

optional near_rt_ric_apis_common_ies.v2.EndpointInformation endpoint_information = 4;

repeated SupportedDataTypeItem list_of_supported_data_types = 5;

message SupportedDataTypeItem {

optional string data_type_id = 1;

}    };

};

// --------------C--------------

enum Cause {

UNSPECIFIED = 0;

SYSTEM_BUSY = 1;

DUPLICATE_API_INFORMATION = 2;

UNKNOWN_API_ID = 3;

INVALID_DATA_TYPE = 4;

UNKNOWN_SUBSCRIPTION_ID = 5;

}

// --------------D--------------

enum DataFormat {

DATA_FORMAT_PROTO3 = 0;

DATA_FORMAT_JSON = 1;

DATA_FORMAT_ASN1 = 2;

};

enum EventType {

EVENT_TYPE_NEAR_RT_RIC_API_AVAILABLE = 0;

EVENT_TYPE_NEAR_RT_RIC_API_UNAVAILABLE = 1;

EVENT_TYPE_NEAR_RT_RIC_API_UPDATE = 2;

}

// --------------P--------------

enum Protocol {

PROTOCOL_GRPC = 0;

PROTOCOL_SCTP = 1;

PROTOCOL_HTTP = 2;

};

// --------------S--------------

message SubscriptionId {

optional string value = 1;

}

9.3.5	Message Transfer Syntax

9.3.5.1	Solution 1: gRPC with Protocol Buffers

The message transfer syntax for Solution 1 is serialized Protocol Buffers [4].

10	Common definitions across APIs

10.1	Information element definitions

10.1.1	PTI

The Procedure Transaction Identity (PTI) IE allows identifing a procedure among all ongoing parallel procedures of the same type initiated by the same RICAPI peer. The PTI value is assigned to the initiating message by the procedure initiator, and subsequent messages shall include the same PTI value as in the initiating message, if any. PTI is released when the procedure is completed.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PTI | M |  | INTEGER (0..232-1) |  |

10.1.2	xApp ID

The xApp ID IE uniquely identifies an xApp.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| xApp ID | M |  | STRING | The format of the xApp ID shall be a Universally Unique Identifier (UUID) version 4, as described in [9]. |

10.1.3	API Name

The API Name IE indicates the Name of each Near-RT RIC API.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| API Name | M |  | ENUMERATED (E2_Subscription_API, E2_Indication_API, E2_Control_API, E2_Guidance_API, A1_Policy _API, A1_EI_API, SDL_Information_API, Xapp_Data_Sharing_API, ...) | Contains the Near-RT RIC API name. |

The table 10.1.3-1 lists the API Names.

Table 10.1.3-1: List of API Names

| API Category | API Name |
| --- | --- |
| E2 related APIs | E2_Subscription_API, E2_Indication_API, E2_Control_API, E2_Guidance_API |
| A1 related APIs | A1_Policy_API, A1_EI_API |
| SDL APIs | SDL_Information_API, Xapp_Data_Sharing_API |

10.1.4	API Version

The API Version IE indicates the Version of each Near-RT RIC API.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| API Version | M |  | STRING | Contains the Near-RT RIC API Version. |

10.1.5	A1 Policy Type ID

The A1 Policy Type ID IE identifies the type of one A1 policy.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| A1 Policy Type ID | M |  | STRING | Applicable values defined in [7]. |

10.1.6	Global E2 Node ID

Defined in 9.2.6 of [3].

10.1.7	RAN Function ID

Defined in 9.2.8 of [3].

10.1.8	SCTP API PDU

The SCTP API PDU IE serves as the outer layer encapsulation for the payload when SCTP is used as the transport protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| API Name | M |  | 10.1.3 |  |
| API Version | M |  | 10.1.4 |  |
| API Payload | M |  | OCTET STRING |  |

10.1.9	Endpoint Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Domain Name | O |  | STRING | (NOTE1) |
| IPv4 Address | O |  | STRING | (NOTE1) |
| IPv6 Address | O |  | STRING | (NOTE1) |
| Port | O |  | INTEGER |  |
| NOTE1: At least one of these IEs shall be present. |  |  |  |  |

10.2	Information element abstract syntax

10.2.1	Abstact syntax in Protocol Buffers

Table 10.2.1-1 lists the common IEs abstract syntax included in this document.

Table 10.2.1-1: List of common IEs abstract syntax

| API Name / Contents | Clause | Version | File Name |
| --- | --- | --- | --- |
| Near-RT RIC API common information elements | 10.2.1 | 2.0.0 | near_rt_ric_apis_common_ies_ver_2_0_0.proto |

syntax = "proto3";

package ricapi.near_rt_ric_apis_common_ies.v2;

// Information elements used across different API categories

// ----------------------------- Information Elements -----------------------------

// --------------A--------------

// From A1TD Clause 5.2

message A1PolicyTypeId {

optional string value = 1;

}

enum ApiName {

APINAME_E2_SUBSCRIPTION_API = 0;

APINAME_E2_INDICATION_API = 1;

APINAME_E2_CONTROL_API = 2;

APINAME_E2_GUIDANCE_API = 3;

APINAME_A1_POLICY_API = 4;

APINAME_A1_EI_API = 5;

APINAME_SDL_INFORMATION_API = 6;

APINAME_XAPP_DATA_SHARING_API = 7;

};

message ApiVersion {

optional string value = 1;

}

// --------------B--------------

// BitString definition

message BitString {

optional bytes value = 1;

optional uint32 len = 2;

}

// --------------E--------------

message EndpointInformation {

optional string domain_name = 1;

optional string ipv4_address = 2;

optional string ipv6_address = 3;

optional uint32 port = 4;

};

// --------------G--------------

// From E2AP Clause 9.2.6, "Global E2 Node ID"

message GlobalE2NodeId {

oneof node_type {

GlobalE2NodeGnbId gnb = 1;

GlobalE2NodeEnGnbId en_gnb = 2;

GlobalE2NodeNgEnbId ng_enb = 3;

GlobalE2NodeEnbId enb = 4;

}

message GlobalE2NodeGnbId {

optional GlobalGnbId global_gnb_id = 1;

optional GlobalEnGnbId global_en_gnb_id = 2;

optional GnbCuUpId gnb_cu_up_id = 3;

optional GnbDuId gnb_du_id = 4;

}

message GlobalE2NodeEnGnbId {

optional GlobalEnGnbId global_en_gnb_id = 1;

optional GnbCuUpId en_gnb_cu_up_id = 2;

optional GnbDuId en_gnb_du_id = 3;

}

message GlobalE2NodeNgEnbId {

optional GlobalNgEnbId global_ng_enb_id = 1;

optional GlobalEnbId global_enb_id = 2;

optional NgEnbDuId ng_enb_du_id = 3;

}

message GlobalE2NodeEnbId {

optional GlobalEnbId global_enb_id = 1;

}

}

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message GlobalEnbId {

optional PlmnIdentity plmn_identity = 1;

oneof enb_id_choice {

BitString macro_enb_id = 2;

BitString home_enb_id = 3;

BitString short_macro_enb_id = 4;

BitString long_macro_enb_id = 5;

}

}

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message GlobalEnGnbId {

optional PlmnIdentity plmn_identity = 1;

oneof en_gnb_id_choice {

BitString en_gnb_id = 2;

}

}

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message GlobalGnbId {

optional PlmnIdentity plmn_identity = 1;

oneof gnb_id_choice {

BitString gnb_id = 2;

}

}

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message GlobalNgEnbId {

optional PlmnIdentity plmn_identity = 1;

oneof ng_enb_id_choice {

BitString macro_ng_enb_id = 2;

BitString short_macro_ng_enb_id = 3;

BitString long_macro_ng_enb_id = 4;

}

}

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message GnbCuUpId {

optional uint64 value = 1;

}

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message GnbDuId {

optional uint64 value = 1;

}

// --------------N--------------

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message NgEnbDuId {

optional uint64 value = 1;

};

// --------------P--------------

// From E2AP Clause 9.2.6, as part of "Global E2 Node ID"

message PlmnIdentity {

optional bytes value = 1;

};

message ProcedureTransactionId {

optional uint32 value = 1;

};

// --------------R--------------

// From E2AP Clause 9.2.8, "RAN Function ID"

message RanFunctionId {

optional uint32 value = 1;

};

//---------------S--------------

// Outer layer encapsulation for SCTP based APIs

message SctpApiPdu {

optional ApiName api_name = 1;

optional ApiVersion api_version = 2;

optional bytes api_payload = 3;

}

// --------------X--------------

message XappId {

optional string value = 1;

}

Annex (informative): 
Change History

| Date | Revision | Description |
| --- | --- | --- |
| 2024.04.01 | 02.00 | Version inceremented for TSC approval. |
| 2024.04.01 | 01.00.06 | Editorial updates to capture comments received during on-line vote. |
| 2024.03.23 | 01.00.05 | Enclosure of CRs: CMCC-2024.02.29-WG3-CR-0014-RICAPI-API registration service stage 3 definitions-v02.docx, NEC.AO-2024.02.07-WG3-CR-027-RICAPI- Introduction of gRPC for E2 related and SDL API - v5.docx, CMCC-2024.03.13-WG3-CR-0015-RICAPI-xApp's Endpoints for E2 Indications and E2 Subscription Delete Notifications-v01.docx agreed at RICARCH #102. NOK-2023.12.04-WG3-CR0003-RICAPI-v6.docx agreed at WG3 regular #218. Editorial updates. |
| 2024.03.14 | 01.00.04 | Enclosure of CRs: CICT-2024.01.19-WG3-CR-0015-RICAPI corrections_v2.docx agreed at RICARCH #100. NOK-2023.02.01-WG3-CR0004-RICAPI-Timer-v3.docx agreed at 202402 Athens F2F RICARCH stream. CMCC-2024.01.27-WG3-CR-0013-RICAPI-SDL Information API stage 3 updates-v03.docx, and CMCC.AO-2024.01.21-WG3-CR-0012-RICAPI-xApp Data Sharing API and support for RAI-v04.docx agreed at RICARCH #101. Editorial updates. |
| 2023.11.23 | 01.00.03 | Enclosure of CRs: NEC-2023.08.17-WG3-CR-0019-RICAPI- Clarification on Data format-v02.docx agreed at RICARCH #93. CICT-2023.09.11-WG3-CR-0014-Corrections on RICAPI_v1.docx agreed at 202310 Phoenix F2F RICARCH stream. Editorial updates. |
| 2023.08.14 | 01.00.02 | Adopt new TS template. Enclosure of CR: CMCC-2023.06.13-WG3-CR-0011-RICAPI-Correction of the BitString definition in GPB code-v02.docx agreed at RICARCH #92. Editorial updates. |
| 2023.06.08 | 01.00.01 | Enclosure of CR: CMCC.AO-2023.02.03-WG3-CR-0002-RICAPI-E2 Subscription Delete API Procedure (Platform initiated) descriptions and messages_v05.docx agreed at 202302 Prague F2F RICARCH stream. Editorial updates. |
| 2022.11.20 | 01.00 | Version inceremented for TSC approval. |
| 2022.11.16 | 00.06.00 | Enclosure of CR: VMW.AO-2022.11.11-WG3-CR-0003-RICAPI-Improvements on Proto3 definitions-v2.zip agreed at RICARCH #79. Editorial updates. |
| 2022.11.12 | 00.05.00 | Enclosure of CR: CMCC.AO-2022.11.09-WG3-CR-0010-RICAPI-Collection of Common IEs across API categories under Section 10-v1.zip agreed at RICARCH TG 20221111 Ad-hoc meeting. Editorial updates. |
| 2022.11.09 | 00.04.00 | Copyright statement update in accordance to O-RAN alliance rules. Enclosure of CRs: CMCC.AO-2022.05.13-WG3-CR-0003-RICAPI-E2 related APIs overview_v04.docx agreed at RICARCH #76. AsiaInfo.AO-2022.08.05-WG3-CR-0004-RICAPI-Cause and Criticality Diagnostics IE Definition-v04.docx, and CMCC.AO-2022.09.16-WG3-CR-0006-RICAPI-A1 related APIs overview_v01.docx agreed at RICARCH #77. CMCC.AO-2022.10.10-WG3-CR-0005-RICAPI-Proto3 Definitions for E2 related APIs-v1.zip agreed at 202210 Madrid F2F RICARCH stream. CMCC-2022.08.17-WG3-CR-0004-RICAPI-Basic contents in Section 4-v3.docx, CMCC-2022.09.16-WG3-CR-0007-RICAPI-Enablement APIs overview for Section 9.1-v2.docx, and CMCC-2022.11.01-WG3-CR-0009-RICAPI-Alignment on Message Type IEs for all API categories-v1.docx agreed at RICARCH #78. CICT.AO-2021.06.01-WG3-CR-0004-procedural descriptions and msgs applied to A1-related API_v9.zip, CMCC.AO-2022.10.25-WG3-CR-0008-SDL APIs overview and Proto3 Definitions-v3.zip, AsiaInfo.AO-2022.10.11-WG3-CR-0005-RICAPI-xApp Registration Procedure Description and Messages-v3.docx, AsiaInfo.AO-2022.10.20-WG3-CR-0006-RICAPI-Proto3 Definitions for Management APIs-v1.zip, and CMCC.AO-2022.08.22-WG3-CR-0005-RICAPI-Proto3 Definitions for Enablement APIs-v4.zip agreed at RICARCH TG 20221108 Ad-hoc meeting. Editorial updates. |
| 2022.08.10 | 00.03.00 | Clause assigned per API message. Enclosure of CR: AsiaInfo.AO-2021.11.17-WG3-CR-0001-RICAPI-E2 Subscription API Procedure descriptions and messages_v09.docx agreed at RICARCH #74. Minor editorial update. |
| 2022.06.06 | 00.02.00 | Adoption of new O-RAN TS template. Enclosure of CRs: NOK-2022.03.21-WG3-CR-0001-RICAPI-E2NodeState-v02.docx agreed at RICARCH #67. Lenovo.AO-2022.05.13-WG3-RICAPI-Enablement API procedure descriptions and definitions_v04.docx, CICT-2022.04.22-WG3-CR-0009-Procedure Transaction Identity_v3.docx , AsiaInfo.AO-2022.01.04-WG3-CR-0002-RICAPI-E2 Indication API Procedure descriptions and messages_v04.docx, and AsiaInfo.AO-2022.03.04-WG3-CR-0003-RICAPI-E2 Control API Procedure descriptions and messages_v04.docx agreed at RICARCH #70. Minor editorial update. |
| 2021.10.12 | 00.01.00 | Enclosure of CR VMW.AO-2021.07.14-WG3-CR-0002-SDLAPI-v04.docx agreed at RICARCH #56. |
| 2021.05.18 | 00.00.03 | Approved TS skeleton. |