## ORAN.WG7.OMC-HRD.07-2-R003-v04.00

Technical Specification

O-RAN White Box Hardware Working Group Hardware Reference Design Specification for Outdoor Micro Cell

with Split Architecture Option 7.2

Copyright © 2023 by the O-RAN ALLIANCE e.V.

The copying or incorporation into any other work of part or all of the material available in this specification in any form without the prior written permission of O-RAN ALLIANCE e.V. is prohibited, save that you may print or download extracts of the material of this specification for your personal use, or copy the material of this specification for the purpose of sending to individual third parties for their information provided that you acknowledge O-RAN ALLIANCE as the source of the material and that you inform the third party that these conditions apply to them and that they must comply with them.

O-RAN ALLIANCE e.V., Buschkauler Weg 27, 53347 Alfter, Germany

# Revision History

| Date | Revision | Author | Description |
| --- | --- | --- | --- |
| 02/15/2023 | v04.00 | WG7 | Addition of Optional Die to Die Interfacing in 2.3.2.3 |
| 06/22/2022 | v03.00 | WG7 | Expanded chapter 2.3.7 to include detailed information on thermal design. Added chapter 2.3.2.3 to provide details of optional internal interfaces within an O-RU. Updated header and footers. |
| 07/21/2021 | v02.00 | WG7 | Add normative and informative notification to scope. |
| 02/03/2021 | v01.00 | WG7 | First Published Version |

2

3

# Contents

- Revision History	2

- Tables	7

- Figures	8

- Chapter 1	Introductory Material	10

- 1.1	Scope	10

- 1.2	References	10

- 1.3	Definitions and Abbreviations	12

- 1.3.1	Definitions	12

- 1.3.2	Abbreviations	12

- Chapter 2	Hardware Reference Design 1	17

- 2.1	O-CU Hardware Reference Design	17

- 2.2	O-DU7-2 Hardware Reference Design	17

- 2.2.1	O-DU7-2 High-Level Functional Block Diagram	18

- 2.2.2	O-DU7-2 Hardware Design Description	18

- 2.2.3	O-DU7-2 Hardware Components	19

- 2.2.3.1	Digital Processing Unit	19

- 2.2.3.1.1	Digital Processing Unit Requirements	19

- 2.2.3.1.1.1	Interfaces	20

- 2.2.3.1.1.1.1	Memory Channel Interfaces	20

- 2.2.3.1.1.1.2	PCIe	20

- 2.2.3.1.1.1.3	Ethernet	20

- 2.2.3.1.2	Digital Processing Unit Design	20

- 2.2.3.2	Hardware Accelerator	21

- 2.2.3.2.1	Accelerator Design Solution 1	21

- 2.2.3.2.1.1	Accelerator Requirements	21

- 2.2.3.2.1.2	Accelerator Design	23

- 2.2.3.2.2	Accelerator Design Solution 2	23

- 2.2.3.2.2.1	Accelerator Requirement	24

- 2.2.3.2.2.2	Accelerator Design	24

- 2.2.3.2.3	Accelerator Design Solution 3	25

- 2.2.3.2.3.1	Accelerator Hardware Features	25

- 2.2.3.2.3.2	Hardware Accelerator Firmware Features	26

- 2.2.3.2.4	Accelerator Design Solution 4	27

- 2.2.3.2.4.1	Accelerator Hardware Features	27

- 2.2.3.2.4.2	Hardware Accelerator Firmware Features	28

- 2.2.3.2.5	Accelerator Design Solution 5	28

- 2.2.3.2.5.1	Accelerator Requirements	29

- 2.2.3.2.5.2	Accelerator Design	29

- 2.2.3.3	Baseboard Management Controller	30

- 2.2.4	Synchronization and Timing	31

- 2.2.4.1	Synchronization and Timing Design 1	31

- 2.2.4.2	Hardware Requirements	31

- 2.2.4.3	Hardware Design	31

- 2.2.5	External Interface Ports	31

- 2.2.5.1	Hardware Requirements	31

- 2.2.5.2	Hardware Design	32

- 2.2.6	O-DU Firmware	32

- 2.2.7	Mechanical	32

- 2.2.7.1	Mother Board	32

- 2.2.7.2	Chassis	33

- 2.2.7.3	Cooling	34

- 2.2.8	Power Unit	34

- 2.2.8.1	Hardware Requirements	34

- 2.2.8.2	Hardware Design	35

- 2.2.9	Thermal	35

- 2.2.10	Environmental and Regulations	35

- 2.3	O-RU7-2 Hardware Reference Design	36

- 2.3.1	O-RU7-2 High-Level Functional Block Diagram	36

- 2.3.2	O-RU7-2 Hardware Components	36

- 2.3.2.1	Digital Processing Unit	39

- 2.3.2.1.1	FPGA/ASIC Solution	40

- 2.3.2.1.1.1	FPGA Specifications	40

- 2.3.2.1.1.2	FPGA Design	41

- 2.3.2.2	RF Processing Unit	42

- 2.3.2.2.1	Transceiver Reference Design	42

- 2.3.2.2.1.1	Hardware Specifications	42

- 2.3.2.2.1.1.1	Interface	42

- 2.3.2.2.1.1.2	Algorithm	44

- 2.3.2.2.1.1.3	Device Configuration	44

- 2.3.2.2.1.1.4	Power Dissipation	44

- 2.3.2.2.1.1.5	RF Specifications	44

- 2.3.2.2.1.2	Hardware Design	47

- 2.3.2.2.2	Power Amplifier (PA) Reference Design	48

- 2.3.2.2.2.1	Hardware Specifications	48

- 2.3.2.2.2.1.1	Interface	48

- 2.3.2.2.2.1.2	Power Specifications	48

- 2.3.2.2.2.1.3	RF Specifications	48

- 2.3.2.2.2.2	Hardware Design	48

- 2.3.2.2.3	Low Noise Amplifier (LNA) Reference Design	49

- 2.3.2.2.3.1.1	Interface	49

- 2.3.2.2.3.1.2	Power Specifications	49

- 2.3.2.2.3.1.3	RF Performance Specifications	50

- 2.3.2.2.3.2	Hardware Design	50

- 2.3.2.2.4	Circulator Reference Design	50

- 2.3.2.2.4.1	Hardware Specifications	50

- 2.3.2.2.4.1.1	Interface	50

- 2.3.2.2.4.1.2	RF Specifications	51

- 2.3.2.2.4.2	Hardware Design	51

- 2.3.2.2.5	Antenna / Phased Array Reference Design	51

- 2.3.2.2.5.1	Hardware Specifications for Omnidirectional Antenna	51

- 2.3.2.2.5.1.1	Hardware Design for Omnidirectional Antenna	52

- 2.3.2.2.5.2	Hardware Specifications for Directional Antenna	52

- 2.3.2.2.5.2.1	Hardware Design for Directional Antenna	53

- 2.3.2.3	Internal Interfaces	54

- 2.3.2.3.1	Optional JES204 Interfacing	54

- 2.3.2.3.2	Optional Die to Die Interfacing	59

- 2.3.3	Synchronization and Timing	61

- 2.3.3.1	Hardware Specifications	61

- 2.3.3.1.1	Interface	61

- 2.3.3.1.2	Performance Specifications	62

- 2.3.3.1.3	Synchronizer	62

- 2.3.3.1.3.1	Hardware Specifications	62

- 2.3.3.1.3.1.1	Interface	62

- 2.3.3.1.3.1.2	Performance Specifications	62

- 2.3.3.1.3.2	Hardware Design	63

- 2.3.3.1.4	Reference Synthesizer	63

- 2.3.3.1.4.1	Hardware Specifications	63

- 2.3.3.1.4.1.1	Interface	63

- 2.3.3.1.4.1.2	Performance Specifications	64

- 2.3.3.1.4.2	Hardware Design	64

- 2.3.3.2	Hardware Design	64

- 2.3.4	External Interface Ports	64

- 2.3.4.1	Hardware Specifications	64

- 2.3.4.2	Hardware Design	65

- 2.3.4.2.1	Fronthaul Interface	65

- 2.3.4.2.2	Debug Interface	65

- 2.3.4.2.3	Power Interface	66

- 2.3.4.2.4	RF Interface	66

- 2.3.5	Mechanical	67

- 2.3.6	Power Unit	69

- 2.3.6.1	Hardware Specifications	69

- 2.3.6.2	Hardware Design	69

- 2.3.7	Thermal	70

- 2.3.8	Environmental and Regulations	71

- 2.4	Integrated O-DU & O-RU (gNB-DU) Reference Design	72

- 2.4.1	O-DU Portion of Integrated Reference Design	72

- 2.4.1.1	O-DU High-Level Functional Block Diagram	72

- 2.4.1.2	O-DU Hardware Components	72

- 2.4.1.2.1	Digital Processing Unit	72

- 2.4.1.2.2	Hardware Accelerator (if required by design)	72

- 2.4.1.2.2.1	Accelerator Design 1	72

- 2.4.1.2.2.2	Accelerator Design 2	72

- 2.4.1.3	Synchronization and Timing	72

- 2.4.2	O-RU Portion of Integrated Reference Design	72

- 2.4.2.1	O-RU High-Level Functional Block Diagram	72

- 2.4.2.2	O-RU Hardware Components	72

- 2.4.2.2.1	Digital Processing Unit	72

- 2.4.2.2.2	RF Processing Unit	72

- 2.4.2.2.2.1	Transceiver Reference Design	72

- 2.4.2.2.2.2	Power Amplifier (PA) Reference Design	72

- 2.4.2.2.2.3	Low Noise Amplifier (LNA) Reference Design	72

- 2.4.2.2.2.4	RF Switch Reference Design	72

- 2.4.2.2.2.5	Antenna / Phased Array Reference Design	72

- 2.4.2.2.3	Synchronization and Timing	72

- 2.4.3	External Interface Ports	72

- 2.4.4	O-DU/O-RU Firmware (if required by design)	72

- 2.4.5	Mechanical	72

- 2.4.6	Power Unit	72

- 2.4.7	Thermal	73

- 2.4.8	Environmental and Regulations	73

- 2.5	Integrated O-CU & O-DU Reference Design	73

- 2.5.1	O-CU Portion of Integrated Reference Design	73

- 2.5.1.1	Integrated O-CU High-Level Functional Block Diagram	73

- 2.5.1.2	O-CU Hardware Components	73

- 2.5.1.2.1	Digital Processing Unit	73

- 2.5.1.2.2	Hardware Accelerator (if required by design)	73

- 2.5.1.2.2.1	Accelerator Design 1	73

- 2.5.1.2.2.2	Accelerator Design 2	73

- 2.5.1.3	Synchronization and Timing	73

- 2.5.1.4	O-CU Firmware (if required by design)	73

- 2.5.2	O-DU Portion of Hardware Reference Design	73

- 2.5.2.1	O-DU High-Level Functional Block Diagram	73

- 2.5.2.2	O-DU Hardware Components	73

- 2.5.2.2.1	Digital Processing Unit	73

- 2.5.2.2.2	Hardware Accelerator (if required by design)	73

- 2.5.2.2.2.1	Accelerator Design 1	73

- 2.5.2.2.2.2	Accelerator Design 2	73

- 2.5.2.3	Synchronization and Timing	73

- 2.5.3	External Interface Ports	73

- 2.5.4	O-DU Firmware	73

- 2.5.5	Mechanical	73

- 2.5.6	Power Unit	74

- 2.5.7	Thermal	74

- 2.5.8	Environmental and Regulations	74

- 2.6	FHGW Hardware Reference Design	74

- 2.6.1	FHGW High-Level Functional Block Diagram	74

- 2.6.2	FHGW Hardware Components	74

- 2.6.2.1	Digital Processing Unit	74

- 2.6.3	Synchronization and Timing	74

- 2.6.4	External Interface Ports	74

- 2.6.5	Mechanical	74

- 2.6.6	Power	74

- 2.6.7	Thermal	74

- 2.6.8	Environmental	74

- Annex 1: Parts Reference List	74

- Annex 1.1 O-CU Reference Parts List	74

- Annex 1.2 O-DU7-2 Reference Parts List	75

- Annex 1.3 O-RU7-2 Reference Parts List	75

9

- Tables

- Table 2.2.3-1 Processor Feature List	20

- Table 2.2.3-2 Memory Channel Feature List	20

- Table 2.2.3-3 Accelerator Hardware Feature List	22

- Table 2.2.3-4 Accelerator Firmware Feature List	22

- Table 2.2.3-5 Accelerator Hardware Feature List	24

- Table 2.2.3-6 Accelerator Firmware Feature List	24

- Table 2.2.3-7 Accelerator Hardware Features	25

- Table 2.2.3-8 Hardware Accelerator Firmware Features	26

- Table 2.2.3-9 Accelerator Hardware Features	27

- Table 2.2.3-10 Hardware Accelerator Firmware Features	28

- Table 2.2.3-11 Accelerator Hardware Feature List]	29

- Table 2.2.3-12 Accelerator Firmware Feature List	29

- Table 2.2.5-1 External Port List	32

- Table 2.2.8-1 Power Supply Rail Requirements	34

- Table 2.2.8-2 Power Supply Unit Feature List	35

- Table 2.2.10-1 Environmental Features	35

- Table 2.3.2-1 FPGA Interface Specifications	41

- Table 2.3.2-2 FPGA Functional Blocks	42

- Table 2.3.2-3 RF Processing Unit Interface Specifications	42

- Table 2.3.2-4 JESD204B/C Serial Data Rates	44

- Table 2.3.2-5 Transceiver RF Specifications	44

- Table 2.3.2-6 Power Amplifier Interface Specifications	48

- Table 2.3.2-7 Power Amplifier RF Specifications	48

- Table 2.3.2-8 LNA Interfaces	49

- Table 2.3.2-9 LNA RF Specifications	50

- Table 2.3.2-10 Circulator Specifications	51

- Table 2.3.2-11 Omnidirectional Antenna Specifications	52

- Table 2.3.2-12 Directional Antenna Specifications	53

- Table 2.3.2-13 JESD204 Abridged Lexicon	55

- Table 2.3.3-1 Synchronization and Timing Interface Specifications	61

- Table 2.3.3-2 Synchronizer Interface Specifications	62

- Table 2.3.3-3 Synchronizer Performance Specifications	62

- Table 2.3.3-4 Synthesizer Interface Specifications	63

- Table 2.3.3-5 TCXO Clock Performance Specifications	64

- Table 2.3.4-1 External Port List	65

- Table 2.3.6-1 Transceiver Module Specifications	69

- Table 2.3.6-2 RF Front End Module Specifications	69

- Table 2.3.7-1 Thermal Conductivity of common Metals [13]	70

- Table 2.3.7-2 Convective heat transfer coefficient [14]	70

- Table 2.3.8-1 Environmental Features	71

11

# Figures

- Figure 2.2.1-1 O-DU7-2 Functional Block Diagram	18

- Figure 2.2.2-1 O-DU7-2 Hardware Block Diagram	19

- Figure 2.2.3-1 Accelerator Design 1 without optional NIC Device	23

- Figure 2.2.3-2 Accelerator Design 1 with optional NIC Device	23

- Figure 2.2.3-3 Structured ASIC Accelerator Design	24

- Figure 2.2.3-4 Dual-chip FPGA-based Hardware Acceleration in O-DU7-2	26

- Figure 2.2.3-5 Single-chip FPGA-based Hardware Acceleration in O-DU7-2	28

- Figure 2.2.3-6 Accelerator Design Using PCIe	30

- Figure 2.2.3-7 Accelerator Design Using Ethernet	30

- Figure 2.2.4-1 O-DU Timing Synchronization	31

- Figure 2.2.5-1 External Interface Reference Design	32

- Figure 2.2.7-1 Mother Board Layout Diagram	33

- Figure 2.2.7-2 Chassis Mechanical Diagram	34

- Figure 2.3.1-1 High-Level Functional Block Diagram	36

- Figure 2.3.2-1 4T4R General Block Diagram with full RF front end	37

- Figure 2.3.2-2 8T8R General Block Diagram with RF front end	38

- Figure 2.3.2-3 O-RU Architecture Example Implementation 1	39

- Figure 2.3.2-4 Digital Processing Unit Block Diagram (L1 Processing + optional Beamforming)	40

- Figure 2.3.2-5 Digital Processing Unit Block Diagram (DFE)	40

- Figure 2.3.2-6 Transceiver with Integrated CFR and DPD	47

- Figure 2.3.2-7 Power Amplifier Reference Design	49

- Figure 2.3.2-8 LNA Reference Design	50

- Figure 2.3.2-9 Circulator Reference Design	51

- Figure 2.3.2-10 Canister Style Omni Antenna	52

- Figure 2.3.2-11 Mounting arrangement scenario 1 of 4T4R radios on the back of Directional Antenna	53

- Figure 2.3.2-12 Mounting arrangement scenario 2 of 4T4R radios remotely mounted with Directional

- Antenna	54

- Figure 2.3.2-13 Example 4T4R JESD204 Configuration	56

- Figure 2.3.2-14 4T4R Example Data Mapping	57

- Figure 2.3.2-15 Example 8T8R JESD204 Configuration	58

- Figure 2.3.2-16 8T8R Example Data Mapping	59

- Figure 2.3.17-1 Basic die-to-die parallel architecture	59

- Figure 2.3.3-1 Clock and Synchronization Functional Block Diagram	61

- Figure 2.3.3-2 Synchronizer Block Diagram	63

- Figure 2.3.3-3 Synthesizer Block Diagram	64

- Figure 2.3.4-1 SFP+ case and connector	65

- Figure 2.3.4-2 RJ45 Interface	66

- Figure 2.3.4-3 Power Interface	66

- Figure 2.3.4-4 (a) 4.3-10+ RF Connector [9] [11] (b) M-LOC cluster/multiport connector [12]	67

- Figure 2.3.5-1 Mechanical Enclosure Side View	68

- Figure 2.3.5-2 Mechanical Enclosure End View	68

- Figure 2.3.6-1 Power Reference Design	70

15

16

# Chapter 1	Introductory Material

## 1.1 Scope

- This Technical Specification has been produced by the O-RAN.org.

- The contents of the present document are subject to continuing work within O-RAN WG7 and may change

- following formal O-RAN approval. Should the O-RAN.org modify the contents of the present document, it

- will be re-released by O-RAN Alliance with an identifying change of release date and an increase in version

- number as follows:

- Release x.y.z

- where:

- x  the first digit is incremented for all changes of substance, i.e. technical enhancements, corrections,

- updates, etc. (the initial approved document will have x=01).

- y  the second digit is incremented when editorial only changes have been incorporated in the

- document.

- z  the third digit included only in working versions of the document indicating incremental changes

- during the editing process. This variable is for internal WG7 use only.

- The present document specifies system requirements and high-level architecture for the Outdoor Micro Cell

- for FR1 deployment scenario as specified in the Deployment Scenarios and Base Station Classes document

- [1].

- In the main body of this specification (in any “chapter”) the information contained therein is informative,

- unless explicitly described as normative. Information contained in an “Annex” to this specification is always

- informative unless otherwise marked as normative.

## 1.2 References

- The following documents contain provisions which, through reference in this text, constitute provisions of

- the present document.

- [1] ORAN-WG7.DSC.0-V02.00 Technical Specification, ‘Deployment Scenarios and Base Station Classes

- for White Box Hardware’. https://www.o-ran.org/specifications

- [2] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- [3] 3GPP TR 38.104: "NR; Base Station (BS) radio transmission and reception".

- http://www.3gpp.org/ftp//Specs/archive/38_series/38.104/38104-g10.zip

- [4] ORAN-WG4.CUS.0-v05.00 Technical Specification, ‘O-RAN Fronthaul Working Group Control, User

- and Synchronization Plane Specification’. https://www.o-ran.org/specifications

- [5] 3GPP	TS	38.113:"NR:	Base	Station	(BS)	Electromagnetic	Compatibility	(EMC)".

- http://www.3gpp.org/ftp//Specs/archive/38_series/38.113/38113-f80.zip

- [6] ORAN-WG7.OMC. HAR-v01.00 Technical Specification, ‘Outdoor Micro Cell Hardware Architecture

- and Requirement (FR1) Specification’. https://www.o-ran.org/specifications.

- [7] “Data Plane Development Kit”, https://www.dpdk.org/

- [8] “Baseband Device Library”, https://doc.dpdk.org/guides/prog_guide/bbdev.html.

- [9] IEC 61169-54 International Standard, “Radio frequency connectors – Part 54: Sectional specification for

- coaxial connectors with 10 mm inner diameter of outer conductor, nominal characteristic impedance 50 Ohms,

- Series 4.3-10”, https://webstore.ansi.org/Standards/BSI/BSENIEC61169542021

- [10] AISG 2.0, “Control interface for antenna line devices”, Antenna Interface Standards Group, June 2006.

- [11]

- https://www.rosenberger.com/0_documents/de/catalogs/ba_communication/catalog_coax/15_Chapter_43-

- 10_41-95.pdf (Accessed on March 30, 2022)

- [12]	https://www.commscope.com/product-type/cable-assemblies/wireless-cable-assemblies/coaxial-cable-

- assemblies/itemmloc-f1xm-4 (Accessed on March 30, 2022)

- [13] Zhong, J., Liu, D., Li, Z., &amp; Sun, X, High thermal conductivity materials and their application on the

- electronic products. In 2012 IEEE Asia-Pacific Conference on Antennas and Propagation, pp. 173-175, August

- 2012.

- [14] https://www.engineersedge.com/heat_transfer/convective_heat_transfer_coefficients	13378.htm

- (Accessed on April 8, 2022)

- [15] https://www.thermoworks.com/emissivity-table/ (Accessed on April 8, 2022)

- [16] “Serial Interface for Data Converters”, JESD204C.1, December 2021, www.jedec.org

- [17]	“JESD204x  Frame  Mapping  Table  Generator”,  https://www.analog.com/en/license/licensing-

- agreement/jesd204x-frame-mapping-table-generator.html

- [18] AIB specification: https://github.com/chipsalliance/AIB-specification

- [19] BoW PHY specification: https://opencomputeproject.github.io/ODSA-BoW/bow_specification.html

- [20] Universal Chiplet Interconnect Express web site:https://www.uciexpress.org/ 28

## 1.3 Definitions and Abbreviations

### 1.3.1 Definitions

- For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [2] and the

- following apply. A term defined in the present document takes precedence over the definition of the same

- term, if any, in 3GPP TR 21.905 [2]. For the base station classes of Pico, Micro and Macro, the definitions

- are given in 3GPP TR 38.104 [3].

- Integrated architecture: In the integrated architecture, the O-RU and O-DU are implemented on one

- platform. Each O-RU and RF front end is associated with one O-DU. They are then aggregated to O-CU and

- connected by F1 interface.

- Split architecture: The O-RU and O-DU are physically separated from one another in this architecture. A

- switch may aggregate multiple O-RUs to one O-DU. O-DU, switch and O-RUs are connected by the

- fronthaul interface as defined in WG4.

### 1.3.2 Abbreviations

- For the purposes of the present document, the abbreviations given in [2] and the following apply. An

- abbreviation defined in the present document takes precedence over the definition of the same abbreviation,

- if any, as in [2].

- 3GPP	Third Generation Partnership Project

- 5G	Fifth-Generation Mobile Communications

- 7-2	Fronthaul interface split option as defined by O-RAN WG4, also referred to as 7-2x

- ACLR	Adjacent Channel Leakage Ratio

- ADC	Analog to Digital Converter

- AGC	Automatic Gain Control

- AIB	Advanced Interface Bus

- AISG	Antenna Interface Standards Group

- ASIC	Application Specific Integrated Circuit

- BBDEV	Baseband Device

- BH	Backhaul

- BIOS	Basic Input/Output System

- BMC	Baseboard Management Controller

- BoW	Bunch of Wires

| 1 | BS | Base Station |
| --- | --- | --- |
| 2 | BW | Bandwidth |
| 3 | C2C | Chip to Chip Interface |
| 4 | CDR | Clock and Data Recovery |
| 5 | CFR | Crest Factor Reduction |
| 6 | CISPR | International Special Committee on Radio Interference |
| 7 | CLB | Configurable Logic Block |
| 8 | CPU | Central Processing Unit |
| 9 | CRC | Cyclic Redundancy Check |
| 10 | CU | Centralized Unit as defined by 3GPP |
| 11 | DAC | Digital to Analog Converter |
| 12 | DDC | Digital Down Conversion |
| 13 | DDR | Double Data Rate |
| 14 | DEVCLK | Device Clock |
| 15 | DIMM | Dual-Inline-Memory-Modules |
| 16 | D2D | Die-to-Die Interface |
| 17 | DL | Downlink |
| 18 | DMA | Direct Memory Access |
| 19 | DPD | Digital Pre-Distortion |
| 20 | DPDK | Data Plane Development Kit |
| 21 | DPU | Digital Processing Unit |
| 22 | DSP | Digital Signal Processor |
| 23 | DU | Distributed Unit as defined by 3GPP |
| 24 | DUC | Digital Up Conversion |
| 25 | ECC | Error Correcting Code |
| 26 | eCPRI | evolved Common Public Radio Interface |
| 27 | EMC | Electro Magnetic Compatibility |
| 28 | ENIAC | Embedded NIC Inline Accelerator |
| 29 | EVM | Error Vector Magnitude |

| 1 | FCC | Federal Communications Commission |
| --- | --- | --- |
| 2 | FEC | Forward Error Correction |
| 3 | FFT | Fast Fourier Transform |
| 4 | FH | Fronthaul |
| 5 | FHGW | Fronthaul Gateway |
| 6 | FHGWx | Fronthaul gateway with no FH protocol translation, supporting an O-DU with split option x |
| 7 |  | and an O-RU with split option x, with currently available options 6 6, 7-2 7-2 and 8 8 |
| 8 | FHGWxy | Fronthaul Gateway that can translate fronthaul protocol from an O-DU with split option x to |
| 9 |  | an O-RU with split option y, with currently available option 7-2 8. |
| 10 | FHHL | Full Height Half Length |
| 11 | FPGA | Field Programmable Gate Array |
| 12 | GbE | Gigabit Ethernet |
| 13 | GMC | Grand Master Clock |
| 14 | GNSS | Global Navigation Satellite System |
| 15 | GPIO | General Purpose Input Output |
| 16 | GPP | General Purpose Processor |
| 17 | GPS | Global Positioning System |
| 18 | GUI | Graphical User Interface |
| 19 | HAR | Hardware Architecture Requirement |
| 20 | HARQ | Hybrid Automatic Repeat request |
| 21 | HDD | Hard Disk Drive |
| 22 | HHHL | Half Height Half Length |
| 23 | HRD | Hardware Reference Design |
| 24 | HW | Hardware |
| 25 | I/O | Input/Output |
| 26 | I2C | Inter-Integrated Circuit (multi IC interface standard) |
| 27 | IEEE | Institute of Electrical and Electronics Engineers |
| 28 | IFFT | Inverse Fast Fourier Transform |
| 29 | IMC | Integrated Memory Controller |

| 1 | IMD | Inter Modulation Distortion |
| --- | --- | --- |
| 2 | JTAG | Joint Test Action Group |
| 3 | KVM | Kernel-based Virtual Machine |
| 4 | L1 | Layer 1 |
| 5 | L2 | Layer 2 |
| 6 | LDPC | Low-Density Parity Codes |
| 7 | LNA | Low Noise Amplifier |
| 8 | LRDIMM | Load-Reduced Dual In-line Memory Module |
| 9 | LTE | Long Term Evolution |
| 10 | LUT | Look-up Table |
| 11 | MH | Midhaul |
| 12 | NEBS | Network Equipment-Building System |
| 13 | NFV | Network Functions Virtualization |
| 14 | NIC | Network Interface Controller |
| 15 | NR | New Radio |
| 16 | OAM | Operations, administration and management |
| 17 | O-CU | O-RAN Centralized Unit as defined by O-RAN |
| 18 | OCXO | Oven Controlled Crystal Oscillator |
| 19 | O-DUx | A specific O-RAN Distributed Unit having fronthaul split option x where x may be 6, 7-2 |
| 20 |  | (as defined by WG4) or 8. |
| 21 | OMC | Outdoor Microcell |
| 22 | O-RUx | A specific O-RAN Radio Unit having fronthaul split option x, where x is 6, 7-2 (as defined |
| 23 |  | by WG4) or 8, and which is used in a configuration where the fronthaul interface is the same |
| 24 |  | at the O-DUx. |
| 25 | ORx | Observation Receiver |
| 26 | PA | Power Amplifier |
| 27 | PCH | Platform Controller Hub |
| 28 | PCIe | Peripheral Component Interface Express |
| 29 | PHC | Physical Hardware Clock |
| 30 | PHY | Physical Layer |

| 1 | PMBus | Power Management Bus |
| --- | --- | --- |
| 2 | PPS | Pulse Per Second |
| 3 | PRACH | Physical Random-Access Channel |
| 4 | PTP | Precision Time Protocol |
| 5 | PWM | Pulse Width Modulation |
| 6 | QSFP | Quad Small Form-factor Pluggable |
| 7 | RAN | Radio Access Network |
| 8 | RDIMM | Registered Dual In-line Memory Module |
| 9 | RE | Resource Element |
| 10 | RF | Radio Frequency |
| 11 | RoE | Radio over Ethernet |
| 12 | RU | Radio Unit as defined by 3GPP |
| 13 | Rx | Receiver |
| 14 | SDFEC | Soft-Decision Forward Error Correction |
| 15 | SERDES | Serializer / De-serializer |
| 16 | SFP | Small Form-factor Pluggable |
| 17 | SFP+ | Small Form-factor Pluggable plus |
| 18 | SMA | Sub-Miniature Version A (connector) |
| 19 | SoC | System On Chip |
| 20 | SPI | Serial Peripheral Interface |
| 21 | SSD | Solid State Drive |
| 22 | SyncE | Synchronous Ethernet |
| 23 | TCXO | Temperature Compensate Crystal Oscillator |
| 24 | TDD | Time Division Duplex |
| 25 | TDP | Thermal Design Power |
| 26 | TR | Technical Report |
| 27 | TR | Transmit Receive (switch) |
| 28 | TRx | Transceiver |
| 29 | TS | Technical Specification |

- Tx	Transmitter

- UEFI	Unified Extensible Firmware Interface

- UCIe	Universal Chiplet Interface Express

- UL	Uplink

- USB	Universal Serial Bus

- UTC	Coordinate Universal Time

- WG	Working Group

# Chapter 2	Hardware Reference Design 1

- This chapter describes a white box hardware reference design example for outdoor Microcell deployment

- scenario. It includes O-CU and O-DU7-2 for OMC deployment scenario.

## 2.1 O-CU Hardware Reference Design

- The O-CU white box hardware is the platform that perform the O-CU function of upper L2 and L3. The

- hardware systems specified in this document meet the computing, power and environmental requirements of

- use cases configurations and feature sets of RAN physical node. These requirements are described in the early

- hardware requirement specification as well as in the use cases document. The O-CU hardware includes the

- chassis platform, mother board, peripheral devices and cooling devices. The mother board contains processing

- unit, memory, the internal I/O interfaces, and external connection ports. The midhaul (MH) and backhaul (BH)

- interface are used to carry the traffic between O-CU and O-DU7-2 as well as O-CU and core network. The other

- hardware functional components include: the storage for software, hardware and system debugging interfaces,

- board management controller, just to name a few; the O-CU designer will make decision based on the specific

- needs of the implementation.

- The HW reference design of O-CU is the same as O-DU7-2 except for the need of HW accelerator, thus detail

- design will be described in O-DU7-2 section 2.2.

## 2.2 O-DU7-2 Hardware Reference Design

- The O-DU7-2 white box hardware is the platform that performs the O- DU7-2 functions such as upper L1 and

- lower L2 functions. The hardware systems specified in this document meet the computing, power and

- environmental requirements of use case’s configurations and feature sets of RAN physical node. These

- requirements are described in the early hardware requirement specification as well as in the use cases document.

- The O-DU7-2 hardware includes the chassis platform, mother board, peripheral devices and cooling devices.

- The mother board contains processing unit, memory, the internal I/O interfaces, and external connection ports.

- The fronthaul and backhaul interface are used to carry the traffic between O-RU7-2 / FHGW7-2 / FHGW7-28

- and O-DU7-2 as well as O-CU and O-DU7-2. The O-DU7-2 design may also provide an interface for hardware

- accelerator if that option is preferred. The other hardware functional components include: the storage for

- software, hardware and system debugging interfaces, board management controller, just to name a few; the O-

- DU7-2 designer will make decision based on the specific needs of the implementation.

- Note that the O-DU7-2 HW reference design is also feasible for O-CU and integrated O-CU / O-DU7-2.

### 2.2.1 O-DU7-2 High-Level Functional Block Diagram

- Figure 2.2.1-1 shows the major functional blocks of O-DU7-2. The digital processing unit handles the baseband

- processing workload. To make the processing more efficient, an accelerator can be used to assist with the

- baseband workload processing. The memory devices include the random-access memory (RAM) for

- temporary storage of data while flash memory is used for codes and logs. The storage device is for persistent

- storage. The external network cards can be used for fronthaul or backhaul connection.  The baseboard

- management controller (BMC) is a microcontroller which monitors hardware operation on motherboard. The

- clock circuits provide digital processing unit with required clock signals. 12

13

14	Figure 2.2.1-1 O-DU7-2 Functional Block Diagram

15

### 16	2.2.2 O-DU7-2 Hardware Design Description

17	Figure 2.2.2-1 describes the various components and connections inside the O-DU7-2 white box.

1

2	Figure 2.2.2-1 O-DU7-2 Hardware Block Diagram

3

- As described in the previous section, the O-DU7-2 hardware can be implemented with difference design choices.

- Here, we use a SoC design-based as an example which performs most of the workload for O-DU7-2. The

- accelerator can be used to perform other functions based on the overall performance requirement. Several

- Ethernet controllers are also used for front haul link, back haul link and remote console control connection.

- The other parts include: RAM, flash memory, and hard drive storage. The JTAG and USB ports are provided

- for hardware debug and local connection if needed. Finally, BMC block is mainly responsible for monitoring

- the hardware status of the motherboard.

### 2.2.3 O-DU7-2 Hardware Components

- In this section, we describe the details of the O-DU7-2 hardware component’s requirements, their features and

- parameters. The component selection is based on the use case requirements which are listed in the hardware

- architecture and requirements document [6].

#### 2.2.3.1 Digital Processing Unit

- This example of the digital processing unit in O-DU7-2 is based on the General-Purpose Processor (GPP). 17

- 2.2.3.1.1 Digital Processing Unit Requirements

- The GPP requirements are listed in Table 2.2.3-1.

1	Table 2.2.3-1 Processor Feature List

| Item Name | Description |
| --- | --- |
| # of Cores | 16 |
| # of Threads | 32 |
| Base Frequency | 2.20 GHz |
| Max Turbo Frequency | 3.00 GHz |
| Cache | 22 MB |
| Thermal Design Power (TDP) | 100W |
| Max Memory Size (dependent on memory type) | 512 GB |
| Memory Types | DDR4 |
| Max # of Memory Channels | 4 |

2

3	2.2.3.1.1.1	Interfaces

- The interface specifications on the main board are described in the sections below.

- 2.2.3.1.1.1.1  Memory Channel Interfaces

- The system memory capacity, type and related information are described in Table 2.2.3-2.

- Table 2.2.3-2 Memory Channel Feature List

| Item Name | Description |
| --- | --- |
| Memory Types | DDR4 |
| # of Memory Channels | 4 |
| ECC LRDIMM | Up to 512GB |
| ECC RDIMM | Up to 256GB |
| Memory Speed | 2666/2400/2133MHz |
| DIMM Sizes | 128GB, 64GB, 32GB, 16GB |
| Memory Voltage | 1.2 V |

8	2.2.3.1.1.1.2  PCIe

- PCIe Gen 3 should be supported by the processor. There are total of 32 PCIe lanes with 128 Gb/s bandwidth.

- The 32 PCIe lanes can be divided into two x16 slots by using a riser card.

- 2.2.3.1.1.1.3  Ethernet

- The system should be capable to offer aggregated 48 Gb/s Ethernet bandwidth. The breakout the ports are

- discussed in later section. When higher Ethernet bandwidth required, an Ethernet card can be installed in one

- of the PCIe slot.

- 2.2.3.1.2 Digital Processing Unit Design

- The digital processing unit is a system-on-a-chip (SoC) device which is a 64-bit multi-core server class

- processor. This SoC includes an integrated Platform Controller Hub (PCH), integrated high-speed I/O,

- Integrated Memory Controllers (IMC), and four integrated 10 Gigabit Ethernet ports.

- The SoC supports 512-bit wide vector processing instruction set. It also supports hardware virtualization to

- enable dynamic provisioning of services as communication service providers extend network functions

- virtualization (NFV). Figure 2.2.2-1 shows the major functional blocks of the digital processing unit.

#### 2.2.3.2 Hardware Accelerator

- Hardware accelerators can be used in O-DU7-2 to offload computationally intensive functions and to optimize

- the performance under varying traffic and loading conditions. While the hardware acceleration functional

- requirements and implementation are system designer’s choice; however, the O-DU7-2 is required to meet the

- minimum system performance requirements under various loading conditions and deployment scenarios. In

- most cases, a Field Programmable Gate Array (FPGA) or Application Specific Integrated Circuit (ASIC) based

- PCIe card can be used to optimize the system performance. The FPGA(s) are part of a Network Interface

- Controller (NIC) that further provides connectivity services.

- 2.2.3.2.1 Accelerator Design Solution 1

- Channel coding for Low-Density Parity Codes (LDPC) and fronthaul compression requires a significant

- amount of bit level processing and is well suited to a fine-grained FPGA architecture and/or low cost/power

- structured ASIC. Options include:

- •	Virtualization: Multi-channel DMA, Virtual machine integration, load balancing functions

- •	Look-aside FEC: LDPC (de)coding, Polar (de)coding, Rate (De)Matcher, (De) Interleaver, Cyclic

- Redundancy Check (CRC), Hybrid Automatic Repeat request (HARQ) retransmission

- •	Bump-in-wire Fronthaul: compression / decompression for latency and bandwidth reduction 20

- 2.2.3.2.1.1	Accelerator Requirements

- Hardware and firmware requirements for this accelerator design are given in Table 2.2.3-3 and Table 2.2.3-4,

- respectively.

1	Table 2.2.3-3 Accelerator Hardware Feature List

| Item Name | Description |
| --- | --- |
| PCIe (Interface with digital processing unit） | Gen4 x16 (and lower) |
| Form factor | FHHL |
| Connectivity | 2x QSFP28/56 |
| FPGA | Logic Elements: 1437K M20K Memory: 139Mb Quad A53 Hard Processor Sub-system |
| NIC Device | 100Gb xHAUL for FH, BH & MH traffic shaping. Optional FPGA co-processing. |
| DDR Main | 8+8+1GB DDR4 |
| Flash (FPGA images) | >=1 Gbit |
| BMC | Telemetry, Security, remote upgrade |
| Clocking | For O-RAN C1, C2, C3 & C4 |
| Fronthaul | eCPRI, RoE IEEE1914.3, O-RAN WG4 |
| GPS | SMA for 1 PPS & 10MHz (in/out) |
| Operating Temperature (ambient) | NEBS Compliant |
| Power | <75W (without optional NIC device) |
| Clock Accuracy | Low-Jitter, configurable clock ranging from 10MHz to 750MHz. Option for OCXO (TCXO as standard) |

2

3	Table 2.2.3-4 Accelerator Firmware Feature List

| Item Name | Description |
| --- | --- |
| Remote system upgrade | Securely upgradable FPGA flash image |
| Queuing | 64 Queues supported equally split between UL & DL. |
| LDPC Acceleration | NR LDPC Encoding with, interleaving and rate-matching. NR LDPC Decoding with sub-block de-interleaving function of reverse rate matching. Early Termination, CRC attachment and HARQ buffering. 5G Throughput: DL 14.8Gbps, UL 3.2Gbps |
| Load Balancing (channel coding) | Load balancer distributes the pending encoder/decoder requests to encoder/decoders |
| Descriptor Format (channel coding) | Code block-based interface. Software enablement by BBDEV API (DPDK) [7], [8] |
| Fronthaul Compression | In-line compression/decompression for Mu-Law, block-floating point and quantization according to the O-RAN WG4 specification. |
| Open programmable acceleration environment | Support for: - FPGA Flashing upgrade - Firmware version reporting - PCIe diagnostics - Ethernet diagnostics - Temperature and voltage telemetry information |

- 2.2.3.2.1.2	Accelerator Design

- Figure 2.2.3-1 illustrates Accelerator Design 1 without optional NIC Device. Figure 2.2.3-2 illustrates the

- Accelerator Design 1 with optional NIC Device.

4

5	Figure 2.2.3-1 Accelerator Design 1 without optional NIC Device

6

7

- Figure 2.2.3-2 Accelerator Design 1 with optional NIC Device

9

- 2.2.3.2.2 Accelerator Design Solution 2

- Channel coding for LDPC and fronthaul compression requires a significant amount of bit level processing and

- is well suited to a fine-grained structured ASIC. Features include:

- •	Virtualization: Multi-channel DMA, Virtual machine integration, load balancing functions

- •	Look-aside FEC: Turbo (de)coding, LDPC (de)coding, Rate (De)Matcher, (De) Interleaver, CRC, HARQ

- retransmission & (de) interleaver.

1	2.2.3.2.2.1	Accelerator Requirement

2	Accelerator Hardware and Firmware features are listed in Table 2.2.3-5 and Table 2.2.3-6, respectively.

3

4	Table 2.2.3-5 Accelerator Hardware Feature List

| Item Name | Description |
| --- | --- |
| PCIe | Gen3 x16 |
| Form factor | HHHL |
| DDR | DDR4 (64-bit +ECC), 2667Mbps Interface for HARQ buffering |
| Board Management Controller | Telemetry, Security. |
| Power | <35W |

5

6

7	Table 2.2.3-6 Accelerator Firmware Feature List

| Item Name | Description |
| --- | --- |
| Queuing | 64 Queues supported equally split between UL & DL. |
| LDPC Acceleration | NR LDPC Encoding with, interleaving and rate-matching. NR LDPC Decoding with sub-block de-interleaving function of reverse rate matching. Early Termination, CRC attachment and HARQ buffering. 5G Throughput: DL 23Gbps, UL 8Gbps |
| Load Balancing (channel coding) | Load balancer distributes the pending encoder/decoder requests to encoder/decoders |
| Descriptor Format (channel coding) | Code block and transport block-based interface. Software enablement by BBDEV API (DPDK) [7], [8] |

8

- 2.2.3.2.2.2	Accelerator Design

- The following diagram shows the structured ASIC based accelerator design.

11

- Figure 2.2.3-3 Structured ASIC Accelerator Design

13

  - 2.2.3.2.3 Accelerator Design Solution 3

  - The O-DU7-2 system is typically implemented using a multi-core processor and one or more hardware

  - accelerators. Parts of O-DU7-2 protocol stack can be implemented in software running on the multi-core

  - processors and the computationally intensive L1 and L2 functions can be offloaded to an FPGA-based

  - hardware accelerator. This accelerator comprises two FPGAs for L1 offload and fronthaul connectivity in

  - lookaside mode. The accelerator hardware and firmware features are listed in Table 2.2.3-7 and Table 2.2.3-8.

7

- 2.2.3.2.3.1	Accelerator Hardware Features

- Table 2.2.3-7 Accelerator Hardware Features

| Item Name | Description |  |  |
| --- | --- | --- | --- |
| SoC Resources | FPGA 1 |  | FPGA 2 |
|  | System Logic cells - 930K |  | System Logic cells - 1,143K |
|  | CLB LUT - 425K |  | CLB LUT - 523K |
|  | SDFEC -8 |  | CLB Flip-Flops -1,045K |
|  | DSP Slices - 4,272 |  | DSP Slices - 1,968 |
|  | BRAM - 38.0Mb |  | BRAM - 34.6Mb |
|  | URAM - 22.5Mb |  | URAM - 36.0Mb |
| Form Factor | FHHL PCIe Form Factor |  |  |
| PCIe Interface | Gen 3 x16 with Bifurcation |  |  |
| On Board Memory | FPGA1 |  | FPGA2 |
|  | Total Capacity 4 GB in |  | Total Capacity 4 GB in |
|  | Programmable Logic, |  | Programable Logic, upgradeable |
|  | upgradeable to 8GB |  | to 8GB |
|  | Total Capacity 2 GB in |  | Total Capacity 2 GB in |
|  | Programmable Subsystem, |  | Programmable Subsystem, |
|  | upgradeable to 4GB |  | upgradeable to 4GB |
| Network Interface(s) | 2xSFP28 optical interfaces to FPGA2 (User Configurable, includes 10/25 Ethernet) |  |  |
| Other External Interface(s) | Micro USB for JTAG support (FPGA programming and debug) and access to BMC |  |  |
| Graphical User interface | GUI for monitoring the basic board parameters, monitoring temperature alerts, firmware upgrades for BMC |  |  |
| Board Management Controller | Telemetry, Security, Remote Upgrade |  |  |
| Clocking | O-RAN C1, C2, C3, C4 |  |  |
| Fronthaul | eCPRI, RoE IEEE1914.3, O-RAN WG4 FH |  |  |
| GPS | GPS | SMA for 1 PPS (In/Out) |  |
| Operating Temperature | NEBS Compliant |  |  |
| Power | < 75 W |  |  |
| Clocking Options | Low-Jitter, configurable clock ranging from10MHz to 750MHz 1 PPS input and output with assembly option for OCXO and TCXO |  |  |

10

11

- 2.2.3.2.3.2	Hardware Accelerator Firmware Features

- Table 2.2.3-8 Hardware Accelerator Firmware Features

| Item Name | Description |
| --- | --- |
| Remote System Upgrade | Securely upgradable FPGA flash image |
| L1 Acceleration | 5G NR LDPC encoding/decoding with interleaving/de- interleaving and rate-matching/rate-de-matching along with early termination, CRC attachment, and HARQ management |
| Hardware-Software APIs | Fully compliant with DPDK/BBDEV APIs |
| Fronthaul Protocols/Compression | Fully compliant with O-RAN WG4 M, C,U & S-planes Specifications |
| Open Programmable Acceleration Environment | Support for: - FPGA flashing upgrade - Firmware version reporting - PCIe diagnostics - Ethernet diagnostics - Temperature and voltage telemetry information |

3

- The hardware accelerator supports x86 or non-x86-based processors. Figure 2.2.3-4 illustrates the two-chip

- acceleration architecture comprising two FPGAs with multi-lane PCIe interfaces toward the CPU and external

- connectivity toward O-RU7-2(s) via eCPRI and O-CU(s) through GbE connectivity. The example architecture

- further depicts multi-lane Gen3 PCIe interfaces between each FPGA and the CPU. The FPGAs communicate

- through high-bandwidth Ethernet (GbE) transport.

9

10

- Figure 2.2.3-4 Dual-chip FPGA-based Hardware Acceleration in O-DU7-2

1

  - 2.2.3.2.4 Accelerator Design Solution 4

  - The O-DU7-2 system is typically implemented using a multi-core processor (CPU) and one or more hardware

  - accelerators. Parts of O-DU7-2 protocol stack can be implemented in software running on the multi-core

  - processors and the computationally intensive L1 and L2 functions can be offloaded to an FPGA-based

  - hardware accelerator. This accelerator comprises a single FPGA for L1 offload in lookaside mode. The

  - accelerator hardware and firmware features are listed in Table 2.2.3-9 and Table 2.2.3-10.

8

- 2.2.3.2.4.1	Accelerator Hardware Features

- Table 2.2.3-9 Accelerator Hardware Features

| Item Name | Description |
| --- | --- |
| SoC Resources | System Logic cells - 930K CLB LUT - 425K SDFEC -8 DSP Slices - 4,272 BRAM - 38.0Mb URAM - 22.5Mb |
| Form Factor | HHHL PCIe Form Factor |
| PCIe Interface | Gen 3 x16 (Gen4 x8) |
| On Board Memory | Total Capacity 4 GB in Programmable Logic, upgradeable to 8GB Total Capacity 2 GB in Processor Subsystem, upgradeable to 4GB |
| Other External Interface(s) | Micro USB for JTAG support (FPGA programming and debug) and access to BMC |
| Graphical User interface | GUI for monitoring the basic board parameters, monitoring temperature alerts, firmware upgrades for BMC |
| Board Management Controller | Telemetry, Security, Remote Upgrade |
| Operating Temperature | NEBS Compliant |
| Power | < 35 W |

11

  - 2.2.3.2.4.2	Hardware Accelerator Firmware Features

  - Table 2.2.3-10 Hardware Accelerator Firmware Features

| Item Name | Description |
| --- | --- |
| Remote System Upgrade | Securely upgradable FPGA flash image |
| L1 Acceleration | 5G NR LDPC encoding/decoding with interleaving/de-interleaving and rate-matching/rate-de-matching along with early termination, CRC attachment, and HARQ management |
| Hardware-Software APIs | Fully compliant with DPDK/BBDEV APIs |
| Open Programmable Acceleration Environment | Support for: - FPGA flashing upgrade - Firmware version reporting - PCIe diagnostics - Ethernet diagnostics - Temperature and voltage telemetry information |

3

- The hardware accelerator supports x86 or non-x86-based processors. Figure 2.2.3-5 illustrates the single-

- chip acceleration architecture comprising one FPGA with Gen3 x16 or Gen4 x8 PCIe interfaces toward the

- CPU.

7

FPGA

(L1 Offload)

X86 or ARM-based

CPU

(L2 and Partial L1)

PCIe Interface

PCIe Gen3x16 or Gen4 x8 100G

F1 Interface

Ethernet (10/25G)

8

- Figure 2.2.3-5 Single-chip FPGA-based Hardware Acceleration in O-DU7-2

10

- 2.2.3.2.5 Accelerator Design Solution 5

- This design includes an Embedded NIC Inline Accelerator (ENIAC) with an ASIC that is capable of providing

- NIC functions. One option supports a high-capacity O-DU using multiple PCIe cards in standard racks and

- aligns with the FAPI interface to maximize software reuse. The second option supports a high-capacity O-DU

- using an Ethernet-based backplane interconnect and aligns with the nFAPI interface to maximize software

- reuse. ENIAC unloads L1 from the host, allowing it to focus on L2.

1

- 2.2.3.2.5.1	Accelerator Requirements

- Hardware and firmware requirements for this accelerator design are given in Table 2.2.3-11 and Table 2.2.3-12,

- respectively.

- Table 2.2.3-11 Accelerator Hardware Feature List]

| Item Name | Description |
| --- | --- |
| PCIe (Interface with digital processing unit） | Gen4 x16 (and lower) |
| Form factor | FHHL |
| Connectivity | QSFP28/SFP |
| NIC Device | 100Gb xHAUL for FH, BH & MH traffic shaping. |
| DDR Main | Min 4 GB DDR4 |
| Flash (FPGA images) | >=1 Gbit |
| BMC | Telemetry, Security, remote upgrade |
| Clocking | For O-RAN C1, C2, C3 & C4 |
| Fronthaul | eCPRI, RoE IEEE1914.3, O-RAN WG4 |
| GPS | SMA for 1 PPS & 10MHz (in/out) |
| Operating Temperature (ambient) | NEBS Compliant |
| Power | <75W |
| Clock Accuracy | Low-Jitter, configurable clock ranging from 10MHz to 750MHz. Option for OCXO (TCXO as standard) |

6

7	Table 2.2.3-12 Accelerator Firmware Feature List

| Item Name | Description |
| --- | --- |
| Remote system upgrade | Securely upgradable image |
| Queuing | Command queuing at FAPI interface |
| LDPC Acceleration | Rel15 compliant |
| Fronthaul Compression | In-line compression/decompression for Mu-Law, block-floating point and quantization according to the O-RAN WG4 specification. |
| Open programmable acceleration environment | Support for: - Flashing upgrade - Firmware version reporting - PCIe diagnostics - Ethernet diagnostics - Temperature and voltage telemetry information. |

8

- 2.2.3.2.5.2	Accelerator Design

- Figure 2.2.3-6 illustrates Accelerator Design 5 with integrated NIC and PCIe connectivity to L2. Figure 2.2.3-7

- illustrates Accelerator Design 5 with integrated NIC and Ethernet connectivity to L2.

L1 Upper Phy Accelerator

8 Layer MIMO

Embedded NIC with eCPRI

L2

Processing

Timing

FAPI on

PCIe4x16

O-RAN 7-2x eCPRI FH QSFP28

PPS

  - 10 MHz

  - Figure 2.2.3-6 Accelerator Design Using PCIe

3

100 Gbps Ethernet SyncE

L1 Upper Phy Accelerator

8 Layer MIMO

Embedded NIC with eCPRI

L2

Processing

Timing

FAPI on

Ethernet

O-RAN 7-2x eCPRI FH QSFP28

PPS

4	10 MHz

100 Gbps Ethernet SyncE

5	Figure 2.2.3-7 Accelerator Design Using Ethernet

6

#### 2.2.3.3 Baseboard Management Controller

- BMC is used to perform hardware power control (power on, power off and power cycle), monitor hardware

- status (temperatures, voltages, etc), monitor Basic I/O System (BIOS)/ Unified Extensible Firmware Interface

- (UEFI) firmware status, and log system events. It provides remote access via shared or dedicated NIC. System

- user can do console access via serial or physical/Kernel-based Virtual Machine (KVM). The BMC has

- dedicated RAM and flash memory. It provides access via serial port or Ethernet port. Figure 2.2.2-1 describes

- the BMC connections with related components.

### 2.2.4 Synchronization and Timing

#### 2.2.4.1 Synchronization and Timing Design 1

- This section describes the synchronization and timing mechanism that is used in the O-DU7-2.

#### 2.2.4.2 Hardware Requirements

- The O-DU7-2 shall support following timing synchronization methods:

- 1. GNSS Synchronization

- 2. Ethernet based IEEE1588V2 Synchronization

- 3. GNSS and IEEE1588 switching

#### 2.2.4.3 Hardware Design

- Depending on the timing distribution topologies used, the O-DU7-2 system clock is able to synchronize with

- the Grand Master Clock (GMC) using IEEE1588 via either the front haul NIC or backhaul NIC or

- synchronizing timing using Global Navigation Satellite System (GNSS). In the case of IEEE1588, the Physical

- Hardware Clock (PHC) within the NIC is synchronized with the GMC first, then the O-DU7-2 system clock is

- synchronized with the PHC. The O-DU7-2 is also capable to provide a clock to the O-RU7-2 via front haul if

- needed. When GNSS becomes available to O-DU7-2, it will be able to synchronize the system clock to

- Coordinated Universal Time (UTC). Figure 2 7 outlines the O-DU7-2 timing synchronization mechanisms.

17

18	Figure 2.2.4-1 O-DU Timing Synchronization

19

### 2.2.5 External Interface Ports

- The external interfaces of O-DU7-2 are described below.

#### 2.2.5.1 Hardware Requirements

- The following table shows the external ports or slots that the system provides.

1	Table 2.2.5-1 External Port List

| Item Name | Description |
| --- | --- |
| Ethernet | Octave Gigabit Ethernet LAN connectors Dual 10GbE Base-T Ethernet connectors Dual 10GbE SFP+ Faber Ethernet connectors 2x100G QSFP28 or 2x25G SFP28 |
| USB | 2 USB 3.0 ports |
| Serial Port | 1 COM port via RJ45 |
| Antenna port | 1 SMA connector for GNSS |

2

#### 2.2.5.2 Hardware Design

  - The digital processing unit is a SoC device which provides the external ports described in the hardware

  - requirement section. The system includes 2 USB 3.0 ports, and the serial RS232 port that can be used for

  - Console Redirection, e.g. Out-of-Band Management. The system provides eight 1Gbps and four 10Gbps

  - Ethernet ports. There are two or four 25G eCPRI ports in system depends on the accelerator card used. The

  - system also provides a RF interface to connect GNSS antenna. The following diagram outlines the external

  - interfaces that supported.

10

11	Figure 2.2.5-1 External Interface Reference Design

12

### 2.2.6 O-DU Firmware

- BIOS and BMC firmware are needed in the system and shall be installed.

### 2.2.7 Mechanical

- The mechanical design for mother board, chassis, and cooling are listed in this section.

#### 2.2.7.1 Mother Board

- The mechanical layout of the mother board shows the location of major components and interface ports. The

- following diagram also provides the dimension of the board.

1

2

3	.

#### 2.2.7.2 Chassis

Figure 2.2.7-1 Mother Board Layout Diagram

- The 1U rack mount chassis contains the layout of the power supply, Solid State Drive (SSD) and fans. The

- chassis dimension is showed in following figure.

1

2

3

#### 2.2.7.3 Cooling

Figure 2.2.7-2 Chassis Mechanical Diagram

- The system installs 4x 40x28mm PWM fans for the cooling. Up to 6 fans can be installed if needed.

6

### 2.2.8 Power Unit

- In a fully loaded system with two PCIe slots populated with 75W each, the system power consumption should

- be less than 400W. The total system power requirement shall be kept less than 80% of the power supply

- capacity.

#### 2.2.8.1 Hardware Requirements

- The O-DU requires a 500W high-efficiency power supply with Power Management Bus (PMBus) 1.2

- capability. The required power input range and output power rails are listed in the table below.

- Table 2.2.8-1 Power Supply Rail Requirements

| Item Name | Description |
| --- | --- |
| AC Input | 100-240V, 50-60Hz, 6.6A max |
| DC Output | +3.3V: 12A +5V: 15A +5V standby: 3A +12V: 41A -12V: 0.2A |

15

#### 2.2.8.2 Hardware Design

  - The O-DU7-2 chassis design includes one 500W power supply unit. The power supply unit is auto-switching

  - capable, which enables it to automatically sense and operate at a 100v to 240v input voltage. The power supply

  - unit features are listed in the following table.

  - Table 2.2.8-2 Power Supply Unit Feature List

| Item Name | Description |
| --- | --- |
| Output connectors | 24pin/8pin/4+4pin/HDD/I2C |
| Dimension (W x D x H) | 3.9 x 7.1 x 1.6 inch |
| Maximum Output Power | +3.3V: 12A +5V: 15A +12V: 41A -12V: 0.2A +5Vsb: 3A |
| Rated Input Voltage/Current | 100-240Vac / 6.6A max |
| Rated Input Frequency | 50-60HZ |
| Inrush current | Less than 30A |

6

### 2.2.9 Thermal

- Active cooling with up to 6 fans is integrated in the chassis.

- The hardware acceleration cards described in 2.2.3.2 use passive cooling and a custom heatsink and is equipped

- with temperature sensors. It is designed to operate in temperatures ranging from -5°C to +55°C.

### 2.2.10 Environmental and Regulations

- The O-DU7-2 hardware system is RoHS Compliant. The power supply unit is EMC FCC/CISPR Class B

- compliant. Table 2 10 lists the environmental related features and parameters.

- Table 2.2.10-1 Environmental Features

| Item Name | Description |
| --- | --- |
| Operating Temperature | -5°C to +55°C |
| Non-operating Temperature | -40°C to 70°C |
| Operating Relative Humidity | 8% to 90% (non-condensing) |
| Non-operating Relative Humidity | 5% to 95% (non-condensing) |

15

- The hardware accelerator described in 2.2.3.2 is designed to operate in indoor environments and in

- temperatures ranging from -5°C to +55°C.

## 2.3 O-RU7-2 Hardware Reference Design

- This chapter defines the hardware reference design for the Outdoor Microcell O-RU7-2. This is defined by a

- 7-2 interface to the O-DU on one side and up to 8T8R on the antenna port. Because this design is up to 10W

- per channel of conducted power, the external power is supplied by either AC or a standard 48V telecom source.

### 2.3.1 O-RU7-2 High-Level Functional Block Diagram

- Figure 2.3.1-1 provides a high-level functional block diagram depicting the major HW/SW components. It

- also highlights the internal/external interfaces that are required. This document shows how to implement the

- system defined by the OMC-HAR [6] document.

O-RU7-2

To/From

O-DU7-2

O-RAN

FH Interface

O-RAN FH

Processing Unit

Digital Processing Unit

To/From Antenna

GNSS

(optional)

RF

Processing Unit

Local timing from CDR

Local timing from GNSS or equivalent

#### Timing Unit

Power Unit

9

  - Figure 2.3.1-1 High-Level Functional Block Diagram

11

### 12	2.3.2 O-RU7-2 Hardware Components

13	Figure 2.3.2-1 shows a 4T4R implementation and Figure 2.3.2-2 shows an 8T8R implementation. In each of

14	these diagrams, the Digital Processing Unit is further detailed in 2.2.3.1. Items under the umbrella of RF

15	Processing Unit, including the Transceiver, RFFE and other RF items are reviewed in additional detail in

16	2.3.2.2. Clock and Synchronization are reviewed in 2.3.2.3. The Power Unit is detailed in 2.3.6.

TRx

Reference

Tx

enable

Rx

enable

bypass

enable

Rx0

RFFE

Tx0

enable

PA

SERDES

Digital Processing Unit

ORx0

ORx1

SPI

Tx1

enable

PA

RFFE

Network

Clock

SPI

Clean

Clock

Rx1

bypass

enable

Clock

Transceiver

Port A

bypass

enable

Rx0

RFFE

Tx0

enable

PA

Synchronizer

ORx0

ORx1

Tx1

enable

PA

RFFE

bypass

enable

Power Unit

Rx1

2

3	Figure 2.3.2-1 4T4R General Block Diagram with full RF front end

TRx

Reference

TRx

Reference

Tx	Rx

enable	enable

Rx0

bypass	enable

RFFE

Tx0

enable

PA

SERDES

Digital Processing Unit

ORx0

ORx1

SPI

Tx1

enable

PA

RFFE

Rx1

Network	SPI

Clock

Clean

Clock

bypass

enable

Clock

Port A

bypass

enable

Rx0

RFFE

Tx0

PA

enable

ORx0

Synchronizer

ORx1

Tx1

enable

PA

RFFE

bypass

enable

Rx1

Rx0

bypass	enable

RFFE

Tx0

enable

PA

ORx0

ORx1

Tx1

PA

enable

RFFE

Rx1

bypass

enable

Transceiver

Port A

bypass

enable

Rx0

RFFE

Tx0

PA

enable

ORx0

ORx1

Tx1

enable

PA

RFFE

bypass

enable

Rx1

Power Unit

Transceiver

2

3	Figure 2.3.2-2 8T8R General Block Diagram with RF front end

4

- Figure 2.3.2-3 shows an example of an high-level block details of an O-RU Hardware implementation. The

- description of each block from left to right is given below.

- An O-RU7-2 communicate with the O-DU7-2 through the O-RAN Fronthaul interface and the control/data

- packets are processed by the O-RAN Fronthaul unit with the help of the CUS plane processing block.

- Subsequently the packets are processed by the low-PHY functional blocks such as encoding, scrambling,

- modulation, layer mapping, precoding, beamforming, and resource element mapping in the downlink. Exactly

- the reverse operation is done during the uplink process. The output of the low-PHY block is fed to the digital

- processing unit through packet interface. Here the frequency conversion in digital domain i.e., digital up

- conversion (DUC) in DL and digital down conversion (DDC) in UL are done. Furthermore, DPD and CFR

- algorithms are implemented in the digital processing unit to enhance the power amplifier efficiency by

- reducing the PAPR/ACLR in the RF front end. The output of digital processing unit is fed to the RF processing

- unit through the digital interface. The digital signals are converted to analog in the DL and vice versa in the

- UL with the help of DAC and ADC functional blocks in the Transceiver unit, respectively. The signals are

- further amplified with the help of functional blocks like the PA in DL and LNA in UL both of which are part

- of the analog RF front end unit. The output of the power amplifier is fed to cavity filter to suppress the

- unwanted signals in DL and is done before amplification in the case of UL operation i.e., receive operation to

- improve selectivity. The TDD switch is used to switch the operation between DL and UL, i.e., transmit and

- receive operations. The output of the cavity filter is fed to antenna unit and then transmitted over the air. The

- processing units such as FH, Digital, and RF powered from the power unit and clocking and synchronization

- is provided from the timing/synchronization block. In the case of a beamforming scenarios, antenna calibration

- is implemented using the antenna calibration processing block. This coordinates with the low-PHY processing

- unit and calibration switch (Cal switch) and process the signals coming from and going to the antenna

- calibration port to compensate the amplitude and phase offsets existing in each RF chain. Other

- implementations are also possible by rearranging the functional blocks of O-RAN FH, Digital processing, and

- RF processing units.

24

25

26	Figure 2.3.2-3 O-RU Architecture Example Implementation 1

27

#### 2.3.2.1 Digital Processing Unit

- The digital processing unit of the O-RU7-2 performs the fronthaul interface, lower L1, Synchronization, RF

- interface, antenna calibration, digital beamforming and OAM. The following block diagram describes the

- digital processing unit as per example implementation in

1

2

  - Figure 2.3.2-4 Digital Processing Unit Block Diagram (L1 Processing + optional Beamforming)

eCPRI/CPRI

Boot Flash

DDR

OAM

Digital Processing

| CPU |  | DFE |
| --- | --- | --- |
|  | FPGA/ASIC |  |

4

JESD204B/C

DFE

Clock & Control

Clock & Sync

5

6	Figure 2.3.2-5 Digital Processing Unit Block Diagram (DFE)

7

- 2.3.2.1.1 FPGA/ASIC Solution

- This solution for the digital processing unit is based on an FPGA with embedded ARM processor.

- 2.3.2.1.1.1	FPGA Specifications

- Table 2.3.2-2 shows the general interfaces required for an FPGA based design based on Figure 2.3.2-4 and

- Figure 2.3.2-5.

1	Table 2.3.2-1 FPGA Interface Specifications

| Item Name | Description |
| --- | --- |
| eCPRI Port | High speed bi-directional interface transporting data and control information. Total payload will be determined by the antennas/layers served, bandwidth supported, bit precision, subcarrier spacing, etc. It is typically served by one or more QSFP+ or SFP28 connectors depending on the payload. |
| SERDES | Up to 16 high speed JESD204B/C serial interface lanes between FPGA fabric and transceiver depending on transceiver configuration selected. |
| Boot Clock | Clock for any uC or fabric functions required immediately at power on |
| Sysref | JESD204B/C synchronization signal |
| DevCLK | JESD204B/C device clock |
| ARM Clock | Clock for the ARM processor |
| Memory Clock | Memory clock |
| OAM Port | Control interface used for maintenance |
| DDR | Interface for external DDR memory |

2

3	2.3.2.1.1.2	FPGA Design

  - Table 2.3.2-2 shows the general blocks required for the design. The core elements implemented in this design

  - are required to be sized to support the requirements shown in the OMC-HAR [6] document. Specifically, the

  - DPU should support up to 200 MHz of occupied BW with up to two 100 MHz component carriers. In addition

  - to signal processing, the DPU should support C, U, S and M plane handling. Major processing blocks in the

  - DPU are shown in Table 2.3.2-2.

1	Table 2.3.2-2 FPGA Functional Blocks

| Item Name | Description | Cat A | Cat B |
| --- | --- | --- | --- |
| FFT & CP- | FFT and Cyclic Prefix removal for uplink processing | Required | Required |
| iFFT & CP+ | iFFT and Cyclic Prefix insertion for downlink processing | Required | Required |
| PRACH Front End | PRACH front end processor | Required | Required |
| Channel DDC | Component Carrier Digital Down Converter | Optional | Optional |
| Channel DUC | Component Carrier Digital Up Converter | Optional | Optional |
| JESD204B/C | JEDEC High speed serial interface type B or C framer/de-framer | Required | Required |
| CFR | Crest Factor Reduction | Optional | Optional |
| DPD | Digital Pre-Distortion | Optional | Optional |
| C/U/M Plane | Control, User and management Plane processing | Required | Required |
| 1588v2 Stack | IEEE 1588v2 Synchronization Software Stack | Required | Required |
| eCPRI | evolved Common Public Radio Interface framer/de-framer | Required | Required |
| I/Q Compression | I/Q compression/decompression for fronthaul lane rate reduction | Optional | Optional |
| Beamforming | Create & steer several beams | Optional | Optional |
| Pre-coding | Pre-coding | Optional | Required |
| RE Mapping | Resource Element Mapping to Antenna port | Optional | Required |

2

#### 2.3.2.2 RF Processing Unit

- The RF Processing Unit of O-RU7-2 includes the power amplifier (PA), low noise amplifier (LNA), Tx/Rx

- filters and conversion between analog and digital domain. Physical and logical partitioning within the RF

- processing unit need not occur at any specific boundary.

- 2.3.2.2.1 Transceiver Reference Design

- For the O-RU7-2 the sampling function and frequency conversion function is performed by transceiver. The

- purpose of the transceiver is to translate the RF signal to digital as early as possible in the signal chain and to

- reduce the data rate as low as possible to save power. The Transceiver is to convert between high-speed

- baseband data and a low-level RF for both transmit and receive signal chain.  The transceiver may be

- responsible for orchestration of external gain control elements in both the receive and transmit paths. Other

- radio functions may need to be controlled by the FPGA/ASIC where they need to be control fast and

- synchronized with the TDD frame structure. Such elements include the transmit / receive switch, PA and LNA

- enable.

- 2.3.2.2.1.1	Hardware Specifications

- 2.3.2.2.1.1.1  Interface

- Table 2.3.2-3 RF Processing Unit Interface Specifications

| Item Name | Description |
| --- | --- |
| High Speed Data (SERDES Out, SERDES In) | High speed data represents the baseband information being transmitted or received. Depending on the configuration of the ORAN device, various bandwidths may be supported leading to a range of payload |

|  | rates. Options for data include JESD204B and JESD204C. Up to 8 lanes in each direction may be supported although fewer is preferred. Options such as DPD and numeric precision will impact the payload rate. Several options are shown in the following table. All data represents IQ 16-bit (N=16) precision. Some devices support IQ 12 bit (N’=12) which may reduce the required data rates accordingly. |
| --- | --- |
| Reference Clock (REF Clock In) | The transceiver should receive a reference for internal clock and LO synthesis needs. This reference clock can function as the JESD204 Device Clock where the interface is by SERDES. The specific clock frequency is determined by the operation mode of the transceiver. |
| SYSREF | If the transceiver supports SERDES, then it should accept a SYSREF signal from the clock or data source as appropriate. The number and configuration for the SYSREF is dependent on the operating mode of the transceiver. |
| SYNC\ | If the transceiver supports SERDES, then it should also support a SYNCB for each link as appropriate. |
| Control (SCLK, CSB, SDO, SDIO, etc) | Control of the transceiver is by way of 3 or 4 wire SPI or I2C functioning as a slave. Support for 1.8V control is required and tolerance of 3.3V is preferred. The transceiver may optionally include a separate SPI master for control of peripheral devices as required. |
| GPIO (GPIO 1 – GPIO #) | The transceiver may optionally include GPIO for controlling peripherals including but not limited to PAs, LNAs and other devices. These GPIOs should at a minimum support 1.8V outputs but the specifics will be determined by the connected devices. The GPIO should also support input from peripheral devices. Input should at a minimum support 1.8V logic with tolerance of 3.3V preferred. |
| Tx Enable | The transceiver should provide an output to support enabling and disabling the external devices in the transmit chain such as a TxVGA (optional) and PA. This may be part of the GPIO pins. |
| Rx Enable | The transceiver should provide an output to support enabling and disabling the external devices in the transmit chain such as a RF Front End Module or LNA. This may be part of the GPIO pins. |
| LNA Bypass | The transceiver should provide an output to support bypassing the LNA appropriately in the condition of a large blocker if so required. This may be part of the GPIO pins. |
| RF Outputs (Tx1 – Tx4) | RF outputs including the main Tx signal should support 50 ohm or 100 ohms signalling. These outputs can be either single ended or differential. |
| RF Inputs (Rx1 – Rx4 & ORx 1 – ORx4) | RF inputs including the main Rx and the Observation Rx (ORx) (for DPD) should support 50 ohm or 100 ohms signalling. These inputs can be either single ended or differential. The device should support at least 1 ORx. |

1

  - Table 2.3.2-4 JESD204B/C Serial Data Rates

| Bandwidth | JESD204B | JESD204C | JESD204B | JESD204C |
| --- | --- | --- | --- | --- |
|  | 4T4R |  | 8T8R |  |
| 20 | 4.9152 | 4.05504 | 9.8304 | 8.11008 |
| 50 | 9.8304 | 8.11008 | 19.6608 | 16.22016 |
| 100 | 19.6608 | 16.22016 | 39.3216 | 32.44032 |
| 200 | 39.3216 | 32.44032 | 78.6432 | 68.88064 |

2

3	2.3.2.2.1.1.2  Algorithm

- The transceiver is required to provide appropriate algorithms to sustain RF operation including but not limited

- to Rx AGC, Tx Power control, DPD and CFR.

- 2.3.2.2.1.1.3  Device Configuration

- The transceiver should support either 4T4R or 8T8R. In addition, at least one ORx path should be provided

- to support DPD functionality.

- 2.3.2.2.1.1.4  Power Dissipation

- Total dissipation of the TRx should be less than 6W for 4T4R.

- 2.3.2.2.1.1.5  RF Specifications

- Table 2.3.2-5 Transceiver RF Specifications

| Parameter | Symbol | Min | Typ | Max | Unit | Test Condition/Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitters |  |  |  |  |  |  |
| Center Frequency |  | 650 |  | 6000 | MHz |  |
| Transmitter Synthesis Bandwidth |  |  |  | 450 | MHz |  |
| Transmitter Large Signal Bandwidth |  |  |  | 200 | MHz |  |
| Transmitter Attenuator Power Control Range |  | 0 |  | 32 | dB | Signal to noise ratio (SNR) maintained for attenuation between 0 and 20 dB |
| Transmitter Attenuation Power Control Resolution |  |  | 0.05 |  | dB | 20 MHz LTE/NR at -12 dBFS |
| Adjacent Channel Leakage Ratio (ACLR) |  |  | -66 |  | dB | 75 MHz <f≤2800 MHz |
| In Band Noise Floor |  |  | -154.5 |  | dBm/Hz | 0 dB attenuation; in band noise falls 1 dB for each dB of attenuation for attenuation between 0 dB and 20 dB |
| Out of Band Noise Floor |  |  | -153 |  | dBm/Hz | 600 MHz < f ≤ 3000 MHz |

| Parameter | Symbol | Min | Typ | Max | Unit | Test Condition/Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Output Power | OIP3 |  | 6 |  | dBm | 0 dB attenuation; 3 × bandwidth/2 offset |
| Third Order Output Intermodulation Intercept Point |  |  | 27 |  | dBm | 600 MHz < f ≤ 3000 MHz |
| Error Vector Magnitude (3GPP Test Signals) | EVM |  |  |  |  |  |
| 1900 MHz LO |  |  | 0.6 |  | % | 50 kHz RF PLL loop bandwidth |
| 3800 MHz LO |  |  | 0.53 |  | % | 300 kHz RF PLL loop bandwidth |
| Observation Receivers | ORx |  |  |  |  |  |
| Center Frequency |  | 450 |  | 6000 | MHz |  |
| Gain Range |  |  | 30 |  | dB | IIP3 improves dB for dB for the first 18 dB of gain attenuation; QEC performance optimized for 0 dB to 6 dB of attenuation only. |
| Analog Gain Step |  |  | 0.5 |  | dB | For attenuation steps from 0 dB to 6 dB |
| Receiver Bandwidth |  |  |  | 450 | MHz |  |
| Maximum Usable Input Level | PHIGH |  | -11 |  | dBm | 0 dB attenuation; increases dB for dB with attenuation. |
| Integrated Noise |  |  | -58.7 |  | dBFS | 450 MHz Integration bandwidth |
| Third Order Input Intermodulation Intercept Point | IIP3 |  | -57.5 |  | dBFS | 491.52 MHz Integration bandwidth |
| Narrow Band |  |  |  |  |  | Maximum observation receiver gain; test condition: PHIGH-11 dB/tone |
| 1900 MHz |  |  | 15 |  | dBm |  |
| 2600 MHz |  |  | 16.5 |  | dBm |  |
| 3800 MHz |  |  | 18 |  | dBm |  |
| Wide Band |  |  |  |  |  | IM3 products>130 MHz at baseband; test condition: PHIGH - 11 dB/tone; 491.52 MSPS |
| 1900 MHz |  |  | 13 |  | dBm |  |
| 2600 MHz |  |  | 11 |  | dBm |  |
| 3800 MHz |  |  | 13 |  | dBm |  |
| Third Order Intermodulation Product | IM3 |  | -70 |  | dBc | 600 MHz < f ≤ 3000 MHz |

| Parameter | Symbol | Min | Typ | Max | Unit | Test Condition/Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Spurious Free Dynamic Range | SFDR |  | 64 |  | dB | Non IMx related spurs, does not include HDx; (PHIGH − 11) dB input signal |
| Harmonic Distortion |  |  |  |  |  | (PHIGH − 11) dB input signal |
| Second Order Harmonic Distortion Product | HD2 |  | -80 |  | dBc | In band HD falls within ±100 MHz |
|  |  |  | -73 |  | dBc | Out of band HD falls within ±225 MHz |
| Third Order Harmonic Distortion Product | HD3 |  | -70 |  | dBc | In band HD falls within ±100 MHz |
|  |  |  | -65 |  | dBc | Out of band HD falls within ±225 MHz |
| Receivers |  |  |  |  |  |  |
| Center Frequency |  |  |  | 6000 | MHz |  |
| Analog Gain Step |  |  | 30 |  | dB | Attenuator steps from 0 dB to 6 dB |
|  |  |  | 1 |  | dB | Attenuator steps from 6 dB to 30 dB |
| Receiver bandwidth |  |  |  | 200 | MHz |  |
| Maximum Usable Input Level | PHIGH |  |  |  |  | 0 dB attenuation; increase dB for dB with attenuation; continuous wave = 1800 MHz; corresponds to -1 dBFS at ADC |
|  |  |  | -11 |  | dBm | 75 MHz <f≤3000 MHz |
| Noise Figure | NF |  |  |  |  | 0 dB attenuation at receiver port |
|  |  |  | 12 |  | dB | 600 MHz <f≤3000 MHz |
| Input Third Order Intercept Point | IIP3 |  |  |  |  |  |
| Difference Product | IIP3, d |  |  |  |  | Two (PHIGH − 9) dB tones near band edge |
| 2600 MHz (Wideband) |  |  | 17 |  | dBm |  |
| 2600 MHz (Midband) |  |  | 21 |  | dBm |  |
| Sum Product | IIP3, s |  |  |  |  | Two (PHIGH − 9) dB tones, at bandwidth/6 offset from the LO |
| 2600 MHz (Wideband) |  |  | 20 |  | dBm |  |
| HD3 | HD3 |  |  |  |  | (PHIGH − 6) dB continuous wave tone at bandwidth/6 offset from the LO |
|  |  |  | -66 |  | dBc | 600 MHz < f ≤ 4800 MHz |

1

  - 2.3.2.2.1.2	Hardware Design

  - For the O-RU7-2 the sampling function and frequency conversion function can be performed by the transceiver.

  - The transceiver integrates the ADC, DAC, LO, down converter, up converter and related functions. The block

  - diagram of transceiver design is shown in Figure 2.3.2-6.

6

SPI Port

SCLK CSB SDO SDIO

CPIO1

GPIO2 GPIO3

Receiver Block 4

Receiver Block 3

Receiver Block 2

Receiver Block 1

GPIO#

Rx3

Rx2

Rx1

ADC

D

D

Transmit Block 4

Transmit Block 3

Transmit Block 2

Transmit Block 1

SYNC\ IN

Tx3

Tx2

Tx1

JESD204B/C

Serial Interface

DAC

D

D

Interpolation pFIR

Tuning

Device Management

SERDES

Out

SERDES

In

SYNC\ OUT

Observation

Receiver

ADC

Decimation pFIR AGC

Tuning

RSSI

Overload

Device Management

DPD Engine

Decimation pFIR AGC

Tuning

RSSI

Overload

Device Management

Control

Interface

Clock,

Synchronization

& Synthesis

REF Clock

SIYn SRE

Ext LO/ClFock

In

Decimation pFIR AGC

Tuning RSSI

Overload

evice Management

ADC

Decimation pFIR AGC

Tuning

RSSI

Overload

evice Management

ADC

Decimation pFIR AGC

Tuning

RSSI

Overload

evice Management

ADC

D

Rx4

Interpolation

pFIR

Tuning

evice Management

DAC

Interpolation

pFIR

Tuning

evice Management

DAC

Interpolation pFIR

Tuning

evice Management

DAC	D

Tx4

ORx1 ORx2

ORx3 ORx4

7

8	Figure 2.3.2-6 Transceiver with Integrated CFR and DPD

9

- 2.3.2.2.2 Power Amplifier (PA) Reference Design

- 2.3.2.2.2.1	Hardware Specifications

- 2.3.2.2.2.1.1  Interface

- Table 2.3.2-6 Power Amplifier Interface Specifications

| Item Name | Description |
| --- | --- |
| RF Enable | The enable input should be compatible with 1.8V logic and tolerate 3.3V as required. A logic high enables the PA. A logic low disables the device and places it in a minimum dissipation mode. |
| RF Output | RF outputs support 50-ohm single ended to properly interface to a directional coupler, isolator, switch or antenna. |
| RF Input | RF inputs should support 50-ohm, single ended match to the transceiver output or preamp. |

- 2.3.2.2.2.1.2  Power Specifications

- RF output power required is nominally 10 W (40 dBm). DC power is driven by efficiency of the PA and

- should not be more than 27W and the efficiency should be greater than 37%.

- 2.3.2.2.2.1.3  RF Specifications

- The PA and driver should have enough gain to boost the transceiver output to the rated power level. The

- output power should be at least 10W (40 dBm) including the loss of the circulator and antenna filter. The

- ACLR with DPD and CFR should be better than 47dBc according to the related 3GPP test models.

- Table 2.3.2-7 Power Amplifier RF Specifications

| Item Name | n 77, 48 | n 41 |
| --- | --- | --- |
| Gain | >60 dB including driver | To be provided in next version |
| P3dB | 51 dBm |  |
| Input Return Loss | <-15 dB |  |
| Output Return Loss | <-15 dB |  |
| Switching Speed | <1uS |  |
| HD2 | >28 dBc |  |
| HD3 | >30 dBc |  |

- 2.3.2.2.2.2	Hardware Design

- RFin is the input of the PA, RFout is the output of the PA. Vdd and Vbias is the power input of the PA. PAenable

- is the control pin to disable or enable the PA. The input and output should be matched to 50 ohms to optimize

- return loss as much as possible. Proper decoupling shall be provided to minimize impact of supply ripple on

- the RF output.

RFen

RFout

RFin

Vbias

Vdd

1

2	Figure 2.3.2-7 Power Amplifier Reference Design

3

- 2.3.2.2.3 Low Noise Amplifier (LNA) Reference Design

- 2.3.2.2.3.1.1  Interface

- Table 2.3.2-8 LNA Interfaces

| Item Name | Description |
| --- | --- |
| Enable | The enable input should be compatible with 1.8V logic and tolerate 3.3V as required. A logic high enables the LNA. A logic low disables the device and places it in a minimum dissipation mode. |
| LNA Bypass | The LNA Bypass skips the second stage LNA effectively reducing the overall gain. This would be selected when the input signal compromises the linearity of the amplifier. |
| RF Out | RF outputs support 50-ohm single ended to properly interface to the RF filtering and Rx input port of transceiver. |
| TR Switch | The TR switch is used to shunt any reflected RF power that reaches the LNA to a shunt load during the transmit period. |
| RF Input | RF inputs should support 50-ohm, single ended match to circulator. |
| Termination | This RF output port shunts any reflected transmit power to an appropriate load during the Transmit period to protect the input of the LNA. |

7

- 2.3.2.2.3.1.2  Power Specifications

- DC power per channel should not exceed 0.5W.

- 2.3.2.2.3.1.3  RF Performance Specifications

- Table 2.3.2-9 LNA RF Specifications

| Item Name | n 77, 48 | n 41 |
| --- | --- | --- |
| Gain (High/Low) | 32/16 dB | To be provided in next version |
| NF (High/Low) | 1.5/1.5 dB |  |
| OIP3 (High/Low) | 31/31 dBm |  |
| OP1dB (High/Low) | 18/13 dBm |  |
| TR Switch Time | <1uS |  |

3

- 2.3.2.2.3.2	Hardware Design

- Figure 2.3.2-8 shows the configuration for a dual-stage LNA. The RF inputs and outputs should be properly

- matched to optimize the noise performance and flatness of the amplifier. The input switch is used to forward

- any reflected transmit energy to an external load during the transmit period to protect the LNA from damage.

- The DC supply voltage, VCC, should be properly filtered to prevent power supply noise from entering the

- LNA and reducing performance.

VCC

RFout

Terminationext

RFin

LNAEN

TR

LNAbypass

10

11	Figure 2.3.2-8 LNA Reference Design

12

- 2.3.2.2.4 Circulator Reference Design

- For TDD use, the Tx and Rx links work in time duplex. To properly couple the Tx energy to the antenna and

- the Rx signal to the LNA input, a circulator is used to perform this function given the potentially high RF

- power being processed through this device.	The circulator can be configured as a clockwise or

- counterclockwise device depending on the layout details.

- 2.3.2.2.4.1	Hardware Specifications

- 2.3.2.2.4.1.1  Interface

- The circulator includes 3 ports that can function as in input or an output. If the device is labelled clockwise,

- then for each input, its nominal output is the next pin rotating right around the device. If it is counterclockwise,

- the nominal output is the next pin rotating to the left around the device.

1	2.3.2.2.4.1.2  RF Specifications

2

Table 2.3.2-10 Circulator Specifications

| Item Name | n 77, 48 | n 41 |
| --- | --- | --- |
| Rotation | CW or CCW | To be provided in next version |
| Average Power | Up to 46 dBm |  |
| Peak Power | 52 dBm |  |
| Impedance | 50 ohms |  |
| Insertion Loss | -0.3 dB |  |
| Isolation | 20 dB |  |
| Return Loss | 22 dB |  |

3

- 2.3.2.2.4.2	Hardware Design

- The example on the left in Figure 2.3.2-9 is a clockwise oriented device. The RF input enters pin 1 and exits

- pin 2. Pin 2 can also be used as an input and its output is pin 3. Finally pin 3 can also function as an input

- with pin 1 being the output. The device on the right is counterclockwise oriented and the signal flow is the in

- the opposite direction.

1

3

2

1

3

2

9

- Figure 2.3.2-9 Circulator Reference Design

11

- 2.3.2.2.5 Antenna / Phased Array Reference Design

- The antenna is a transducer, and it is used to convert electrical signal to the Electromagnetic waves during the

- transmit mode and vice versa in the reception mode. radiate the TX power and to receive the RX signal. Based

- on the radiation pattern characteristics of the antenna, it is broadly classified as omni-directional and directional

- antennas and examples of such antennas are Two options are listed below for reference.

- 2.3.2.2.5.1	Hardware Specifications for Omnidirectional Antenna

- The following table shows the typical electrical omnidirectional antenna specifications for the O-RU7-2.

1	Table 2.3.2-11 Omnidirectional Antenna Specifications

| Frequency band | n 77 | n 48 | n 41 |
| --- | --- | --- | --- |
| Antenna Type | Omnidirectional | Omnidirectional | To be provided in next version |
| VSWR (max) | 1.5:1 | 1.5:1 |  |
| Power capacity | ≥10 W | ≥10 W |  |
| Gain (max) | 5.5 dBi | 5.5 dBi |  |
| Beam width (vertical) | 35º | 35º |  |
| Polarization | ±45º | ±45º |  |

2

- 2.3.2.2.5.1.1  Hardware Design for Omnidirectional Antenna

- One possible choice of the omnidirectional antenna is a cannister type with multiple ports including the 4

- ports for the 4T4R radio. Conceptual design is shown in the following figure.

6

7	Figure 2.3.2-10 Canister Style Omni Antenna

8

9

- 2.3.2.2.5.2	Hardware Specifications for Directional Antenna

- The following table shows the typical electrical directional antenna specifications for the O-RU7-2.

  - Table 2.3.2-12 Directional Antenna Specifications

| Frequency band | n 77 | n 48 | n 41 |
| --- | --- | --- | --- |
| Antenna Type | Directional | Directional | To be provided in next version |
| Antenna with Embedded RET & AISG[10] Compliant | Antenna with Embedded RET & AISG[10] Compliant | Antenna with Embedded RET & AISG[10] Compliant | Antenna with Embedded RET & AISG[10] Compliant |
| VSWR (max) | 1.5:1 | 1.5:1 | 1.5:1 |
| Power capacity | ≥10 W | ≥10 W | ≥10 W |
| Gain (max) | 16 dBi | 16 dBi | 16 dBi |
| Beam width (horizontal) | 65º | 65º | 65º |
| Beam width (vertical) | 11º | 11º | 11º |
| Polarization | ±45º | ±45º | ±45º |
| Variable Tilt | 0-10 º | 0-10 º | 0-10 º |
| Tilt Accuracy | ±1 º | ±1 º | ±1 º |
| 3rd Order PIM, @2x43 dBm (dBc) | ≤-153 | ≤-153 | ≤-150 |
| Isolation (RF Ports) (dB) | ≥25 | ≥25 | ≥20 |

2

- 2.3.2.2.5.2.1  Hardware Design for Directional Antenna

- One possible choice of directional antennas are the radios mounted on the back of the antenna with 4 ports for

- the 4T4R radio on the back of the antenna just below each radio. Conceptual design is shown in the following

- figure.

7

- Figure 2.3.2-11 Mounting arrangement scenario 1 of 4T4R radios on the back of Directional Antenna

- Another possible choice of directional antennas are the radios located away from the antenna with ports for

- each of the 4T4R radios on the bottom cap of the antenna. Conceptual design is shown in the following figure.

11

1

  - Figure 2.3.2-12 Mounting arrangement scenario 2 of 4T4R radios remotely mounted with Directional Antenna

3

#### 2.3.2.3 Internal Interfaces

- The following sections describe potentially useful optional interfaces within the white box. These optional

- interfaces are not required and may be used at the discrepancy of the manufacturer. This section is provided

- strictly as information only to better clarify the functionality of these interfaces.

- 2.3.2.3.1 Optional JES204 Interfacing

- O-RU solutions requiring chip to chip interfacing of high-speed time domain data may benefit from utilization

- of JEDEC defined standard JESD204 [16]. This interface is specified for chip-to-chip interfacing that provides

- fast, low latency and time aligned data. The benefit of such an interface is to enable an open standard that

- allows for devices from multiple vendors to seamlessly interface. From a logistical perspective JESD204

- allows multiple devices to communicate over the same link in a multiplex fashion while preserving time

- alignment and providing deterministic latency. This makes JESD204 a good candidate interface for carrying

- the signals for multiple antennas, multiple component carriers including I and Q data. This allows for relative

- simplicity requiring only a few differential pairs along with a ‘device clock’ and ‘sysref’ signals to carry and

- synchronize all time aligned signals.

- Figure 2.3.2-13 provides an example of a typical configuration for a JESD204 configuration. The lines labeled

- ‘JESD for UL x2’ and ‘JESD for DL x2’ are the data bearing connections and consist of differential pairs. In

- this example, there are 2 differential pairs for the UL data (receiver path) and 2 differential pairs for the DL

- data (transmit path). These connections provide data using an embedded clock and synchronization with a

- transport layer coding used to ensure sufficient transitions that the embedded clock can be recovered on the

- receiving end.

- The second signal of is labeled ‘Device Clock’. The device clock provides the frequency reference to devices

- in the configuration. Within the scope of JESD204, this signal provides the reference for the interface. In

- practice, the Device Clock also provide the frequency reference to the individual devices and is usually a

- multiple of 30.72 MHz. JESD204 connected devices would use this reference for both the interface as well as

- other frequency services within the device such as sample clocks and other PLLs.

- The final signal is the ‘Sysref’ which enables deterministic latency in the system. Each interconnected device

- uses this signal in conjunction with ‘Device Clock’ to determine exactly when the sample edge should occur.

- There are different ways to configure and use ‘Sysref’ to achieve different goal.	For more details on

- configuration, see [16].

- JESD204 is an evolving standard. Although it has been around for a while, as process technology evolves and

- data needs grow, JESD has evolved to keep pace with throughput needs. The initial variation supported up to

- 3.125 Gbps. JESD204A brough speeds up to 6.25 Gbps. JESD204B brought throughput to 12.5 Gbps. Most

- recently JESD204C offers throughputs up to 32.45 Gbps. Each variety offers at least one option for transport

- layer coding like 8B/10B as well as a range of other features including sub-channel coding which allows

- control channels to be passed along with the primary data. Currently most devices support JESD204B and

- JESD204C. As of this writing, JEDEC is underway defining the next phase of JESD204x which will further

- increase data rates as demanded by increasing bandwidth in wireless and other application.

- Table 2.3.2-13 JESD204 Abridged Lexicon

| Parameter | Description |
| --- | --- |
| M | Number of converters/converter device (link) |
| NP (or N’) | Total number of bits per sample |
| S | Samples transmitted/single converter/frame cycle |
| F | Octets/Frame |
| K | Number of frames per multi-frame |
| SC | Sample Clock Rate |
| L | Number of lanes/converter device (link) |

- To better comprehend the JESD204 standard, understanding the lexicon is important.  There are many

- parameters that can be used, but those in Table 2.3.2-13 provide the core definitions. Most of these are basic

- parameters related to common data converter characteristics and include the number of converters, number of

- bits and sample rate. The remaining parameters are related to the transport layer and physical implementation

- of the interface.

- Combined, the parameters in this lexicon help define the data throughput, number of lanes required as well as

- the framing of data onto the interfaces. Starting with the payload, it is determined by computing the serial data

- rate and adding the transport layer (ie 8B/10B, 64B/66B etc) to that as defined in the following equation:

24	𝑀 × 𝑁𝑃 × 𝑆𝐶 × 𝑇𝑟𝑎𝑛𝑠𝑝𝑜𝑟𝑡 𝐿𝑎𝑦𝑒𝑟 𝐶𝑜𝑑𝑖𝑛𝑔 = 𝑃𝑎𝑦𝑙𝑜𝑎𝑑 𝑅𝑎𝑡𝑒

- This equation provides the total payload to be delivered between devices. This data is delivered over one or

- more lanes. If they payload exceeds the capacity of one lane, additional lanes are added to ensure that the

- throughput per lane matches the capability of the standard. As highlighted in preceding paragraphs, currently

- JESD204B supports up to 12.5 Gbps and JESD204C supports up to 32.45 Gbps but future evolutions will

- include higher lane capacity. Where the Payload Rate exceeds an individual lanes capacity, the total Payload

- Rate is divided by the lane capacity, currently 12.5 Gbps or 32.45 Gbps, to determine the approximate number

- of lanes required. The actual number is the next higher integer to ensure that the lane rate is less than the

- maximum specified or desired. As the standard continues to evolve, this same math will be used to determine

- the payload and number of lanes, L.

3	𝐿 = 𝐶𝑒𝑙𝑖𝑛𝑔 (

𝑃𝑎𝑦𝑙𝑜𝑎𝑑 𝑅𝑎𝑡𝑒

)

𝑀𝑎𝑥 𝐿𝑎𝑛𝑒 𝑅𝑎𝑡𝑒

- The final important term is F, the number of octets per frame. In JESD204, an octet represents an encoded 8-

- bit word. Fundamentally, F defines the frame structure given the number of lanes and the specific payload to

- deliver. F is defined by the following equation and represents the number of octets per frame:

7	𝐹 =

𝑀 × 𝑋 × 𝑁𝑃 8 × 𝐿

- Figure 2.3.2-13 provides an example 4T4R configuration supporting a typical 100 MHz waveform. The low

- PHY device processes complex (I & Q data) for each datapath and thus there are a total of 8 data streams that

- must be moved between the digital chip and the data conversion. The bit precision of this data is sufficient to

- meet the noise requirements of the solution and could be 12 or more bits. For this example, 16-bits will be

- assumed. To support a 100 MHz bandwidth, a complex data rate of 122.88 MSPS will be used as indicated

- by Table B.5.2-2 of [3].

JESD for UL x2

DFE & Low PHY

Or Low PHY

Fronthaul

JESD for DL x2

ADC

ADC

JESD

PHY

ADC

ADC

Clock Synthesis

Sysref A

Sysref B

DAC

DAC

JESD PHY

DAC

DAC

Device Clock A	Device Clock B

To RFFE

14

- Figure 2.3.2-13 Example 4T4R JESD204 Configuration

- Using this information and the equations above and applying 8B10B transport layer coding, the total payload,

- number of lanes and octets per frame can be computed. Since this is a 4T4R application and each path is

- represented by a complex stream, ‘M’ equals 8. ‘NP’ is 16 bits. ‘SC’ equals 122.88 MSPS. Total data payload

- can be computed using the following equation:

3	𝑃𝑎𝑦𝑙𝑜𝑎𝑑 = 8 × 16 × 122.886 × 10 = 19.6608𝐺𝑏𝑝𝑠

8

- Assuming the interface is based on JESD204B, then the maximum data rate for a single lane would be 12.5

- Gbps. Therefore, we need to divide the data onto two physical lanes of 9.8304 Gbps each. Hence in this

- example, ‘L’ equals 2.

7	𝐿 = 𝐶𝑒𝑖𝑙𝑖𝑛𝑔 (

19.6608 𝐺𝑏𝑝𝑠

12.5 𝐺𝑏𝑝𝑠

) = 2

8	Finally, ‘F’ the number of Octets/Frame is computed to be 8.

9	𝐹 =

8 × 1 × 16

= 8

8 × 2

- Using this information, the mapping of data can be determined by using online tools such as [17]. These tools

- describe how the converter data is mapped onto each lane by octet as shown in Figure 2.3.2-14. In this example

- on lane 0, the first octet conveyed are the most significant bits of converter 0. This is followed by the LSBs

- in the second octet. This is followed by converter 1, 2 and 3 on lane 0. In parallel, lane 1 provides the data

- for converters 5 through 7 with the most significant octet leading. In this figure, M#SO represents the logical

|  | 4 (L = 2, M = 8, F = 8, S = 1, NP = 16) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frame 0 |  |  |  |  |  |  |  |
| Lanes | Octet 0 | Octet 1 | Octet 2 | Octet 3 | Octet 4 | Octet 5 | Octet 6 | Octet 7 |
| 0 | M0S0[15:8] | M0S0[7:0] | M1S0[15:8] | M1S0[7:0] | M2S0[15:8] | M2S0[7:0] | M3S0[15:8] | M3S0[7:0] |
| 1 | M4S0[15:8] | M4S0[7:0] | M5S0[15:8] | M5S0[7:0] | M6S0[15:8] | M6S0[7:0] | M7S0[15:8] | M7S0[7:0] |

converter where # identifies the specific converter.

16

- Figure 2.3.2-14 4T4R Example Data Mapping

- More complicated examples are also supported as shown in Figure 2.3.2-15 where an 8T8R is described.  This

- configuration supports a 200 MHz iBW and thus requires a complex data rate of 245.76 MSPS. In this case,

- there are 8 transmit and 8 receive paths. Each of these requires a complex path to the low PHY device.

- Therefore ‘M’ equals 16. Each of these are represented by 16 bits. In this example, JESD204C is assumed

- and thus 64B/66B coding is used. Thus, the total throughput becomes:

3	16 × 16 × 245.766 × 66 = 64.88064𝐺𝑏𝑝𝑠

64

Sysref A

Sysref B

JESD for UL x2

DFE & Low PHY

Or Low PHY

Fronthaul

JESD for DL x2

DAC

DAC

DAC

DAC

JESD

PHY

DAC

DAC

DAC

DAC

ADC

ADC

ADC

ADC

JESD

PHY

ADC

ADC

ADC

ADC

Clock Synthesis

Device Clock A	Device Clock B

To RFFE

4

- Figure 2.3.2-15 Example 8T8R JESD204 Configuration

- JESD204C supports up to 32.45 Gbps and therefore 2 lanes will be required as shown below. In this example,

- 2 UL and 2 DL JESD lanes connect between the two devices.

8	𝐿 = 𝐶𝑒𝑖𝑙𝑖𝑛𝑔 (

64.88064𝐺𝑏𝑝𝑠

32.45 𝐺𝑏𝑝𝑠

) = 2

- Using the equation for ‘F’ determines the octets per frame to be 16 in this example. The frame structure as

- defined by JESD204 is shown in Figure 2.3.2-16 As with the 4T4R example, the frame structure is little endian

- with lane 0 beginning with converter 0 and lane 1 beginning with converter 8. Complete details on JESD204

- can be found [16].

5	𝐹 =

6

16 × 1 × 16

8 × 2

= 16

- Figure 2.3.2-16 8T8R Example Data Mapping

- 2.3.2.3.2 Optional Die to Die Interfacing

- The existing specification O-RAN.WG7.OMC-HRD-Opt7-2-v03.00 has several references to SERDES links,

- either in text or diagrams. The SERDES link is very appropriate for deployments where RF and baseband

- chips are separated by 10s of cm on a PCB, as it provides low wire count with decent power consumption

- (pJ/bit). For multi-chip modules, however, where die are separated by 10 mm or less, die-to-die (D2D) parallel

- interfaces such as Advanced Interface Bus (AIB) [18], Bunch of Wires (BoW) [19] or Universal Chiplet

- Interconnect Express (UCIe) [20] offer a lower power, lower complexity option, at the cost of more I/O.

- Figure 2.3.27-17 Basic die-to-die parallel architecture

- The simplicity of the die-to-die parallel architecture is illustrated in the figure above which depicts a 16-bit

- unidirectional “data unit” with single-ended drivers and receivers, plus a differential forwarded clock. The

- salient features of die-to-die interfaces are:

  - 1. Uni-directional throughput rates in the 100-1000 Gbps/mm depending on packaging technology, clock rate,

  - channel length, and use of source/load terminations. 1

  - 2.	Power consumption in the .5-1.0 pJ/bit range.

  - 3.	Scaleable, via the addition of more data units consisting of 16-20 I/O each, encompassing more silicon

  - beachfront or bump rows.

  - 4.	Simple PHY architecture not requiring any complex pre-emphasis or equalization…usually only Duty

  - Cycle Correction (DCC) and a Delay Lock Loop (DLL).

  - 5.	Reliable link such that FEC is typically not required.

9

- The diagram above shows an implementation of D2D in a Multi-chip module including the Digital Baseband

- unit and the Transceiver in one package. Referring to Figure 2.3.2-2, the Digital Processing unit in the figure

- corresponds to the Digital Baseband unit in the figure above. This figure shows DL I/Q data flowing to the

- transceiver, but there would be a corresponding path in the other direction for UL I/Q data flowing from

- Transceiver to the Digital Baseband unit.

- In Figure 2.3.27-17, the clock source is shown on the left of the data transmitting unit. Since DDR mode may

- be used for max throughput, and the clock is typically in the 1-4 GHz range, an accurate duty cycle is required.

- The Duty Cycle Correction (DCC) block would be employed to ensure that a duty cycle close to 50% is

- maintained.

- On the RX side, a Delay Lock Loop (DLL) may be used to ensure that the clock strobes data into the data

- register at approximately the middle of the data bits. If so, a calibration or initialization procedure would

- determine the clock phase shift required to center the strobe.

- The packaging technology and bump pitch used for each die greatly influences the final throughput expressed

- as Gbps/mm. Standard chip scale bump pitches of 100 um or greater, used with organic substrate chip scale

- packaging, allow for simplified D2D interconnect but will not give the maximum throughput. The use of

- micro-bumps and specialized interposer technology, or even EMIB, will provide for throughput approaching

- 1 Tbps/mm and lower power.

- With the AIB, UCIe, and BoW D2D interfaces mentioned initially, a standardized bump map for each data

- unit is either required, or strongly recommended. (At a minimum, there needs to be a wiring order so that

- packages with different bump pitches can still be used and data lines routed through the RDL without criss-

- crossing or excess routing skew.) The # of routing layers in the packaging interconnect determines how many

- data units (with their associated physical bump patterns) can be adjoined (inwards towards the die), in order

- to further increase throughput for the same silicon beachfront.

- Initial D2D deployments used un-terminated drivers and receivers, which was sufficient for clock rates in the

- low GHz range and channel lengths of a few mm. To achieve longer channel lengths and/or higher base clock

- rates, terminations may be employed at the source driver, the receiver, or both.

36

1 Here the linear mm dimension refers to the silicon edge beachfront dedicated to the parallel I/O

### 2.3.3 Synchronization and Timing

- The clock and synchronization are determined by the PTP and SyncE signals transported by ethernet L2 via

- the fronthaul in conjunction with clock recovery in the hardware. Together these provide the local time of day

- and frequency reference for the O-RU7-2.

Control

Ethernet Clock

Embedded CLK

Clock, Sync &

Control

GPS/Aux Clock

DevCLK

Clock, Sync &

Control

TCXO

GPS/Aux or Cascade

Synchronizer

QSFP

Ethernet TRx

RF TRx

Clock Software Stack

XTAL

Clock Synthesizer

, Sysref

5

6	Figure 2.3.3-1 Clock and Synchronization Functional Block Diagram

7

#### 2.3.3.1 Hardware Specifications

- 2.3.3.1.1 Interface

- Table 2.3.3-1 Synchronization and Timing Interface Specifications

| Item Name | Description |
| --- | --- |
| Clock Inputs | Depending on the configuration, the following clock inputs may exist: QFSP, Ethernet, GPS, Cascaded unit clocks, Synchronized Clock. |
| Clock Outputs | Depending on functional requirements, the following clocks may be delivered: QFSP, Ethernet, DDR, CPU, RF Transceiver clock, Synchronizer reference. |
| Clock Control | SPI or I2C to control the clock synthesizer and synchronizer. |
| Clock Sync | Logic signal to phase align and time align the synchronizer with master. |
| Crystal Reference | One or more reference oscillators to provide required phase noise and short/long term stability. |

11

  - 2.3.3.1.2 Performance Specifications

  - See device specifications in Table 2.3.3-3.

  - 2.3.3.1.3 Synchronizer

  - The purpose of the Synchronizer is to function as part of the local clock recovery in conjunction with an

  - IEEE1588v2 software stack to provide local time of day and to generate a local synchronous reference signals

  - to be used for synthesis of necessary clocks to other functions within the O-RU7-2.

  - 2.3.3.1.3.1	Hardware Specifications

  - 2.3.3.1.3.1.1  Interface

  - Table 2.3.3-2 Synchronizer Interface Specifications

| Item Name | Description |
| --- | --- |
| REFA, REFAA | differential or single ended reference pulse inputs. |
| REFB, REFBB | differential or single ended reference pulse inputs. |
| Status and Control | control core operation and report on state of the PLL. |
| XOA, XOB | complimentary reference inputs or reference crystal connections |
| Output0 and Output0N & Output1 and Output1N | reference outputs to feed further synthesis, transceiver or logic blocks. |

10

- 2.3.3.1.3.1.2  Performance Specifications

- Table 2.3.3-3 Synchronizer Performance Specifications

| Item Name | Description | Notes |
| --- | --- | --- |
| 10 Hz Offset | −77 dBc/Hz | Device configuration: fOSC = 52 MHz crystal, fREF = 30.72 MHz, fVCO = 2457.6 MHz, fOUT = 245.76 MHz, DPLL BW = 50 Hz, internal zero delay operation |
| 100 Hz Offset | −93 dBc/Hz |  |
| 1 kHz Offset | −114 dBc/Hz |  |
| 10 kHz Offset | −125 dBc/Hz |  |
| 100 kHz Offset | −130 dBc/Hz |  |
| 1 MHz Offset | −140 dBc/Hz |  |
| 10 MHz Offset | −156 dBc/Hz |  |

13

  - 2.3.3.1.3.2	Hardware Design

2

3	Figure 2.3.3-2 Synchronizer Block Diagram

4

- 2.3.3.1.4 Reference Synthesizer

- The purpose of the clock synthesizer is to generate low jitter, high stability reference clocks as needed

- throughout the design.

- 2.3.3.1.4.1	Hardware Specifications

- 2.3.3.1.4.1.1  Interface

- Table 2.3.3-4 Synthesizer Interface Specifications

| Item Name | Description |
| --- | --- |
| Reference Clock(s) Input | The clock inputs device should receive a stable reference from a crystal, network reference or other suitable source. More than one input is allowed that may be selected between as required by the system. |
| Control | Control of the transceiver is by way of 3 or 4 wire SPI or I2C functioning as a slave. Support for 1.8V control is required and tolerance of 3.3V is preferred. Typically, the clock devices will be part of an IEEE1588v2 protocol loop controlled by way of this control interface, or other GPIO lines as required by the hardware implementation. |
| Clock Outputs | One or more clock outputs are required to drive the digital device and transceiver clock inputs. Each output should be independently programmable in frequency to suit the application. The outputs should nominally be differential to preserve clocking edge and to maximize tolerance of high common mode signals. |

11

  - 2.3.3.1.4.1.2  Performance Specifications

  - Table 2.3.3-5 TCXO Clock Performance Specifications

| Item Name | Description |
| --- | --- |
| Frequency | 25 MHz |
| Stability | +/- 0.28 ppm |
| 10 Hz Offset | -90 dBc/Hz |
| 100 Hz Offset | −120 dBc/Hz |
| 1 kHz Offset | −140 dBc/Hz |
| 10 kHz Offset | −151 dBc/Hz |
| 100 kHz Offset | −152 dBc/Hz |
| 1 MHz Offset | −154 dBc/Hz |

3

4	2.3.3.1.4.2	Hardware Design

5

6

7

#### 2.3.3.2 Hardware Design

Figure 2.3.3-3 Synthesizer Block Diagram

### 2.3.4 External Interface Ports

#### 2.3.4.1 Hardware Specifications

- The following table shows the external ports or slots provided by the O-RU7-2. External connections can be

- aggregated into a composite connector to minimize the points of environmental ingress as required for a

- specific deployment.

  - Table 2.3.4-1 External Port List

| Port Name | Feature Description |
| --- | --- |
| Fronthaul interface | One or more 25Gbps optical interfaces including a primary link and optionally a second link for redundancy or daisy chain. |
| Debug interface | RJ45 for maintenance access. |
| Power interface | Two pin male panel mount connectors (x1) |
| RF Interface | 4.3-10+ IP67 connector (x4) [9] [11] / M-LOC cluster/multiport RF connector [12] |
| Calibration Port (Optional) | N Female or 4.3-10+ Female |
| AISG[10] RET Port | RS-485, IEC 60130-9, or RF connectors |

2

#### 2.3.4.2 Hardware Design

  - 2.3.4.2.1 Fronthaul Interface

  - The fronthaul interface to the OMC is provided by an optical fiber or fibers connected to the chassis by way

  - of a SFP or SFP+ cage connector. In addition, an IP67 connector seal should be selected to provide appropriate

  - environmental protection.

8

9

10

11

- 2.3.4.2.2 Debug Interface

Figure 2.3.4-1 SFP+ case and connector

- The local maintenance port or debug port is by way of a standard ethernet connector. The RJ45 Ethernet

- connector are standardized components and available from multiple vendors. The following figure describes

- the general form factor of one such RJ45 connector.

1

2

- 2.3.4.2.3 Power Interface

Figure 2.3.4-2 RJ45 Interface

- The standard power interface is defined by a 2-pin panel mount connector as shown in the following figure.

5

6

- 2.3.4.2.4 RF Interface

Figure 2.3.4-3 Power Interface

- The standard RF interface are the 4.3-10+ Low PIM RF connector and the M-LOC cluster/multiport connector

- as shown in the following figure.

4.13

mm 10 mm

15.16 mm

20 mm

#### (a)	(b)

1

  - Figure 2.3.4-4 (a) 4.3-10+ RF Connector [9] [11] (b) M-LOC cluster/multiport connector [12]

### 2.3.5 Mechanical

  - One such mechanical structure is shown in the following figure.  The shell should provide appropriate

  - environmental protection, RF shielding and passive thermal dissipation. Internally, thermal pads are used to

  - contact local heat sources and transfer that heat to the frame which in turn dissipates the heat through bulk

  - metal and fins used to increase the effective surface area. Other enclosures are possible.

1

  - Figure 2.3.5-1 Mechanical Enclosure Side View

3

4	Figure 2.3.5-2 Mechanical Enclosure End View

### 2.3.6 Power Unit

- For the Outdoor Microcell, the power solution of the O-RU7-2 is provided externally from a telco -48V supply.

- Internally, the telco supply is converted and distributed to devices using appropriate DC-DC and LDO

- converters. Optionally the unit can be powered by AC line power which is not shown here.

#### 2.3.6.1 Hardware Specifications

- The power unit is responsible for isolating and converting the telco supply to appropriate voltages for

- distribution to individual supply domains in the O-RU7-2. The typical dissipation is outlined below. The RF

- front end is broken out separately since different deployment scenarios will require different frequencies and

- power levels resulting is significantly different total dissipation. Total Power is the sum of the two sections

- below.

- Table 2.3.6-1 Transceiver Module Specifications

| Module | 4T4R (Watts) |  | Notes |
| --- | --- | --- | --- |
| FPGA | 16 |  |  |
| Transceiver | 7 |  |  |
| Synchronization & Clocking | 3 |  |  |
| DDR | 1 |  |  |
| Others | 3 |  |  |
| SFP+ Module | 1.5 |  | 1 module installed |
| total device consumption | 31.5 |  |  |
| O-RU7-2 Low PHY Transceiver power consumption | 36.8 |  | On board power efficiency 85% |

12

- Table 2.3.6-2 RF Front End Module Specifications

| Module | 4T4R (Watts) |  | Notes |
| --- | --- | --- | --- |
| PA | 125 |  | 75% Tx, Max RF. 10W rms output @ 100 MHz |
| PA Drive Amp | 4 |  |  |
| LNA | 0.5 |  | 25% Rx |
| Others | 2.5 |  | Bias generators and controller |
| total device consumption | 132 |  |  |
| O-RU7-2 RF Front End power consumption | 155.3 |  | On board power efficiency 85% |

#### 2.3.6.2 Hardware Design

- The block diagram of the power unit is shown in the figure below, which depict the power distribution to

- various functional modules such as RF PAM board and FPGA/RF SoC board (Digital) along with approximate

- power requirements.

1

2

### 2.3.7 Thermal

Figure 2.3.6-1 Power Reference Design

  - Heat dissipation in O-RU7-2 is widely done through passive cooling via three modes: conduction, convection,

  - and radiation. These mechanisms are accomplished with help of heat sink, heat pipes and vapor chamber etc.

  - The transfer of heat though conduction can usually be described by Fourier’s law, which is stated:

7	𝑞 = (𝑘⁄𝑠)𝐴. 𝑑𝑇

- k is the thermal conductivity of the material, and has units of W/m·K, s = material thickness, A = heat transfer

- area, dT = temperature difference between inside and outside wall.

- Table 2.3.7-1 Thermal Conductivity of common Metals [13]

| Type of material | Thermal Conductivity (W/m.K) |
| --- | --- |
| Diamond | 2300 |
| Pure Al | 237 |
| Hard Aluminium 4.5% Cu | 177 |
| Au | 315-317 |
| Ag 99.9% | 411-429 |
| Iron | 80 |
| Tin | 67 |

- The heat sinks transfer heat to the space by thermal convection and radiation. For thermal convection, it can

- be calculated by the Newton’s Equation:

13	𝑄𝑑 = 𝛼𝑐𝐴∆𝑡 = 𝛼𝑐𝐴(𝑡𝜔 − 𝑡𝑓)

- where, Qd is the convention heat (W), tω is the surface temperature of object (K), tf is the temperature of airflow

- (K), A is the surface area of the object involved in heat transfer (m2), ∆t is the temperature difference between

- the surface temperature of object and airflow (K), αc is convective heat transfer coefficient (W/m2.K).

- Table 2.3.7-2 Convective heat transfer coefficient [14]

| Flow Type | Heat transfer coefficients (W/m2.K) |
| --- | --- |

| Forced convection; low speed flow of air over a surface | 10 |
| --- | --- |
| Forced convection; moderate speed flow of air over a surface | 100 |
| Forced convection; moderate speed cross flow of air over a cylinder | 200 |
| Forced convection; moderate flow of water in a pipe | 3000 |

1

2	For thermal radiation, it can be calculated by the following Equation:

3	𝑄𝑟 = 𝜀𝛿𝐴(𝑡4 − 𝑡4)

𝜔	𝑓

- where, Qr is the radiation heat (W), ε is the thermal emissivity, δ is the Stefan–Boltzmann constant, its value

- is 5.67032 x10-8 W/ (m-2. K-4). From these Equations, we can see that the increase of surface area A and

- thermal emissivity ε will improve the heat dissipation.

- Different materials can have widely different emissivity values within the range of 0 to 1.00.

8

| Material | Emissivity Value (ε) |
| --- | --- |
| Aluminium: anodized | 0.77 |
| Aluminium: polished | 0.05 |
| Brass: highly polished | 0.03 |
| Brass: oxidized | 0.61 |
| Carbon: candle soot | 0.95 |
| Carbon: graphite, filed surface | 0.98 |
| Copper: polished | 0.05 |
| Copper: oxidized | 0.65 |

9

### 2.3.8 Environmental and Regulations

- The O-RU7-2 hardware system is RoHS Compliant. Unit is passively cooled.

- Table 2.3.8-1 Environmental Features

| Item Name | Description | Notes |
| --- | --- | --- |
| Operating Temperature | -40°C to +55°C | Cold and hot start |
| Operating Relative Humidity | 5% to 95% |  |
| Atmospheric Pressure | 70 to 106Kpa |  |

13

## 2.4 Integrated O-DU & O-RU (gNB-DU) Reference Design

### 2.4.1 O-DU Portion of Integrated Reference Design

#### 2.4.1.1 O-DU High-Level Functional Block Diagram

  - 2.4.1.2 O-DU Hardware Components

  - 2.4.1.2.1 Digital Processing Unit

  - 2.4.1.2.2 Hardware Accelerator (if required by design)

  - 2.4.1.2.2.1	Accelerator Design 1

  - 2.4.1.2.2.2	Accelerator Design 2

  - 2.4.1.3 Synchronization and Timing

### 2.4.2 O-RU Portion of Integrated Reference Design

#### 2.4.2.1 O-RU High-Level Functional Block Diagram

  - 2.4.2.2 O-RU Hardware Components

  - 2.4.2.2.1 Digital Processing Unit

  - 2.4.2.2.2 RF Processing Unit

  - 2.4.2.2.2.1	Transceiver Reference Design

  - 2.4.2.2.2.2	Power Amplifier (PA) Reference Design

  - 2.4.2.2.2.3	Low Noise Amplifier (LNA) Reference Design

  - 2.4.2.2.2.4	RF Switch Reference Design

  - 2.4.2.2.2.5	Antenna / Phased Array Reference Design

  - 2.4.2.2.3 Synchronization and Timing

  - 2.4.3 External Interface Ports

  - 2.4.4 O-DU/O-RU Firmware (if required by design)

  - 2.4.5 Mechanical

  - 2.4.6 Power Unit

- 2.4.7 Thermal

- 2.4.8 Environmental and Regulations

## 2.5 Integrated O-CU & O-DU Reference Design

### 2.5.1	O-CU Portion of Integrated Reference Design

#### 2.5.1.1 Integrated O-CU High-Level Functional Block Diagram

- 2.5.1.2 O-CU Hardware Components

- 2.5.1.2.1	Digital Processing Unit

- 2.5.1.2.2	Hardware Accelerator (if required by design)

- 2.5.1.2.2.1  Accelerator Design 1

- 2.5.1.2.2.2  Accelerator Design 2

- 2.5.1.3 Synchronization and Timing

- 2.5.1.4 O-CU Firmware (if required by design)

### 2.5.2	O-DU Portion of Hardware Reference Design

#### 2.5.2.1 O-DU High-Level Functional Block Diagram

- 2.5.2.2 O-DU Hardware Components

- 2.5.2.2.1	Digital Processing Unit

- 2.5.2.2.2 Hardware Accelerator (if required by design)

- 2.5.2.2.2.1  Accelerator Design 1

- 2.5.2.2.2.2  Accelerator Design 2

- 2.5.2.3 Synchronization and Timing

- 2.5.3	External Interface Ports

- 2.5.4	O-DU Firmware

- 2.5.5	Mechanical

- 2.5.6	Power Unit

- 2.5.7	Thermal

- 2.5.8	Environmental and Regulations

- 2.6 FHGW Hardware Reference Design

| 5 | 2.6.1 | FHGW High-Level Functional Block Diagram |
| --- | --- | --- |
| 6 | 2.6.2 | FHGW Hardware Components |
| 7 | 2.6.2.1 | Digital Processing Unit |
| 8 | 2.6.3 | Synchronization and Timing |
| 9 | 2.6.4 | External Interface Ports |
| 10 | 2.6.5 | Mechanical |
| 11 | 2.6.6 | Power |
| 12 | 2.6.7 | Thermal |
| 13 | 2.6.8 | Environmental |

# Annex 1: Parts Reference List

## Annex 1.1 O-CU Reference Parts List

- The tables below represent potential options for component selection for O-CU7-2.

- The HW reference design of O-CU is identical to the O-DU7-2 except for the need of HW accelerator as

- outlined in section 2.2.

19

## Annex 1.2 O-DU7-2 Reference Parts List

  - The tables below represent potential options for component selection for O-DU7-2. 3

4	Intel Based O-DU7-2

| No. | Item Name | Description |
| --- | --- | --- |
| 1 | Intel® Xeon® Processor D-2183IT | Intel® Xeon® Skylake D D-2100 SoC processor |
| 2 | Intel® I350 | Intel® 1GbE x4 Ethernet controller |
| 3 | Intel® X557-AT2 | Intel® 10GbE x2 Ethernet controller |
| 4 | Intel® I210 | Intel® 1GbE Ethernet controller |
| 5 | ASPEED® AST-2500 | ASPEED® BMC controller |
| 6 | UniIC® SCQ08GU03H1F1C-26V | UniIC® DDR4 8GB |
| 7 | Powerleader® MTFDDAK480TDC- 1AT1ZABYY | Powerleader® SSD 480GB |
| 8 | Accelink® RTXM228-551 | Accelink® 10Gbps Optical module |

5

## 6	Annex 1.3 O-RU7-2 Reference Parts List

7	The tables below represent potential options for component selection for O-RU7-2.

1	ADI Based O-RU7-2

| No. | Item Name | Description |
| --- | --- | --- |
| 1 | ADP196ACBZ-R7 | 5V, 3A Logic Controlled High-Side Power Switch |
| 2 | LTC7150SEY#PBF | 20V, 20A Synchronous Ste-Down Regulator |
| 3 | ADRV9029 | Integrated, Quad RF Transceiver with Observation Path and DPD/CFR |
| 4 | ADP223ACPZ-R7 | Dual, 300 mA Adjustable Output, Low Noise, High PSRR Voltage Regulator |
| 5 | LTC4217IDHC#PBF | 2A Integrated Hot Swap Controller |
| 6 | ADCLK846BCPZ-REEL7 | 1.8V, 6 LVDS/12 CMOS Output Low Power Clock Fanout Buffer |
| 7 | AD9545BCPZ | IEEE 1588 V2 and 1 pps Synchronizer and Adaptive Clock Translator |
| 8 | ADRF5545A | Dual-Channel 2.4 to 4.2 GHz Receiver Front End |
| 9 | LTC3315AIV#TRMPBF | Dual 5V, 2A Synchronous Step-Down DC/DCs |
| 10 | LTC3634EUFD#PBF | 15V Dual 3A Monolithic Step-Down Regulator for DDR Power |
| 11 | LTC2928IUHF#BPF | Multichannel Power Supply Sequencer and Supervisor |
| 12 | ADP5054ACPZ-R7 | Quad Buck Regulator Integrated Power Solution |
| 13 | AD9576BPZ | Dual PLL, Asynchronous Clock Generator |
| 14 | LT1389ACS8-1.25#PBF | Nanopower Precision Shunt Voltage Reference |
| 15 | Intel® Arria® 10 SoC | Intel® Arria® 10 SoC 1AS048F35 |
| 16 | MAX1619MEE | Temp Sensor with Dual-Alarm Outputs and PMBus |

2